| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M ranging channel indication using UL-MAP for M2M in 16e |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Honggang Li, Shantidev Mohanty Rui Huang, Xiangying Yang Intel Inbar Anson Bratspiess Sequans Erik Colban Huawei Chun-Yen Hsu, Tsung-Yu Tsai Institute for Information Industry (III) Jaesun Cha ETRI Hassan Alkanani NEC Hyunjeong Kang, Anil Agarwal Samsung | E-mail: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M ranging channel indication using UL-MAP for M2M in 16e

Honggang Li,

Shantidev Mohanty

Rui Huang,

Xiangying Yang

Intel

Inbar Anson Bratspiess

Sequans

Erik Colban

Huawei

Chun-Yen Hsu, Tsung-Yu Tsai

Institute for Information Industry (III)

Jaesun Cha

ETRI

Hassan Alkanani

NEC

Hyunjeong Kang, Anil Agarwal

Samsung

# Introduction

Dedicated ranging channel for M2M devices was agreed on in March meeting, to address the large ranging load coming from the M2M devices.

This contribution proposes changes to the 16e standard to implement the dedicated ranging channel for fixed M2M devices.

# Proposed Text

2.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 14 in page 2.]

8.4.5.4.4 UL-MAP Extended IE

8.4.5.4.4.1 UL-MAP Extended IE format

[Modify Table 381 as follows]

Table 381—Extended UIUC code assignment for UIUC = 15

| Extended UIUC (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Power Control IE |
| 0x1 | Reserved |
| 0x2 | AAS UL IE |
| 0x3 | CQICH Allocation IE |
| 0x4 | UL Zone IE |
| 0x5 | UL-MAP Physical Modifier IE |
| 0x6 | Reserved |
| 0x7 | UL-MAP Fast Tracking IE |
| 0x8 | UL PUSC Burst Allocation in Other Segment IE |
| 0x9 | Fast Ranging IE |
| 0xA | UL Allocation Start IE |
| 0xB | Dedicated Ranging Channel for M2M IE |
| 0xBC … 0xF | Reserved |

----------------------------------------------  Text End -----------------------------------------------

2.2 Proposed Text #2

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 14 in page 2.]

8.4.5.4.4 UL-MAP Extended IE

8.4.5.4.4.30 M2M Ranging Allocation UL-MAP Extended IE format

The M2M Ranging Allocation UL-MAP Extended IE is used to indicate if the allocated ranging channel is used for M2M devices.

[Create Table xxx as follows]

Table xxx—M2M Ranging Allocation UL-MAP Extended IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| M2M Ranging Allocation UL-MAP Extended IE(){ | - | - |
| Extended UIUC | 4 | M2M Ranging Allocation UL-MAP Extended = 0xB |
| Length | 4 | Length is TBD |
| OFDMA Symbol offset | 8 | - |
| Subchannel offset | 7 | - |
| No. OFDMA Symbols | 7 | - |
| No. Subchannels | 7 | - |
| Ranging Method | 2 | 0b00: Initial ranging/Handover Ranging over two symbols 0b01: Initial ranging/Handover Ranging over four symbols 0b10-0b11: reserved |
| Dedicated ranging indicator | 1 | 0: The OFDMA region and ranging method defined are used for the purpose of normal ranging 1: The OFDMA region and ranging method defined are used for the purpose of ranging using dedicated CDMA code and transmission opportunities assigned in the MOB_PAG-ADV message |
| M2M Dedicated Ranging Allocation Indicator | 1 | When this bit is set to 1, the ranging allocation defined by OFDMA UL-MAP IE format shall not be used by M2M devices |
| } | - | - |

----------------------------------------------  Text End -----------------------------------------------