| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized schemes of network re-entry for M2M in 16e |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Honggang Li, Shantidev Mohant, Rui Huang, Xiangying Yang Intel Wei-Chieh Huang, Chia-Lung Tsai, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI | E-mail: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimized schemes of network re-entry for M2M in 16e

Honggang Li,Shantidev Mohant,Rui Huang,Xiangying Yang

Intel

Wei-Chieh Huang, Chia-Lung Tsai, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

The traffic characteristics of M2M devices for different purposes are different, some are periodic traffic or in some known pattern, the others are burst in unknown pattern, for the devices with the traffic in the known pattern, the contention free mechanism can be used for network re-entry if the device in idle mode, for the device with the traffic in the unknown pattern, the contention based mechanism should be kept.

Another factor to be considered is mobility characteristics of M2M devices, for fixed M2M device, the large scale channel condition can be much more stable, and the small scale channel condition in UL can be measured through DL synchronization and measurement esp. in TDD mode, so that the ranging process for UL synchronization can be saved. But for mobile M2M device, the ranging process should be kept for reliable measurement and transmission in UL.

Based on this analysis, the optimization of network re-entry for M2M in 16e is proposed in this following section.

# Optimization of network re-entry for M2M

For different category of M2Mmobility and traffic characteristics, the mechanism of network re-entry can be optimized and categorized as shown in the following table:

Table 1, optimized schemes of network re-entry for M2M

|  | Known traffic pattern | Unknown traffic pattern |
| --- | --- | --- |
| Fixed | - Grouping paging/paging - Dedicated ranging channel allocation for contention free access - Ranging process with or without ranging code transmission | - Common /Dedicated ranging channel allocation for contention based access |
| Mobile | - Grouping paging/paging - Dedicated ranging channel allocation for contention free access | - Common/Dedicated ranging channel allocation for contention based access |

Based on the mobility and traffic characteristics of the M2M device, the BS can select the proper network re-entry scheme for M2M device, the selection can be based on the table-1, and the BS shall indicate to the M2M device which network re-entry scheme in MOB_PAG-ADV or other MAC management messages. In this contribution, MOB_PAG-ADV is proposed to be used for network re-entry scheme indication.

The mobility and traffic characteristics of the M2M device can be known by:

- Explicitly DSA-REQ(traffic characteristics) and/or REG-REQ(SS capability) and/or location update(mobility)

- Or pre-provision to the BS from M2M server or other network entity

- Or the implicit statistics by the BS or some network entity

# Proposed texts

3.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 13 in page 2.]

6.3.23.9 Network reentry from idle mode

Based on the mobility and traffic characteristics of the M2M device, the BS can select the proper network re-entry scheme for M2M device based on the table-xxx, and the BS shall inform the M2M device the network re-entry scheme indicator in MOB_PAG-ADV message.

Table xxx, Scheme selection of network re-entry for M2M

| Network re-entry scheme indicator | Network re-entry scheme | Note |
| --- | --- | --- |
| 0 | Dedicated ranging channel allocation for M2M group or M2M device in MOB_PAG-ADV | Mobile M2M, Known traffic pattern, paging |
| 1 | Ranging process without Ranging code transmission, the UL_MAP IE offset for RNG_REQ is indicated in MOB_PAG-ADV | Fixed M2M, Known traffic pattern, paging |
| 2 | Dedicated ranging channel for M2M group or M2M device in UL_MAP message | All M2M |
| 3 | Common ranging channel | All M2M and human devices |

If the network re-entry indicator is set to “0”, the BS shall allocate the dedicated ranging channel for M2M device in MOB_PAG-ADV message, the M2M device can find the dedicated CDMA code assignment and transmission opportunity offset in “CDMA code and transmission opportunity assignment” of MOB_PAG-ADV message for ranging and network re-entry.

If the network re-entry indicator is set to “1”, the M2M device doesn’t need to send CDMA code for ranging but decodes UL-MAP IE directly for slot allocation for RNG-REQ message. If the SS receives an UL-MAP containing a CDMA Allocation IE with the parameters of the code which allocated in MOB_PAG-ADV message, it shall proceed to send a unicast RNG-REQ on the allocated bandwidth.

If the network re-entry indicator is set to “2”, the BS shall allocate the dedicated ranging channel for M2M device in UL-MAP message.

If the network re-entry indicator is set to “3”, the BS shall indicate to M2M device to use common ranging channel for network re-entry.

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 13 in page 2.]

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message

The MOB_PAG-ADV message shall be sent on the Broadcast CID or Idle Mode Multicast CID during the BS paging interval.

The format of the message is shown in Table 154.

[Modify Table154 as follows]

Table 154—MOB_PAG-ADV message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_PAG-ADV_Message_format() { | — | — |
| Management Message Type = 61 | 8 | — |
| Num_Paging_Group_IDs | 8 | Number of Paging Group IDs in this message |
| for (i = 0; i < Num_Paging_Group_IDs; i++) { | — | — |
| Paging Group ID | 16 | — |
| } | — | — |
| Num_MACs | 8 | Number of MS MAC addresses |
| For (j = 0; j < Num_MACs; j++) { | — | — |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved |
| M2M indicator | 1 | 0b0 = Not M2M device 0b1 = M2M device |
| M2M network re-entry indicator | 2 | Indicate the network re-entry scheme for M2M device |
| Reserved | 63 | — |
| } | — | — |
| Padding | variable | Padding bits to ensure octet aligned |
| TLV Encoded Information | variable | TLV-specific |
| } | — | — |

……

For OFDMA PHY, the following TLV may be included in the MOB_PAG-ADV management message:

CDMA code and transmission opportunity assignment for M2M group (see 11.17.3)

OFDMA-PHY-specific parameter used to indicate CDMA code and transmission opportunity assigned to one or more M2M groups being paged in this message. One CDMA code and transmission opportunity assignment in the TLV corresponds to one M2M device paged. The order of the assignment is the same as the order of appearance of M2M Group ID and the M2M device bitmap within an M2M group.

----------------------------------------------  Text End -----------------------------------------------

3.3 Proposed Text #3

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 13 in page 2.]

11.17.3 CDMA code and transmission opportunity assignment for M2M group

The CDMA Code and Transmission Opportunity Assignment field indicates the assigned code and transmission opportunity for an M2M device in an M2M group that is paged to use over dedicated CDMA ranging region.

| Type | Length(bits) | Value | Scope |
| --- | --- | --- | --- |
| 153 | Variable | Bits 0-7: Num_group, Number of M2M groups for (i = 0; i < Num_group; i++) { 8 bits: Num_M2M, Number of M2M devices in ith M2M group for (j = 0; j < Num_M2M; j++) { x bits: M2M ID for jth M2M device paged in ith M2M group 2 bits: Action Code 2 bits: M2M network re-entry indicator } } | OFDMA |

----------------------------------------------  Text End -----------------------------------------------