| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of M2M device with long paging cycle in idle mode |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Inuk Jung, Jin Sam Kwak LG Electronics Hyunjeong Kang Samsung Electronics Co., Ltd. Shantidev Mohanty Intel Corporation | giwon.park@lge.com +82-31-450-1879 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes the idle mode operation to support M2M device with very long paging cycle. |  |
| Purpose | To be discussed and adopted by 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of M2M device with long paging cycle in idle mode

Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Inuk Jung, Jin Sam Kwak

LG Electronics

Hyunjeong Kang

Samsung Electronics Co., Ltd.

Shantidev Mohanty

Intel Corporation

- Introduction

This contribution proposes the paging operation of M2M device with long paging cycle in idle mode.

- Reference

[1] IEEE C802.16p-11/00018r2 P802.16p Amendment Working Document (AWD)

- Discussion

Proposal . M2M device with long paging cycle receives paging multiple times at paging cycle.

With such large value of paging cycle (e.g., above several minutes or hours), if M2M device misses the paging message, M2M device needs to get the paging message at the next paging cycle. Thus, whenever M2M devices wake up every long paging cycle, it can be beneficial that M2M devices check paging multiple times according to the additional paging offset to make sure reliable reception of paging, e.g., as shown in figure 1.

Figure 1. Multiple paging message transmission for M2M device with long paging cycle

Moreover, if the device misses the paging message as well as PGID-Info message, it can implicitly know that paging cycle and paging offset is changed and then, device performs the location update to adopt the changed paging cycle or paging offset.

- Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 16.2.18.7.1(page 22, line 19) as follows:]

16.2.18.7 Idle Mode for M2M application

16.2.18.7.1 Paging operation

Two paging offsets may be assigned to the M2M device with a long paging cycle (e.g., above several minutes or hours) at the idle mode initiation. If the M2M device does not receive the AAI-PAG-ADV message at its first paging offset, the M2M device shall monitor the transmission of the AAI-PAG-ADV message at its second paging offsets. After transmitting the AAI-PAG-ADV message with action code 0b0 (Performing network reentry) during M2M device’s first paging offset, if the ABS does not receive a response from the paged M2M device, the ABS may re-page this M2M device at its second paging offset which is indicated in AAI-DREG-RSP message.

[Remedy 2. Add the following parameters on page 13, line 37 in 16.2.3.22 AAI-DREG-RSP]

16.2.3.22 AAI-DREG-RSP

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M paging offset | 10 | Used to indicate the superframe within the paging cycle at which the M2M device's paging listening interval starts. The superframe is determined by concatenating the M2M paging offset field and the Paging offset field. M2M paging offset shall be interpreted as the MSB. Shall be smaller than Paging cycle value. | May be present when the Paging cycle value is set to 0x08, 0x09, or 0x10. |
| Second paging offset | 12 | Used to indicate additional paging offset for the M2M device. | Optional. |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------