| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Transmission of Dedicated Ranging Information and Signaling Support for M2M Devices |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Jinsoo Choi, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak LG Electronics | js.choi@lge.com |
| Re: | Call for Comments on the 802.16p Amendment Working Document IEEE 802.16p-10/0018r2 |  |
| Abstract | The contribution provides the proposed text for transmission of M2M dedicated ranging information and related signaling support to be included in IEEE 802.16p amendment document. |  |
| Purpose | To be discussed and adopted in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Transmission of Dedicated Ranging Information and Signaling Support for M2M Devices

Jinsoo Choi, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

In the previous #72 Singapore meeting, it has been accepted that BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. M2M devices will perform ranging for network (re-)entry using assigned dedicated ranging resources [1]. In current 802.16m/D12, there is only dedicated ranging allocation for handover ranging so it is need to define where the information of dedicated ranging for M2M devices is transmitted and how the BS can control it efficiently with existing shared ranging. In this proposal, we provide the proposed text for transmission of dedicated ranging information and related signaling support for M2M devices based on existing 802.16m system.

- Rationale and Proposal

- Channel to transmit the dedicated ranging information for M2M devices

  - If the dedicated ranging is assigned to M2M devices who try network re-entry, it would target multiple M2M devices rather than a specific M2M device. Thus dedicated ranging through unicast transmission which is applied optionally in dedicated handover ranging in 802.16m system might not be proper way for M2M devices in aspect of overhead. Some broadcast channel that can be transmitted to M2M devices is recommended to carry the dedicated ranging information.

  - AAI-SCD message is the broadcast channel which is transmitted periodically, and all devices must confirm/update the information every re-entry stage, so M2M devices who use dedicated ranging can easily obtain the related information with assured reliability through the AAI-SCD update procedure.

- Information to be carried for M2M dedicated ranging

- Ranging opportunity index: Indicates the index of the allocated ranging opportunity of the ranging channel used in the RA-ID.

- Ranging opportunity subframe index: Indicates the subframe index of the allocated ranging opportunity. The subband of a ranging channel is same as the ranging channel allocated by SFH.

- Periodicity of ranging: Indicates the periodicity of ranging allocation.

- Signaling support to operate M2M dedicated ranging with minimized impact on existing AMSs

- M2M ranging indicator: To control the access load between the dedicated ranging and normal (shared) ranging with existing AMSs, BS can indicate the usage of ranging for M2M devices according to system environment. For example, in the environment where the small number of dedicated ranging is applied to AMS, BS can indicate the usage of dedicated ranging to M2M devices. Inversely, in the environment where the many dedicated ranging is already applied to AMSs, BS will indicate ranging without dedicated allocation to M2M devices. Or both are allowed to M2M devices and BS let M2M devices select most feasible ranging way based on back-off window and waiting offset time. This information is targeting for M2M devices who can be assigned by dedicated ranging, so existing AMSs can operate transparently.

- M2M configuration change count and M2M SCD count: To avoid the unnecessary decoding/update the dedicated ranging information for M2M devices and impact on existing AMSs’ operation, additionally BS can transmit M2M configuration change count and M2M SCD count like Configuration Change Count in AAI-SCD and SCD count in S-SFH SP3. Except these information is targeting only for M2M devices who are assigned by dedicated ranging, the basic operation with these fields is same to existing Configuration Change Count and SCD count as defined in 16.2.3.31.

- References

[1] IEEE C802.16p-10/0018r2, “P802.16p Amendment Working Document (AWD)”

[2] IEEE C802.16p-10/0004r2, “802.16p M2M system requirements document (SRD)”

- Text proposal for inclusion in the 802.16m amendment

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-------------------------------------------------------  Text Start  --------------------------------------------------------------

<Remedy-1>

In line 8 of page 25 in C802.16p-10/0018r2, insert the description as following

16.3.5.5.1.2 S-SFH IE

Table 835 - S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ... | ... | ... |
| M2M ranging indicator | 2 | Indicate the ranging configuration for M2M devices. 0b00: normal ranging as defined in Table 833 in 16.3.5.5.1.2 0b01: dedicated ranging for M2M devices 0b10: allow both normal and dedicated ranging 0b11: no ranging (not allow network re-/entry) |
| If ((M2M ranging indicator == 0b01 ) or (M2M ranging indicator == 0b10 ) { |  |  |
| M2M SCD count | 4 | The value is increased whenever the contents of the dedicated ranging information for M2M devices are changed. The value rolls over from 0 to 15. The operation of this field is same with SCD count as defined in 16.2.3.31. |
| } |  |  |
| ... | ... | ... |

<Remedy-2>

In the Table 708 of page 18 in C802.16p-10/0018r2, insert the description as following

16.2.3.31 AAI-System Configuration Descriptor (SCD) Message

Table 708 - AAI-SCD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| If ((M2M ranging indicator == 0b01 ) or (M2M ranging indicator == 0b10 ) { |  |  |  |
| M2M Configuration Change Count | 4 | The value is increased whenever the contents of the dedicated raging information for M2M devices are changed. The value rolls over from 0 to 15. The operation of this field is same with Configuration Change Count as defined in 16.2.3.31. |  |
| M2M ranging opportunity index | 1 | Indicates the index of the allocated ranging opportunity of the ranging channel used in the RA-ID dedicated for M2M devices. 0b0: 0b01 0b1: 0b10 |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. |  |
| Periodicy of the M2M ranging | [3] | Indicates the periodicy of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: reserved] |  |
| } |  |  |  |
| ... | ... | ... |  |

<Remedy-3>

From the line 29 of page 22 in C802.16p-10/0018r2, insert the description as following

16.2.18.7.2 Network re-entry from idle mode for M2M devices

…

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. When BS assigns the CDMA Allocation A-MAP IEs for RNG-REQ to those M2M devices, the opportunity index in RA-ID masked for the CDMA Allocation A-MAP IEs can be set to one of opportunity index '0b01' and '0b10'. In this case, the opportunity index for assignment of the dynamic NS-RCH shall be set to the other value. The information of dedicated ranging resources is transmitted in the AAI-SCD message. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2. The configuration of ranging assignment for M2M devices is indicated through M2M ranging indicator in the S-SFH SP3.

-------------------------------------------------------  Text End  ----------------------------------------------------------------