| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast transmission end indication for M2M devices in idle mode |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes the method of indicating the end of M2M multicast data transmission for lower power consumption of M2M devices in idle mode |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multicast transmission end indication for M2M devices in idle mode

Jeongki Kim, Youngsoo Yuk, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

According to the 802.16p AWD [1], M2M devices in idle mode can receive the multicast data without any additional procedure (i.e., network reentry) by a group paging method indicating the multicast traffic existence. In this case the device will wait to receive the multicast data until next paging listening interval. This will increase the unnecessary power consumption of M2M devices in idle mode, especially in case of long paging cycle as shown in figure 1.

Figure 1

# Proposal

For lower power consumption of M2M devices in idle mode, when a BS finishes sending the multicast data, the BS will send the multicast end indication (i.e., Multicast transmission end extended header (MTEEH)) with the last multicast data to M2M devices as show in figure 2. Upon receiving the multicast end indication M2M devices in idle mode will enter the unavailable interval for saving their power.

Figure 2

# Reference

[1] IEEE 802.16p-10_0018r2, Enhancements to Support Machine-to-Machine Applications

# Text proposal

---- Start of proposed text ----

[Remedy #1: Add the Section 16.2.2.10 into the IEEE 802.16p-10/0018r2:]

Add the new section and table as indicated:

16.2.2.2.10 Multicast transmission end extended header (MTEEH)

The BS piggybacks MTEEH in the last MAC PDU carrying the multicast data to notify the end of transmission of multicast data. If an M2M device in idle mode receives the MTEEH during receiving the multicast data, the M2M device is not required to receive the multicast data and may enter the paging unavailable interval.

Table 676a—MTEEH format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MTEEH () { |  |  |
| Type | 4 | Extended header type = 0b1001 (MTEEH Type) |
| Reserved | 4 | For byte alignment |
| } |  |  |

[Remedy #2: Modify the Section 16.2.28.4.3 in IEEE 802.16p-10/0018r2 as follows.]

16.2.28.4.3 M2M Multicast operation in idle mode

An M2M BS may provide the multicast service for M2M devices in idle mode with or without requiring network reentry of the M2M devices. Before an M2M BS sends DL multicast data, the M2M BS may transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The multicast transmission start time may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of multicast transmission start time shall be less than the start time of next paging listening interval of the devices receiving the AAI-PAG-ADV message. The M2M device may power down until the frame indicated by multicast transmission start time in the AAI-PAG-ADV message.

When the multicast data transmission ends, the BS shall notify the end of multicast data transmission to M2M devices by piggybacking MTEEH in the last multicast MPDU. Upon receiving the MTEEH, the device may enter the paging unavailable interval as specified in 16.2.18.2.

[Remedy #3: Add the following section and texts in IEEE 802.16p AWD:]

6.3.2 MAC PDU formats

6.3.2.2.7 Extended subheader format

Modify the table 25 as follows

Table 25—Description of extended subheaders types (DL)

| Extended subheader type | Name | Extended subheader body size (byte) | Description |
| --- | --- | --- | --- |
| 0 | SDU_SN extended subheader | 1 | See 6.3.2.2.7.1 |
| … | … | … | … |
| 5 | PDU SN(long) extended subheader | 2 | See 6.3.2.2.7.8 |
| 6 | Multicast transmission end (MTE) extended subheader | 0 | See 6.3.2.2.7.11 |
| 67–127 | Reserved |  |  |

Insert the following texts and the table as indicated

6.3.2.2.7.11 Multicast transmission end extended subheader

The BS piggybacks the multicast transmission end extended subheader in the last MAC PDU carrying the multicast data to notify the end of transmission of multicast data. If an M2M device in idle mode receives the multicast transmission end extended subheader during receiving the multicast data, the M2M device is not required to receive the multicast data and may enter the paging unavailable interval.

Table xxx—MTE extended subheader

| Name | Size (bits) | Description |
| --- | --- | --- |

[Remedy #4: Modify the following sections and texts in IEEE 802.16p AWD as follows:]

6.3.28 Support of multicast operation for machine to machine application

6.3.28.1 M2M multicast operation in idle mode

A BS may provide a multicast service for M2M devices in idle mode with or without requiring network reentry of the M2M devices. Before a BS sends DL multicast data, the BS may transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The Multicast transmission start time TLV may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of Multicast transmission start time TLV shall be less than the start time of the next paging listening interval of the devices receiving the MOB_PAG-ADV message. The M2M device may power down until the frame indicated by the Multicast transmission start time TLV in the MOB_PAG-ADV message.

When the multicast data transmission ends, the BS shall notify the end of multicast data transmission to M2M devices by piggybacking multicast transmission end extended subheader in the last multicast MPDU. Upon receiving the multicast transmission end extended subheader, the device may enter the paging unavailable interval as specified in 16.2.18.2.

---- End of proposed text ----