| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast transmission end indication for M2M devices in idle mode |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics Ming-Hung Tao ITRI | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes the method of indicating the end of M2M multicast data transmission for lower power consumption of M2M devices in idle mode |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multicast transmission end indication for M2M devices in idle mode

Jeongki Kim, Youngsoo Yuk, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

Ming-Hung Tao

ITRI

# Introduction

According to the 802.16p AWD [1], M2M devices in idle mode can receive the multicast data without any additional procedure (i.e., network reentry) by a group paging method indicating the multicast traffic existence. In this case the device will wait to receive the multicast data until next paging listening interval. This will increase the unnecessary power consumption of M2M devices in idle mode, especially in case of long paging cycle as shown in figure 1.

Figure 1

# Proposal

For lower power consumption of M2M devices in idle mode, when a BS finishes sending the multicast data, the BS will send the multicast end indication (i.e., Multicast transmission end indication MAC control message (AAI-MTE-IND). BS will usually send the AAI-MTE-IND message when the BS does not receive the multicast data during specific time. Upon receiving the multicast end indication M2M devices in idle mode will enter the unavailable interval for saving their power.

Figure 2

# Reference

[1] IEEE 802.16p-10_0018r2, Enhancements to Support Machine-to-Machine Applications

# Text proposal

---- Start of proposed text ----

[Remedy #1: Add the Section 16.2.2.10 into the IEEE 802.16p-10/0018r2:]

Add the new section and table as indicated:

16.2.3.xx AAI-MTE-IND (Multicast transmission end indication)

The ABS may send AAI-MTE-IND message to M2M devices to indicate the end of multicast transmission. If an M2M device in idle mode receives the AAI-MTE-IND message, the M2M device is not required to receive the multicast data and may enter the paging unavailable interval as specified in 16.2.18.2.

Table xxx—AAI-MTE-IND Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID | 4 | Flow ID related to the terminated multicast service |  |

[Remedy #2: Modify the Section 16.2.28.4.3 in IEEE 802.16p-10/0018r2 as follows.]

16.2.28.4.3 M2M Multicast operation in idle mode

An M2M BS may provide the multicast service for M2M devices in idle mode with or without requiring network reentry of the M2M devices. Before an M2M BS sends DL multicast data, the M2M BS may transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The multicast transmission start time may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of multicast transmission start time shall be less than the start time of next paging listening interval of the devices receiving the AAI-PAG-ADV message. The M2M device may power down until the frame indicated by multicast transmission start time in the AAI-PAG-ADV message. When the multicast data transmission ends, the BS shall notify the end of multicast data transmission to M2M devices by sending the AAI-MTE-IND message. Upon receiving the AAI-MTE-IND message, the device may enter the paging unavailable interval as specified in 16.2.18.2.

If the MTST is included in the AAI-PAG-ADV message, M2M devices receiving the AAI-PAG-ADV message shall start the Multicast Traffic Reception timer at the MTST. Otherwise, the device shall start Multicast Traffic Reception timer at the frame when the AAI-PAG-ADV is received. The Multicast Traffic Reception timer shall be reset whenever the multicast data are received. The device shall stop the Multicast Traffic Reception timer when the AAI-MTE-IND message is received. If the Multicast Traffic Reception timer expires, the device may enter the paging unavailable interval as specified in 16.2.18.2.

[Remedy #3: Add the following section and texts in IEEE 802.16p AWD:]

6.3.2 MAC PDU formats

Insert the following texts and the table as indicated

6.3.2.3.xx MOB-MTE-IND (Multicast transmission end indication) message

The BS may send MOB-MTE-IND message to M2M devices to indicate the end of multicast transmission. If an M2M device in idle mode receives the MOB-MTE-IND message, the M2M device is not required to receive the multicast data and may enter the paging unavailable interval as specified in 16.2.18.2.

Table xxx— MOB-MTE-IND message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MOB-MTE-IND_Message_Format() { |  |  |
| CID | 16 | CID related to the terminated multicast traffic |
| } |  |  |

[Remedy #4: Modify the following sections and texts in IEEE 802.16p AWD as follows:]

6.3.28 Support of multicast operation for machine to machine application

6.3.28.1 M2M multicast operation in idle mode

A BS may provide a multicast service for M2M devices in idle mode with or without requiring network reentry of the M2M devices. Before a BS sends DL multicast data, the BS may transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The Multicast transmission start time TLV may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of Multicast transmission start time TLV shall be less than the start time of the next paging listening interval of the devices receiving the MOB_PAG-ADV message. The M2M device may power down until the frame indicated by the Multicast transmission start time TLV in the MOB_PAG-ADV message.

When the multicast data transmission ends, the BS shall notify the end of multicast data transmission to M2M devices by sending the AAI-MTE-IND message. Upon receiving the AAI-MTE-IND message, the device may enter the paging unavailable interval as specified in 16.2.18.2.

If Multicast transmission start time TLV is included in the MOB-PAG-ADV message, M2M devices receiving the MOB-PAG-ADV message shall start the Multicast Traffic Reception timer at the frame indicated by the Multicast transmission start time TLV. Otherwise, the device shall start the Multicast Traffic Reception timer when the device receives the MOB-PAG-ADV. The Multicast Traffic Reception timer shall be reset whenever the multicast data are received. The device shall stop the Multicast Traffic Reception timer when the AAI-MTE-IND message is received. If the Multicast Traffic Reception timer expires, the device may enter the paging unavailable interval as specified in 16.2.18.2.

---- End of proposed text ----