| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reporting MIMO feedback information for fixed M2M device in 802.16-2009 |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Jeongki Kim, Woobong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes texts about UL MIMO feedback for fixed M2M devices |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reporting MIMO feedback information for fixed M2M device in 802.16-2009

Jeongki Kim, Woobong Lee, Youngsoo Yuk, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

At last meeting, the method of reporting the MIMO feedback information by fixed M2M device was defined for optimizing the network reentry procedure in IEEE 802.16m [1]. This contribution is to propose the texts about the method of reporting the MIMO feedback information by fixed M2M device in IEEE 802.16-2009 like that of IEEE 802.16m.

# Reference

[1] C802.16p-11_0115r2, Reporting channel status for fixed M2M device

# Text proposal

---- Start of proposed text ----

[Remedy #1: Add the following texts into C802.16p-10/0018r1 as follows.]

6.3.2.3.5 RNG-REQ (ranging request) message

Add the following texts at the end of the Section 6.3.2.3.5 as indicated

The following TLV parameters shall be included in an RNG-REQ message when the Matrix A or Matrix B is supported by fixed M2M device:

MIMO Feedback information

This TLV includes the 1 bit Matrix indicator indicating the preferred STC/MIMO matrix and 4-bits DL Effective CINR as defined in Table 520.

[Remedy #2: Add the following texts and table into C802.16p-10/0018r1 as follows.]

11.5 RNG-REQ management message encodings

Add a new parameter at the end of Table 568 as indicated

Table 584—RNG-REQ message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| MIMO feedback information | 41 | 1 | Bit #0: Matrix indicator. This field suggests the preferred STC/MIMO matrix for the MS: 0b0: Matrix A 0b1: Matrix B Bit #1~Bit#4: DL effective CINR as defined in Table 520 Bit #5-Bit#7: reserved | All |

---- End of proposed text ----