| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the M2M dedicated ranging region allocated by UL-MAP IE in WirelessMAN-OFDMA |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Jeongki Kim, Jinsoo Choi, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes texts about the clarification of the M2M dedicated ranging region allocated by UL-MAP IE in 16e |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the M2M dedicated ranging region allocated by UL-MAP IE in WirelessMAN-OFDMA

Jeongki Kim, Jinsoo Choi, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

According to 802.16p AWD [1], BS may assign the ranging resources dedicated for M2M devices. At last meeting, the new extended UL MAP IE (i.e., M2M Ranging Allocation UL-MAP Extended IE) was defined to allocate the M2M dedicated ranging region in 802.16-2009 [2].

However, there are two problems on the method as follows.

- The legacy station (i.e., SSs/MSs in 16e) cannot know the location of M2M dedicated ranging region if the MS cannot parse new extended MAP IE. In this case some MSs may use wrong UL resource to send UL burst.

- Because M2M Dedicated Ranging Allocation Indicator is included in M2M Ranging Allocation UL-MAP Extended IE format, the M2M device cannot decide to use the ranging region allocated by OFDMA UL-MAP IE with broadcast CID and UIUC=12 until the device receives the first M2M Ranging Allocation UL-MAP Extended IE.

# Proposal

  - Suggestion 1: Use the UL-MAP IE with UIUC = 12 instead of M2M Ranging Allocation UL-MAP Extended IE to allocate the M2M dedicated ranging region and in this case, use new M2M dedicated ranging CID instead of broadcast CID

    - M2M dedicated ranging CID is included in UL-MAP IE when the BS allocates the M2M dedicated ranging region by using UL-MAP IE

    - M2M dedicated ranging CID is delivered to the M2M devices via UCD message

  - Suggestion 2: Move the M2M Dedicated Ranging Allocation Indicator to UCD

# References

[1] IEEE 802.16p-10_0018r2, Enhancements to Support Machine-to-Machine Applications

[2] C802.16p-11/0090r2, M2M ranging channel indication using UL-MAP for M2M in 16e

# Text proposal

---- Start of proposed text ----

[Remedy #1: Add the following new parameters at the end of Table 568 as indicated.]

Table568—UCD channel encodings

| Name | Type(1byte) | Length | Value |
| --- | --- | --- | --- |
| M2M dedicated ranging CID | x | 2 | This CID shall be used when BS allocates the ranging region dedicated for M2M devices by using UL-MAP IE with UIUC set to 12. |
| M2M Dedicated Ranging Region Indicator | y | 1 | Bit 0, When this bit is set to 1, M2M devices shall not use the ranging allocation defined by OFDMA UL-MAP IE format with UIUC=12 and broadcast CID Bits 1-7: Reserved |

[Remedy #2: Add the following table into the section 6.3.2.3.26 in C802.16p-10/0018r2 as follows.]

6.3.23.10.1 Network reentry from idle mode for M2M devices

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in 6.3.10.3.

The ranging resources dedicated for M2M devices may be assigned by using UL-MAP IE (UIUC=12) with M2M dedicated ranging CID. In this case, the M2M dedicated ranging CID shall be broadcast by using UCD message.

[Remedy #3: Delete the Section 8.4.5.4.4.30 M2M Ranging Allocation UL-MAP Extended IE format in 802.16p AWD.]

---- End of proposed text ----