| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the M2M dedicated ranging region allocated by UL-MAP IE in WirelessMAN-OFDMA |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Jeongki Kim, Jinsoo Choi, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics Erik Colban Huawei Technologies Hyunjeong Kang Samsung Electronics | Email: {jeongki.kim, js.choi}@lge.com ecolban@huawei.com hyunjeong.kang@samsung.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes texts about the clarification of the M2M dedicated ranging region allocated by UL-MAP IE in 16e |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the M2M dedicated ranging region allocated by UL-MAP IE in WirelessMAN-OFDMA

Jeongki Kim, Jinsoo Choi, Jin Lee, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

Erik Colban

Huawei Technologies

Hyunjeong Kang

Samsung Electronics

# Introduction

According to 802.16p AWD [1], BS may assign the ranging resources dedicated for M2M devices. At last meeting, the new extended UL MAP IE (i.e., M2M Ranging Allocation UL-MAP Extended IE) was defined to allocate the M2M dedicated ranging region in 802.16-2009 [2].

However, there are two problems on the method as follows.

- Problem-1: The length field in the new extended MAP IE indicates the size of MAP IE but it doesn’t indicate the actual resource region. Thus MSs (16e SSs/MSs) cannot know the location of M2M dedicated ranging region if the MS cannot parse new extended MAP IE. In this case some MSs may use wrong UL resource to send UL burst because the existing UL-MAP IE for MSs just contains duration field for resource assignment. Even if putting the ranging region to behind of MSs’ UL bursts, there is critical restriction on the resource utilization as shown in following exemplary figure (BS cannot assign MS’s UL burst in the oblique line).

Figure-1: An example of resource utilization with M2M ranging region

- Problem-2: Because M2M Dedicated Ranging Allocation Indicator is included in M2M Ranging Allocation UL-MAP Extended IE format, the M2M devices cannot decide whether they are allowed to use the ranging region allocated by OFDMA UL-MAP IE format (with UIUC=12) until the M2M devices receives the first M2M Ranging Allocation UL-MAP Extended IE, i.e., when M2M devices who will be assigned by dedicated ranging, they can try to use the ranging region allocated by aforementioned UL-MAP IE which will affect on the existing MSs’ ranging performance. In addition, because of the detection capability of BS for ranging signals, it would be difficult to control the ranging load in short term.

# Proposal

  - Suggestion 1: When BS transmits M2M Ranging Allocation UL-MAP Extended IE format for M2M devices, the BS should transmit UL-MAP IE of UIUC=12 to MSs (which indicates the M2M ranging region) by setting dedicated ranging indicator = 1. A MS can ignore the resource region indicated by this MAP IE if BS doesn’t transmit any dedicated ranging information in MOB_PAG-ADV or MOB_SCN-RSP message to the MS.

  - Suggestion 2: It is recommended to move the M2M Dedicated Ranging Allocation Indicator in M2M Ranging Allocation UL-MAP Extended IE format to UCD. No change about original description.

# References

[1] IEEE 802.16p-10_0018r2, Enhancements to Support Machine-to-Machine Applications

[2] C802.16p-11/0090r2, M2M ranging channel indication using UL-MAP for M2M in 16e

# Text proposal

---- Start of proposed text ----

Remedy #1: After the line 59 in page 3 of C802.16p-10/0018r2, add the description as following

6.3.23.10.1 Network reentry from idle mode for M2M devices

BS may assign M2M specific ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. See 8.4.5.4.4.30 and 11.3.1, Table 568. In this case, M2M devices perform ranging for network (re-)entry using the dedicated ranging resources. If the BS does not assign dedicated ranging resources, M2M devices shall perform ranging for network (re-)entry using the ranging resources defined as specified in 6.3.10.3.

When the BS transmits an M2M Ranging Allocation UL-MAP Extended IE, the BS shall also transmit a UL-MAP IE with UIUC=12 identifying the same region as the M2M Ranging Allocation UL-MAP Extended IE and with the dedicated ranging indicator set to 1.

---- End of proposed text ----