| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging to poll M2M uplink transmission in WirelessMAN OFDMA system |  |
| Date Submitted | July 10, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak LG Electronics | {jin1.lee, youngsoo.yuk, kiseon.ryu}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose a paging message to be used for uplink data transmission and its related operation in WirelessMAN OFDMA system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging to poll uplink data transmission in WirelessMAN OFDMA system

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, JinSam Kwak

LG Electronics

# Background

As we agreed at the last session, the paging message can be used to poll M2M devices for uplink transmission. However, for 16p we also agreed a group paging to reduce paging overhead for large number of devices. So some parameters shall be included either for an individual paging or a group paging. Also, MOB_POLL-ADV message is optimized by removing unnecessary parameters.

Additionally, this contribution proposes a new parameter, ‘reception of polling indicator’ in RNG-REQ message. This is for an M2M device to indicate missing of polling within the Max number of paging cycle. By doing so, the BS can realize a ranging request from the device is reasonable to be accepted even if the transmission type of the device is set to 1 (means the device shall receive polling in order to send uplink data during idle mode).

# Text Proposal

[Remedy1:] Modify texts as following in page 2 line 45:

[Note - Remedy1: Since M2M_POLL_ADV is used to poll some M2M devices (by including MAC Address hash) which are allowed to send uplink data at the paging cycle, it is enough to include those MAC address hash without additional action code. In that case, if an M2M device finds its MAC address hash in MOB_POLL-ADV message, it considers that it can send uplink data during this paging cycle.

6.3.2.3.60 M2M_POLL-ADV (M2M broadcast paging) message

An MOB_POLL-ADV message may be transmitted by a BS to indicate periodic uplink data transmission from M2M devices in idle mode. The M2M_POLL-ADV message shall be sent on the Broadcast CID or Idle Mode Multicast CID during the BS paging interval. The M2M_POLL-ADV is to be broadcast on the same downlink frame as MOB_PAG-ADV., so the devices can receive the message after waking up from the idle mode. If an M2M_POLL-ADV message includes M2M device MAC Address hash of the M2M device, the M2M device considers that the BS triggers uplink data transmission.

The format of the message is shown in Table 166a.

Table 166a-M2M_POLL-ADV message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M_POLL-ADV Message_format () { |  |  |
| Management Message Type = 70 | 8 |  |
| Num_M2M_Devices | 8 | Num of M2M devices |
| For (i=0;i<Num_M2M_Devices;i++) |  |  |
| M2M device MAC Address hash | 24 | The hash is obtained by computing a CRC 24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB. |
| M2M action code | 1 | Indicated the opportunity for the M2M device to send the uplink data 0b0: No action required 0b1: Send uplink data |
| Reserved |  |  |
| } |  |  |

[Remedy2 :] Modify texts as following in page 8 line 26 :

[Note – Remedy2]: Action code for uplink report shall be included for a group paging case as well.

11.17.3 M2M group paging parameter

The following M2M group paging parameter TLV may be included in MOB_PAG-ADV message.

The following TLV element shall appear in each M2M group paging parameter TLV

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| MGID | 153.1 | 2 | Bit 0-14: Indicates M2M Group ID: Bit 15: Padding. Will be set to 0 |
| Action code | 153.2 | 1 | Bit0-1 : indicates Action code for the M2M Group ID 0b00 – Performing network reentry 0b01 – Performing location update 0b10 – Receiving multicast traffic 0b11 – Reserved Bit3 – Indication of the opportunity for the M2M group to send the uplink data 0b0: Reserved 0b1: Send uplink data Bit 4-7: Padding. Will be set to 0 |

[Remedy3] Add new subclause 6.3.23.6, in page 3 line 31 :

6.3.23.6 BS Broadcast Paging message

For a BS Broadcast Paging message to be transmitted to indicate the presence of DL traffic pending, there shall be at least a packet in the DL traffic whose Paging Preference indicates paging generation.

MOB_PAG-ADV may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed devices. The interval of periodic uplink data transmission should be longer than or equal to the paging cycle. When an M2M device receives the DREG-CMD message with the Transmission Type set to 1 and Max number of paging cycle TLV, the M2M device shall wait for the MOB_PAG-ADV as long as Max number of paging cycle * paging cycle before sending uplink transmission.

If the M2M device does not receive at least one MOB_POLL-ADV or MOB_PAG-ADV with Bit 3 set to1 within the Max number of paging cycle * paging cycle, it may send RNG-REQ message with reception of paging message indicator Bit0 set to 1.

[Remedy 4] Add a new section at the end of 11.5, in page 7 line 38

11.5 RNG-REQ management message encodings

| Name | Type (1 byte) | Length | Value (variable length) | Scope |
| --- | --- | --- | --- | --- |
| Reception of polling indicator | 24 | 1 | Bit 0: indicates that the M2M device does not receive at least one MOB_POLL-ADV including its MAC Address hash or MOB_PAG-ADV with Bit 3 set to1 prior to expiration of Max time duration. The Max time duration is transmitted in DREG-CMD. Bit 1-7 : reserved | - |

[Remedy 5] Add a new row at the end of 11.14 in page 7 line 57

[Note – Remedy5]: transmission type and Max number of paging cycle (which are already used for 16m M2M devices) shall be also included for OFDMA M2M devices.

11.14 DREG-CMD/REQ message encodings

| Name | Type (1 byte) | Length | Value (variable length) | Scope |
| --- | --- | --- | --- | --- |
| M2M device specific Idle mode timer | 53 | 3 | Length of the maximum interval between location update while the M2M device in idle mode | DREG-CMD |
| Transmission Type | 54 | 1 | Bit 0: allowed to send data only after receiving paging message or MOB_POLL-ADV Bit1-7: reserved. | DREG-CMD |
| Max number of paging cycle | 55 | 8 | This is max number of paging cycle for M2M device to wait for M2M_POLL-ADV or paging message Bit 3 set to 1. The unit is the duration of the paging cycle. | DREG-CMD |

# References

[1] IEEE P802.16m/D12

[2] IEEE C802.16p-11/0067r1