| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging to poll M2M uplink transmission in WirelessMAN OFDMA system |  |
| Date Submitted | July 20, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak LG Electronics Joey Chou Intel | {jin1.lee, youngsoo.yuk, kiseon.ryu}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose a paging message to be used for uplink data transmission and its related operation in WirelessMAN OFDMA system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging to poll uplink data transmission in WirelessMAN OFDMA system

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, JinSam Kwak

LG Electronics

Joey Chou

Intel

# Background

This contribution is to clarify the operation using M2M_POLL-ADV message. But here MOB_PAG-ADV replaces M2M_POLL-ADV. Editorial changes are made in AAI-PAG-ADV message as well.

# Text Proposal

[Remedy1:] Remove the entire section 6.3.2.3.60:

[Remedy2:] Modify table 154 as following in page 2 line 44:

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message –

Table 154— MOB_PAG-ADV message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| : |  |  |
| For (j = 0; j < Num_MACs; j++) { | – | – |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved |
| M2M report code | 1 | Action instruction to M2M device 1: Indication for the M2M device to send the uplink report |
| Reserved | 65 |  |
| } |  |  |
| : |  |  |

[Remedy3] Add new subclause 6.3.23.6, in page 3 line 31 :

6.3.23.6 BS Broadcast Paging message

For a BS Broadcast Paging message to be transmitted to indicate the presence of DL traffic pending, there shall be at least a packet in the DL traffic whose Paging Preference indicates paging generation.

MOB_PAG-ADV with M2M report code may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed devices. When a device receives the DREG-RSP message with the Transmission Type set to 1 and Max number of paging cycle TLV, the device may wait for the AAI-PAG-ADV with M2M report code as long as Max number of paging cycle * paging cycle before sending uplink data. If the M2M device does not receive at least one MOB_PAG-ADV with M2M report code within Max number of paging cycle * paging cycle, it should not send the uplink.

[Remedy 4] Add a new row at the end of 11.14 in page 7 line 57

11.14 DREG-CMD/REQ message encodings

| Name | Type (1 byte) | Length | Value (variable length) | Scope |
| --- | --- | --- | --- | --- |
| M2M device specific Idle mode timer | 53 | 3 | Length of the maximum interval between location update while the M2M device in idle mode | DREG-CMD |
| Transmission Type | 54 | 1 | Bit 0: allowed to send data only after receiving a paging message with M2M report code Bit1-7: reserved. | DREG-CMD |
| Max number of paging cycle | 55 | 8 | This is max number of paging cycle for M2M device to wait for MOB_PAG-ADV with M2M report code. The unit is the duration of the paging cycle. | DREG-CMD |

[Remedy5: Modify table 700 as indicated below in page 17 line 26:]

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| For (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the AMS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..218– 1 | Present if the S-SFH Network Configuration bit == 0b0 |
| MAC Address Hash | 24 | Used to identify the AMS to be paged | Present if the S-SFH Network Configuration bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08–0x15: Reserved | Present if the S-SFH Network Configuration bit == 0b0 |
| Action Code | 1 | Used to indicate the purpose of the AAIPAG- ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| M2M Report code | 1 | Indication for the M2M device to send the uplink report 0b0 : reserved 0b1 : Send uplink report | Present if M2M is supported |
| } |  |  |  |
| } |  |  |  |
| : |  |  |  |
| Action Code | 2 | 0b00: Performing network reentry 0b01 : Performing location update 0b10 : Receiving multicast traffic 0b11 : reserved |  |
| M2M Report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0 : No action required 0b1 : Send uplink report | Present if M2M is supported |

# References

[1] IEEE P802.16m/D12

[2] IEEE C802.16p-11/0067r1