| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging to poll M2M uplink transmission in WirelessMAN IMT-Advanced system |  |
| Date Submitted | July 10, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak LG Electronics | {jin1.lee, youngsoo.yuk}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose a paging message to be used for uplink data transmission and its related operation in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging to poll uplink data transmission in WirelessMAN IMT-Advanced system

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, JinSam Kwak

LG Electronics

# Background

According to 16p AWD, a paging message can be used to poll M2M devices to trigger uplink data transmission. By doing this, a large number of accesses from M2M devices can be controlled by BS. For example, BS may not poll an M2M device at every paging cycle based on its load status. The M2M device should wait for the paging message until the Max time duration. However, if the BS does not poll the M2M device continuously due to its high load status or any other reasons, the device can send a ranging request without even receiving the polling indication. Therefore, this contribution proposes the indication about expiration of the Max time duration so that the BS identifies that the ranging request from the device is reasonable to be accepted.

Fig1 shows how the ‘Reception of polling indicator’ is used. Assume that M2M device is set its transmission type to 1 in AAI-DREG-RSP so the device is supposed to send uplink data after receiving an AAI_PAG-ADV with report code 01. However, if the ABS does not poll the device at least one within the Max time duration (transmitted in AAI-DREG-RSP), then the device may send an AAI-RNG-REQ with ‘Reception of polling indicator’ set to 1. In that case, the ranging request should be considered with priority and be accepted by the ABS.

Fig1. The usage of ‘Reception of polling indictor’ in AAI-RNG-RSP

# Text Proposal

[Remedy1: Modify texts as following in page 22 line 19:]

16.2.18.7.1 Paging operation

AAI-PAG-ADV with M2M report code set to 0b1 may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed devices. The interval of periodic uplink data transmission should be longer than or equal to the paging cycle. When a device receives the AAI-DREG-RSP message with the Transmission Type set to 1 and Max number of paging cycle attribute, the device may should wait for the AAI-PAG-ADV with M2M report code = 0b1 at as long as Max number of paging cycle * paging cycle before sending uplink data. If the device does not receive the AAI_PAG-ADV with M2M report code =1, it should not send the uplink.

If the M2M device does not receive at least one paging message with M2M report code set to 0b1 within the Max number of paging cycle * paging cycle, it may send AAI-RNG-REQ message with reception of paging message indicator set to 1.

[Remedy2: Modify texts as following in page 11 line 15:]

16.2.3.1 AAI-RNG-REQ

Table 678 – AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| : |  |  |  |
| Bandwidth Request Indicator | 1 | 1 : indicates BW grant is required for transmission of BR header after completion of network reentry |  |
| Reception of polling indicator | 1 | 1: indicates expiration of maximum duration (Max number of paging cycle* paging cycle) for M2M device to wait for AAI-PAG-ADV (M2M report code set to 0b1) | Present if needed |
| } else if (Ranging Purpose Indication == 0b0011 ¦ 0b0110 ¦0b0111 ¦ 0b1011) { |  | //Idle mode location update ( and with other additional purposes) |  |

[Remedy3: Modify table 699 as indicated below in page 14 line 45:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| REG-Duration | 8 | Used to indicate waiting value for the AAI-DREG-REQ message with De-registration_Request_Code=0x01 0..-1 : measured in frames | Present if needed |
| M2M device-specific Idle Mode Timer | 24 | Length of the maximum interval between location update while the M2M device in idle mode | May present when the M2M device enters idle mode |
| Transmission Type | 1 | 0 : reserved 1 : Allowed to send data only after receiving paging message with M2M report code 0b1. | Present if needed |
| Max number of paging cycle | 8 | This is for M2M device to wait for AAI-PAG-ADV with M2M report code 0b1. See 16.2.18.7.1. The unit is the duration of the paging cycle. | Present if Transmission Type is set to 1 |
| } |  |  |  |
| : |  |  |  |

# References

[1] IEEE P802.16m/D12

[2] IEEE C802.16p-11/0067r1