| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Small Data Transmission from idle mode M2M device in WirelessMAN OFDMA system |  |
| Date Submitted | July 20, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , Inuk Jung, JinSam Kwak LG Electronics Honggang Li, Rui Huang, Shantidev Mohanty, Xiangying Yang Intel Hyunjeong Kang Samsung | {jin1.lee, youngsoo.yuk}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose small burst transmission from idle mode M2M devices in WirelessMAN OFDMA system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Small Data Transmission from idle mode M2M device in WirelessMAN OFDMA system

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, Inuk Jung, JinSam Kwak

LG Electronics

Honggang Li,Rui Huang,Shantidev Mohanty,Xiangying Yang

Intel

Hyunjeong Kang

Samsung

# Background

According to 16p SRD, 16p system shall minimize signaling overhead and provide efficient security method for small burst transmission. Based on this requirement, the contribution proposes an M2M device in idle mode sends the small data burst piggybacked in RNG-REQ or receives the small data burst piggybacked in RNG-RSP. This solution is mostly consistent with the existing SMS transmission scheme in WirelessMAN IMT-Advanced system.

# Proposal

As we have shown at the last meeting [3], this proposal follows the similar way to SMS Transmission we defined in 16m standard. Since SMS is just encapsulated in RNG msg, the SMS contents may be encrypted by using some end-to-end application layer encryption.

# Fig1. M2M uplink SMS Tx during idle mode             Fig2. M2M downlink SMS Tx during idle mode

# =====================Start Texts Changes ============================

# Text proposal

[Remedy1 : Modify table 582, in page 7 line 37:]

11.5 RNG-REQ management message encodings

Table 582—RNG-REQ message encodings

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M2M SMS Request | 24 | 1 | Bit 0-7: No.of bytes of SMS message | OFDMA |
| M2M SMS | 25 | Variable | M2M SMS message content up to 140bytes Padding bits to align boundary of byte. | OFDMA |
| M2M SMS Confirmation | 26 | 1 | Bit 0: SMS confirmation 0 – NACK 1 –ACK Bit 1-7: Reserved | OFDMA |

[Remedy2: Modify Table585, in page 7 line 38]

11.6 RNG-RSP management message encodings

Table 585—RNG-RSP message encodings

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M2M SMS Response | 41 | 1 | Bit 0-1: accept or reject SMS request 0b0: reject 0b1: accept If(reject){ Bit 2-3: action code 0b00: network re-entry 0b01-0b11: reserved } else { Reserved } Bit 4-7: Reserved | OFDMA |
| Temp CID Timer | 42 | 1 | Life time duration for the Basic CID assigned by BS | OFDMA |
| M2M SMS Confirmation | 43 | 1 | Bit 0: SMS confirmation b0 – NACK b1 –ACK Bit 1-7: Reserved | OFDMA |
| M2M SMS | 44 | Variable | M2M SMS message content up to 140 bytes Padding bits to align boundary of byte. | OFDMA |

[Remedy3: Add a new section 6.3.29 in page 4 line 20]

6.3.29 M2M small burst transmission

To support UL SMS from an M2M device in idle mode, two-round RNG-REQ/RSP are proposed as follows:

If the M2M device identifies the bandwidth allocation is enough for piggybacking SMS contents in RNG-REQ from the UL-MAP, it will omit the 1st round of RNG-REQ/RSP and send the RNG-REQ piggybacking SMS directly in second round of RNG-REQ.

The M2M device sends the RNG-REQ with M2M SMS Request TLV to indicate it has a SMS to send. If the BS receives the RNG-REQ with M2M SMS Request TLV successfully, it may accept or reject the request. In this case the BS shall transmit a RNG-RSP with SMS Response TLV with an action code instructing the M2M device how to proceed. If the BS accepts the SMS Request, the BS shall transmit a RNG-RSP with SMS Response TLV, with a Basic CID and a Temp CID Timer to be used for resource allocation for SMS transmission. For fixed M2M device, the M2M device can send RNG-REQ for SMS with the purpose indication for location update, but the paging related parameters can be removed from the RNG-REQ message to reduce the overhead because the BS is aware of its mobility information. This concludes the first round of RNG-REQ/RSP.

If the M2M device receives a RNG-RSP rejecting its SMS Request, it shall proceed according to the action code. If the M2M device receives a RNG-RSP accepting its SMS Request, it shall wait for bandwidth allocation for RNG-REQ with M2M SMS on its Basic CID and send a RNG-REQ with an M2M SMS TLV. If SMS packet is received successfully, the BS sends RNG-RSP with SMS confirmation to indicate. This concludes the second round of RNG-REQ/RSP.

The Basic CID is released once the M2M device receives the SMS Confirmation, or when the Temp CID Timer expires.

DL SMS TLV may be included in RNG-RSP message when the action code of MOB_PAG-ADV indicates location update.

For DL SMS transmission, the BS should send a Basic CID and a Temp CID Timer. When the M2M device receives the RNG-RSP with the DL SMS, a Basic CID and the Temp CID Timer, it may wait for bandwidth allocation for the RNG-REQ on the Basic CID. When SMS packet is received successfully, a RNG-REQ message is sent as a confirmation of the DL M2M SMS from the M2M device.

# =====================End Texts Changes ============================

# References

[1] IEEE M2M system requirement document, 80216p10_0004r2

[2] IEEE P802.16m/D12

[3] IEEE C80216p-11_0089r6