| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Initiated NE with ranging parameters |  |
| Date Submitted | July 15, 2011 |  |
| Source(s) | Jin Lee, Jinsoo Choi, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, Jin Sam Kwak LG Electronics | {jin1.lee, js.choi, youngsoo.yuk}@lge.com |
| Re: | Call for Comments on Network Entry for the M2M Amendment Working Document (AWD) |  |
| Abstract | This is to propose uniform access distribution and the specific contention resolution for network initiated network entry from M2M devices |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Initiated NE with ranging parameters

Jin Lee, Jinsoo Choi, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, JinSam Kwak

LG Electronics

# Background

Network initiated network entry results from a paging indication which means network can anticipate how many devices would perform network re-entries. In that case, network can assist entry by signaling some ranging parameters. Here, in this contribution we propose ‘waiting offset time’ to differentiate ranging start time for each M2M group and a uniform distribution of ranging opportunity based on the initial ranging window parameter assigned per an M2M group. Additionally, the different backoff window size can be used if the contention occurs. Detail procedure will be followed. We also show how much performance gain we can expect by the suggested remedies based on the network entry evaluation ad-hoc guide [3].

# Proposal

- Proposal-1: For initial ranging procedure

  - Waiting offset time

This parameter is to distribute the start time of ranging for M2M groups, which means each M2M group paged may have different waiting offset time. Since it is not guaranteed each M2M group has a different paging offset, somehow M2M groups paged with the same paging offset shall be distributed when to start ranging. Therefore, this waiting offset time may be included in a paging message to reduce access collision among M2M groups.

  - M2M Device specific ranging parameter

This parameter is assigned by BS during initial network entry to differentiate each M2M device for the uniform distribution. The value may be unique within an M2M group but not always be assigned in order within the same M2M group.

  - Uniform access distribution

Although the waiting offset time is applied to M2M groups, we still see the problem how to reduce access collision from M2M devices belonging to an M2M group. Since the BS may be able to predict how many M2M devices need to perform ranging, here we suggest uniform distribution of ranging opportunities, by the equation of M2M device specific ranging parameter mod K, K is the distinct value transmitted by a paging message. Here, K may be variable based on the number of devices required ranging per M2M group.

Fig-1. Uniform access distribution

  - Performance evaluation

For performance comparison between conventional and proposed method, basic evaluation assumptions follow the guideline document [3]. Beside the guideline, followings are more assumed for the details.

- The number of groups per sector: 30, 60

- Waiting offset time: 5s time gap between each group’s paging cycle (1000frames, assuming each group doesn’t overlap and access events from M2M devices for a group occurs within this time gap)

- Access rate: 40 per sec

- Initial backoff window size of conventional method: 2 (with increasing by 2x factor, x is the number of the ranging retrials)

- K size of the proposed method: 64 (equal size through all ranging retrials), 256 (with decreasing by 2x factor). The opportunity selection of the proposed scheme for ranging retrials is same to the conventional random backoff selection, i.e. uniform access distribution is only applied to the first ranging transmission.

The Figure-2 show the distribution on the ranging transmissions from M2M devices for the conventional random backoff scheme (a) and the proposed uniform access distributed scheme (b) , i.e., backoff window size = 2, 4, 8, 16, 32, 64, 128, 256, 512 for (a) and K = 64 through all ranging retrials for (b), respectively, as the number of ranging retrial increases. The (b) doesn’t exceed the latency of (a) to guarantee maximum 8 ranging retransmissions.

(a) Conventional random backoff scheme	       (b) Proposed uniform access distributed scheme

Figure-2: The distribution on ranging trials per each opportunity for a group

Figure-2 shows the proposed scheme distributes the backoff selection over more opportunities and reduces the number of ranging retrials which will result in reduced ranging collision among different M2M devices. This can achieve more ranging access successes with the given backoff window latency as shown in the following Table-1.

Table -1: The performance comparison between the conventional and the proposed method

| Performance metric | Conventional (30 groups) | Uniform distributed (30 groups) | Conventional (60 groups) | Uniform distributed (60 groups) |
| --- | --- | --- | --- | --- |
| # of ranging occurrence | 4721 | 4721 | 7583 | 7583 |
| # of successful ranging | 1820 | 1884 | 2994 | 3526.8 |
| # of failed ranging | 2901 | 2837 | 4589 | 4056.5 |
| # of average trials for successful ranging | 4.5 | 2.5 | 4.2 | 2.2 |

- Proposal-2: For the restart of ranging procedure (after ranging fails)

  - Backoff window size

When an M2M device restarts ranging procedure after RNG-ACK reception timer expiration, the M2M device will follow the existing backoff reselection rule. However, unlike the existing AMSs’ network access distribution, there are a lot of trials on network access from M2M devices so the existing backoff selection rule might not be proper way to support so many M2M devices’ accesses. The important thing is that a lot of M2M devices’ access requires longer initial backoff window size because the failed ranging events will be connected by the next ranging retrial which can cause the collision among ranging trials from different M2M devices.

This is shown in performance results in the Table-1. And, from the Figure-2, the distribution still shows opportunities more gathering in the front part. Because the failed ranging will cause the next ranging retrials, it can increase collision with ranging transmissions from other M2M devices. If we can set the initial backoff window as longer size and decrease the window size through the ranging retransmissions with maintaining latency, then it is expected that we can reduce the aforementioned collision by ranging retrials among M2M devices.

In this contribution, we propose following backoff window size with retrials of the ranging procedure.

The backoff window size in ranging restart procedure = The initial backoff window size (K) / 2x, x is the number of ranging retrials

  - Alignment of start point of backoff window

If the opportunity which is selected in the initial or previous ranging procedure is located in front part of the window size, the next selected opportunity with its retrial can be easily overlapped with opportunities selected by other M2M device which is located in latter part of the window size, because the proposed backoff window size decreases per each ranging retrial. So it is expected that we can get more access success gain if we provide some delay (including RNG-ACK reception time) and make an alignment for the start point of the backoff window to avoid aforementioned overlapping as shown in following Figure-3.

Figure-3: The backoff selection with ranging retrials

The backoff selection method for ranging restart procedure (from the second ranging retransmission) is same to conventional random backoff selection within given window size.

  - Performance evaluation

The evaluation assumptions are same with the Proposal-1 case. Figure-4 shows the distribution on the ranging trials from M2M devices for the proposed backoff window (a) and backoff window + start point alignment (b), respectively. The proposed method with the alignment of backoff window start point can effectively reduce the aforementioned collision occurrence and it results in the best performance as represented in the following Table-2. Compared to the performance of the conventional scheme as shown in Table-1, the proposed method shows significant successful access performance. Notice that this proposed method still doesn’t exceed the latency of the conventional method to guarantee the given number of maximum ranging retrials.

(a) Proposed backoff window	            (b) Proposed backoff window and aligned start point

Figure-4: The distribution of ranging trials per each opportunity for a group

Table -2: The performance results for the proposed method with ranging retrials

| Performance metric | Proposed backoff window (30 groups) | Proposed backoff window with aligned start point (30 groups) | Proposed backoff window (60 groups) | Proposed backoff window with aligned start point (60 groups) |
| --- | --- | --- | --- | --- |
| # of ranging occurrence | 4721 | 4721 | 7583 | 7583 |
| # of successful ranging | 3607 | 4392 | 6545 | 7383 |
| # of failed ranging | 1114 | 329 | 1038 | 200 |
| # of average trials for successful ranging | 1.3 | 1.1 | 1.2 | 1.0 |

# Text Proposal

[Remedy1: Modify texts as following in page 22 line 42 : ]

16.2.18.7.2 Network re-entry from idle mode for M2M devices

For the network reentry indicated by a paging message that contains ranging configuration, the M2M device shall select a ranging opportunity according to the ranging configuration. Ranging configuration may include differentiated waiting offset time and backoff window size initial ranging window size for an M2M group.

Upon reception of the waiting offset time, the M2M devices belonging to the M2M group shall wait the period of time before selecting ranging opportunities. In this case, the M2M devices determine ranging opportunities using the following equation:

Ranging opportunity offset == M2M Device specific ranging parameter mod K, where K is the initial ranging window indicated by the paging message. The M2M Device specific ranging parameter is assigned by ABS during initial network entry.

When the M2M devices restart the ranging procedure, they shall determine the ranging opportunities within following backoff window size:

Backoff window size (Kx) = The initial backoff window size (K) / 2x, x is the number of ranging retransmissions and the minimum Kx is 2.

The start point of the backoff window shall be determined as following equation:

The start point of the backoff window (Sx) =, K0 is same with the initial backoff window size (K)

The opportunity selection for the restart of ranging procedure is same as defined in 16.2.15.3.

[Remedy2 : Insert the blue texts in table 700, in page17 line 32]

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

Table 700 – AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| For (i=0;<i<Num_MGID; i++) { |  |  |  |
| MGID | 15 | M2M Group ID |  |
| Action Code |  | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11 :reserved |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M is supported |
| If (Action Code == 0b 00) { |  |  |  |
| Waiting offset time |  | An M2M group shall not begin ranging prior to the expiration of waiting time from the reception of a paging message. | Present if needed |
| Initial ranging window (K) |  | Used to indicate ranging opportunity offset for each M2M device. See 16.2.18.7.2 | Present if needed |
| } |  |  |  |
| : |  |  |  |

# References

[1] IEEE P802.16m/D12

[2] C802.16p-11/0018r2, “IEEE P802.16p Amendment Working Document (AWD)”

[3] C802.16p-11/0126, “Evaluation Guideline for Comparison of Network Entry Solutions”

# Annex

- Dedicated ranging code and resource in OFDMA system (a)

MOB_PAG-ADV may include a ranging code and transmission opportunity for each MS to avoid access collision since BS is able to know who would perform ranging by paging indication. Therefore, we could reuse the similar way for M2M devices in case of individual paging but it may not be efficient for a group paging which could be primarily used for M2M devices. Also, this would cause tremendous signaling overhead in paging messages considering a large number of devices.

- Uniform distribution with assigned ranging parameter (b)

As this contribution suggested, a device is assigned a device specific ranging parameter once. When BS sends a paging message, the message may include initial ranging window size (k). From the signaling overhead point of view, k and waiting offset time for an M2M group are the only values to be transmitted through a paging message.

- Comparison of DL signaling overhead between (a) and (b)

We calculated the signaling overhead of the paging message when (a) and (b) are adapted to WirelessMAN IMT-Advanced system.

(a)

| AAI-PAG-ADV | Estimated Number of Bytes (Device #100) | Estimated Number of Bytes (Device #200) | Estimated Number of Bytes (Device #300) | Estimated Number of Bytes (Device #1K) |
| --- | --- | --- | --- | --- |
| - DID (18bit) - Paging cycle (4 bits)) - Action code (1bit) - Code and transmission opportunity (15bits) - Page Response window (8bits) | 38N + 8, where N is the number of devices |  |  |  |

(b)

| AAI-PAG-ADV | Estimated Number of Bytes (Group #1, #10) | Estimated Number of Bytes (Group #2, #20) | Estimated Number of Bytes (Group #3, #30) | Estimated Number of Bytes (Group #10, #100) |
| --- | --- | --- | --- | --- |
| - MGID (15bits) - Action code (2bits) - Waiting time offset (8, 16bits) - K (8bits) | 33N-8, where N is the number of M2M group. *Waiting offset time is assigned to #(N-1) M2M group |  |  |  |

Fig1. Comparison analysis between (a) and (b)           Fig2. Comparison analysis between (a) and (b)