| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dedicated ranging channel allocation |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Wei-Chieh Huang, Chia-Lung Tsai, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI Hsi- Pin Ma, Jen-Ming Wu, Yuan-Hao Huang, Yeong-Luh Ueng, Shih-Yu Chang NTHU | E-mail: aj@itri.org.tw |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes a network re-entry scheme for fixed M2M devices. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dedicated ranging channel allocation

Wei-Chieh Huang, Chia-Lung Tsai, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

Hsi- Pin Ma, Jen-Ming Wu, Yuan-Hao Huang, Yeong-Luh Ueng, Shih-Yu Chang

NTHU

In cellular network systems synchronization, including physical (PHY) layer synchronization and media access control (MAC) layer synchronization, shall be achieved before a device is allowed to access the network. For PHY synchronization, a device may perform timing and frequency synchronization, as well as power control via downlink synchronization and uplink feedback channels. For MAC synchronization, system information negotiation and registration are accomplished via network access (or network entry) processes. This contribution suggests the allocation of dedicated ranging channel for M2M devices.

# Proposed texts

5.1 Proposed Text #1

[Add the following text after line 10 in page 8.]

----------------------------------------------  Text Start -----------------------------------------------

16.3.5.5.1.2 S-SFH IE

Table 835 - S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ... | ... | ... |
| M2M ranging indicator | 1 | Indicate the ranging configuration for M2M devices. 0b0: normal ranging as defined in Table 833 in 16.3.5.5.1.2 0b1: Dedicated ranging for M2M devices |
| If ((M2M ranging indicator == 0b1) { |  |  |
| M2M synchronized ranging channel periodicity configuration | 2 | Indicates the periodicity of the RCH allocation according to Table aaa. |
| M2M non-synchronized ranging channel periodicity configuration | 2 | Indicates the periodicity of the RCH allocation according to Table aaa. |
| } | ... | ... |
| ... |  |  |

----------------------------------------------  Text End -----------------------------------------------

5.2 Proposed Text #2

[Add the following text after line 61 in page 3.]

----------------------------------------------  Text Start -----------------------------------------------

Dedicated non-synchronized ranging channel configuration

The information of the dedicated non-synchronized RCH (DNS-RCH) allocation consists of the ranging configuration with AAI subframe-offset (OSF) for ranging resource allocation in the time domain where OSF is same AAI subframe-offset of the NS-RCH defined in 16.3.8.2.4.1. The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands RSB according to the Equation (286) where IDcell is defined in 16.3.5.1.2 and RSB is LSB-CRU,FPi/4, where LSB-CRU,FPi is the number of allocated subband CRUs as defined in 16.3.7.3 for FPi corresponding to reuse 1 partition or power-boosted reuse 3 partition only if there is no reuse 1 partition

Table aaa shows the information of the DNS-RCH allocation in a regular allocation, which is indicated by the S-SFH. NUL denotes the number of UL AAI subframe per frame. When , the configuration 0 shall not be used for dedicated ranging channel allocation.

Table aaa - The periodicity of the dedicated DNS-RCH allocations

| Configurations | AAI subframe allocating the DNS-RCH |
| --- | --- |
| 0 | mod(OSF+2, NUL)th UL AAI subframe in every frame |
| 1 | mod(OSF+2, NUL)th UL AAI subframe in the third frame in every superframe |
| 2 | mod(OSF+2, NUL)th UL AAI subframe in the third frame in every even superframe, i.e., mod(superframe number, 2)=0 |
| 3 | mod(OSF+2, NUL)th UL AAI subframe in the third frame in every 4th superframe, i.e., mod(superframe number, 4)=0 |

----------------------------------------------  Text End -----------------------------------------------

----------------------------------------------  Text Start -----------------------------------------------

Dedicated synchronized ranging channel configuration

The information of the dedicated synchronized RCH (DS-RCH) allocation consists of the ranging configuration with AAI subframe-offset (OSF) for ranging resource allocation in the time domain where OSF is same AAI subframe-offset of the NS-RCH defined in 16.3.8.2.4.1. The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands RSB according to the Equation (286) where IDcell is defined in 16.3.5.1.2 and RSB is LSB-CRU,FPi/4, where LSB-CRU,FPi is the number of allocated subband CRUs as defined in 16.3.7.3 for FPi corresponding to reuse 1 partition or power-boosted reuse 3 partition only if there is no reuse 1 partition

Table bbb shows the information of the DS-RCH allocation in a regular allocation, which is indicated by the S-SFH. NUL denotes the number of UL AAI subframe per frame. When , the configuration 0 shall not be used for dedicated ranging channel allocation for M2M devices.

Table bbb - The periodicity of the dedicated DS-RCH allocations

| Configurations | AAI subframe allocating the DS-RCH |
| --- | --- |
| 0 | mod(OSF+3, NUL)th UL AAI subframe in every frame |
| 1 | mod(OSF+3, NUL)th UL AAI subframe in the fourth frame in every superframe |
| 2 | mod(OSF+3, NUL)th UL AAI subframe in the fourth frame in every 4th superframe, i.e., mod(superframe number, 4)=0 |
| 3 | mod(OSF+3, NUL)th UL AAI subframe in the fourth frame in every 8th superframe, i.e., mod(superframe number, 8)=0 |

----------------------------------------------  Text End -----------------------------------------------

# Reference

[1] IEEE P802.16m/D12, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb. 2011.

[2] IEEE, “C802.16p-11/0002; IEEE 802.16p Machine to Machine (M2M): Proposed Text from Power Saving (PWR) Rapporteur Group”, Mar. 2011

[3] IEEE C802.16p-11/0126, June 2011