| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID Assignment through DSA message |  |
| Date Submitted | 2011-7-10 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes MGID assignment through DSA message |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID Assignment through DSA message

Yi-Ting Lin, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee

Institute for Information Industry (III)

# Introduction

During the session 73 in Lake Louise, we had agreement that MGID can be added for an M2M device through AAI-DSA-REQ message. This is assumed that AAI-DSA-REQ message is initiated by the 16p BS. However, it is possible that connection establishment is initiated by an M2M device.

In this contribution, we proposed a MGID assignment scheme that is initiated by an M2M device. Similar to the multicast service of 802.16m, we assume each M2M multicast connection is associated with a service flow provisioned with the QoS and traffic parameters for the service flow. When an M2M device commences multicast service, the 16p BS shall include MGID in the AAI-DSA-RSP message to indicate multicast group for the connection that is associated with the service flow in AAI-DSA-RSP.

Fig. 1 Procedure of M2M device initiated MGID assignment

# Reference

[1] IEEE 802.16m-2011, “Part 16: Air Interface for Broadband Wireless Access Systems: Amendment 3: Advanced Air Interface,” May 2011.

[2] IEEE 802.16p-10/0018r2, “P802.16p Amendment Working Document (AWD),” May 2011.

- Text proposal for inclusion in the 802.16p AWD

Suggested Remedy 1

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.3.47.1 AAI-DSA-REQ

Table 734 AAI-DSA-REQ Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| MGID | 15 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an 802.16p BS initiates AAI-DSA-REQ. |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

Suggested Remedy 2

[Add the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.3.47.2 AAI-DSA-RSP

Table 735 AAI-DSA-RSP Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |
| If (Confirmation Code == 0 && AMS initiated AAI-DSA-REQ) { |  |  |  |
| MGID | 15 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an M2M device initiates AAI-DSA-REQ. |
| FID | 4 | An identifier of a service flow |  |
| } |  |  |  |
| … | … | … | … |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------