| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on M2M System Requirements Document (SRD) Functionality Comparison to Current AWD Content |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Jin Lee LG Electronics |  |
| Re: |  |  |
| Abstract | Comments on Functionality comparison between the SRD and the AWD |  |
| Purpose | Discuss and adopt texts changes in red by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M System Requirements Document (SRD) Functionality Comparison to Current AWD Content

1	Overview	3

2	References	3

3	Definitions	4

4	Abbreviations and Acronyms	4

5	General Requirements	5

5.1	Compatibility with other 802.16 equipment	5

5.2	Complexity	5

5.3	M2M Services	5

5.4	802.16p M2M Reference System Architecture	5

6	Functional Requirements	6

6.1	Low Power Consumption	6

6.2	Large Numbers of Devices	6

6.3	Small Burst Transmissions	7

6.4	Security Support	8

7	Performance Requirements	8

# Overview

The 802.16p amendment shall be developed in accordance with the Machine to Machine (M2M) communication project authorization request (PAR) form and five criteria in the approved version of IEEE 802.16ppc-10/0003r7 [1]. The scope of the resulting project shall be as follows:

“This amendment specifies IEEE Std 802.16 medium access control (MAC) enhancements and minimal orthogonal frequency division multiple access (OFDMA) Physical Layer (PHY) modifications in licensed bands to support lower power consumption at the device, support by the Base Station of significantly larger numbers of devices, efficient support for small burst transmissions, and improved device authentication.”

This document contains a comparison between the requirements in the SRD and the text in the AWD

# References

[1] IEEE P802.AWD - WirelessMAN-Advanced: IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems

[2]	IEEE P802.16p System Requirements Document (SRD)

# Definitions

| Terms | Description |
| --- | --- |
| Machine-to-Machine (M2M) communication | This is information exchange between user devices through a Base Station, or between a device and a server in the core network through a Base Station that may be carried out without any human interaction. |
| M2M ASN | Access Service Network that supports M2M service |
| M2M device | 802.16p MS with M2M functionality |
| M2M subscriber | Consumer of M2M service |
| M2M Server | An entity to communicate with M2M devices. M2M server provides an interface which can be accessed by M2M subscriber. |
| M2M feature | Unique characteristic of M2M application that is supported by M2M ASN. One or more features may be needed to support an application |
| M2M group | A group of devices that share one or more features in common, and/or belong to same M2M subscriber |
| Device collaboration | A technique where a certain device helps the other device’s transmission |

# Abbreviations and Acronyms

| M2M | Machine to Machine |
| --- | --- |
| MS | Mobile Station |
| BS | Base Station |
| ASN | Access Service Network |
| CSN | Connectivity Service Network |
| HAN | Home Area Network |
| PHY | Physical layer |
| MAC | Medium Access Control |
| OFDMA | Orthogonal Frequency Division Multiple Access |
| QoS | Quality of Service |
| RRM | Radio Resource Management |
| SRD | System Requirements Document |
| NIST | National Institute of Standards and Technology |

# General Requirements

## Compatibility with other 802.16 equipment

IEEE 802.16p BS provides continuing support for MSs compliant with WirelessMAN-Advanced Air Interface or WirelessMAN-OFDMA R1 Reference System.

## Complexity

IEEE 802.16p provides IEEE Std 802.16 medium access control (MAC) enhancements and minimal OFDMA PHY modifications to provide functionalities for efficient Machine to Machine communication.

## M2M Services

IEEE 802.16p shall support services required for M2M applications (e.g. smart grid, applications on consumer devices, vehicular tracking and healthcare). Detailed M2M services are specified in IEEE 802.16ppc-10/0002r7 Machine to Machine (M2M) Communication Study Report.

## 802.16p M2M Reference System Architecture

Figure 1 captures the high level system architecture for IEEE 802.16p based M2M communications. The IEEE 802.16p M2M device is an IEEE 802.16 MS with M2M functionality. The M2M server is an entity that communicates to one or more IEEE 802.16p M2M devices. The M2M server also provides an interface which can be accessed by an M2M subscriber. The M2M subscriber is a user of M2M services (e.g. a utility company). The M2M Server may reside within or outside of the Connectivity Service Network (CSN) and can provide M2M specific services for one or more IEEE 802.16p M2M devices. The M2M application runs on the IEEE 802.16p M2M device and the M2M server.

The 802.16p M2M service system architecture supports following types of M2M communication:

a) Point-to-point communication between IEEE 802.16p M2M device and IEEE 802.16p M2M server

b) Point-to-multipoint communication between the IEEE 802.16p BS and IEEE 802.16p M2M devices

In the M2M service system architecture shown in Figure 1, an IEEE 802.16p M2M device can act as an aggregation point for non IEEE 802.16 M2M devices. The non IEEE 802.16 M2M devices use different radio interfaces such as IEEE 802.11, IEEE 802.15, Bluetooth, Power Line Communications, etc. This aggregation function is shown only to illustrate an applicable use case for an 802.16p M2M device, however air interface changes to 802.16p device to support aggregation function is out of scope of 802.16p project.

Figure 1 802.16p M2M service reference system architecture

# Functional Requirements

This section provides requirements related to M2M features that are included in the scope of 802.16p PAR[1].  It shall be possible to subscribe to different M2M features independently according to the application or network environment.

## Low Power Consumption

### The 802.16p system shall support mechanisms for low power consumption in M2M devices.

### The 802.16p system shall provide optimized operation for M2M devices with no/low mobility to conserve power.
AWD - WirelessMAN-OFDMA: not covered
AWD - WirelessMAN-Advanced: MFM in RNG-REQ for fast re-entry from idle mode for fixed M2M MSs.

### The 802.16p system shall support optimized operation for time controlled, time tolerant and in-frequent traffic to conserve power.
AWD - WirelessMAN-OFDMA: M2M_POLL-ADV (6.3.2.3.60) for polling M2M MSs.
AWD - WirelessMAN-Advanced: long paging cycle and M2M paging offset in AAI-DREG-RSP (16.2.3.22)
Group paging indication in PG-Info (16.2.3.24, 16.2.18.7.1)
Group paging and report solicitation by MGID in AAI-PAG-ADV (16.2.3.23, 16.2.18.7.1)
Transmission Type and Max number of paging cycle in AAI-DREG-RSP for setting up group polling (16.2.3.22)

Paging Message Indication in PGID-Info (16.23.24, 16.2.18.7.1)

[Jin Lee] Paging Message Indication in PGID-Info (16.23.24, 16.2.18.7.1) also supports this requirement. Add the “Paging Message Indication in PGID-Info (16.23.24, 16.2.18.7.1)” to this requirement.

#### The 802.16p system shall support efficient radio resource management for time-controlled and time-tolerant traffic.
Radio resource management has always been out of the scope of the Air Interface.
Radio resource allocation is not covered.

### The 802.16p system shall support longer inactive periods for increased power savings for M2M devices associated with certain M2M applications.
AWD - WirelessMAN-OFDMA: Device-specific Idle Mode Timer
AWD - WirelessMAN-Advanced: long paging cycle (together with paging offset) in AAI-DREG-RSP; device-specific Idle Mode Timer in AAI-DREG-RSP; Group paging indication in PG-info

[Jin Lee] Group Paging Indication in PGID-Info is not related to this requirement. But the indication is related to 6.1.2 and exists under 6.1.2 section. Remove the “Group paging indication in PGID-Info” in this requirement.

### The 802.16p system may support device collaboration without defining a new 16p peer-to-peer connection (i.e. all MS transmissions must be terminated at the BS).
AWD - WirelessMAN-OFDMA: optional (may), not covered

## Large Numbers of Devices

The 802.16p system shall support very large numbers of devices.

AWD - WirelessMAN-OFDMA: not covered 
AWD - WirelessMAN-Advanced: a DL multicast service for idle AMS’s is defined. The FID and MGID are assigned with DSX transactions. It is not clear how the AMS knows this FID and MGID should be retained when the AMS goes into idle mode.

[Jin Lee] Like E-MBS the M2M device will retain the MGID and its related SF parameters during Idle mode without any indication. Such DL multicast support in idle mode is defined in sections 16.2.3.23, 16.2.28.4.3. Suggested remedy - remove the related texts as above.

In WirelessMAN-OFDMA, this is already defined as MCMBS.
[Jin Lee] MCMBS is not M2M specific. Also MCMBS is not suitable when applied to M2M in terms of efficiency. M2M traffic has infrequent traffic types, which is not applicable for MCMBS. Hence, we may assume that this is not covered. Suggest to change it to ‘not covered’.

### The 802.16p system shall be able to identify a large number of devices individually or by group.
MGID defined. Assigned and changed in DSX transaction.
This creates group with a shared DL SF. UL services are not covered.

### An 802.16p M2M device shall be able to join one or more M2M groups.
Since MGID is associated with a SF, the MS may join (or may be added to) more than one group. However, since SF’s are released when a MS/AMS goes idle, it’s not clear how the MGID shall be retained for idle mode operation, in both cases (e&m).

[Jin Lee] Like E-MBS the M2M device will retain the MGID and its related SF parameters during Idle mode without any indication. Such DL multicast support in idle mode is defined in sections 16.2.3.23, 16.2.28.4.3. Suggested remedy - remove the related texts as above.

### The 802.16p system shall support efficient group management for large numbers of M2M devices.
Efficient group management is not covered: there is no way the BS/MBS can manage the whole group; DSX transactions for setting or changing MGID is per MS/AMS.

### The 802.16p system shall support optimized signaling for resource allocation for large numbers of M2M devices individually or by group.
AWD - WirelessMAN-OFDMA: not covered.
AWD - WirelessMAN-Advanced: BR in AAI-RNG-REQ and BW Grant Indicator in AAI-RNG-RSP touches on optimized signalong for resource allocation. This does not cover connected AMS’s. In short: mot covered.

### The 802.16p system shall support efficient and reliable delivery of control and data to and from a large number of devices.
AWD - WirelessMAN-OFDMA: not covered
AWD - WirelessMAN-Advanced: Not covered. Section 16.2.28.4.4 states the requirement.

### The 802.16p system shall support efficient mechanisms for network access from large numbers of M2M devices.
AWD - WirelessMAN-OFDMA: not covered
AWD - WirelessMAN-Advanced: same as 6.2.4

#### The 802.16p system shall provide mechanisms to reduce access network congestion or overload from large numbers of M2M Devices.
Dedicated ranging
Idle MS/AMS polling?

AWD-WirelessMAN-OFDMA: dedicated ranging resource (6.3.23.10.1) M2M_POLL-ADV msg is to control and distribute network access overload (6.3.2.3.60)

AWD-WirelessMAN-Advanced: dedicated ranging resource, waiting time offset and different backoff window size are used to minimize access network congestion in case of network initiated entry (16.2.18.7.2)

M2M report code in AAI-PAG-ADV is also used to reduce access congestion (16.2.3.23)

[Jin Lee] suggested remedy – Add texts as above.

### The 802.16p system shall have the ability to distinguish between “normal power down” (e.g. associated with a voluntary power outage event) and “abnormal power down” (associated with an involuntary power outage event).
Not covered

### The 802.16p system shall provide mechanisms for allowing access with different levels of priority for M2M devices across different classes of M2M services.
Not covered. Should be clarified if the available tools in the current standard are not sufficient?

### The 802.16p system shall support large numbers of M2M devices in connected state as well as in idle state. Signaling shall be optimized for transitions between the two states.
Not covered.

### The 802.16p system shall support various QoS levels for M2M services.
See 6.2.8

## Small Burst Transmissions

The 802.16p system shall support very small burst transmissions. 
Not covered

### The 802.16p system shall support efficient transmission of small burst sizes. The 802.16p system shall minimize protocol and signaling overhead to support small burst transmission (e.g. support short burst transmission in OFDMA networks without requiring full connection establishment).

## Security Support

The 802.16p system shall support integrity and authentication of M2M devices, as well as integrity and privacy of M2M application traffic which requires a secure connection.

Not covered.
For AWD - WirelessMAN-Advanced: security for multicast services is defined.

Already exists in AWD-WirelessMAN-OFDMA. : Does not cover any of the clauses below.

[Jin Lee] For WirelessMAN-OFDMA, the multi-cast security mechanism is mainly for MBS and is not specific to M2M environments. 
Also: should be clarified what the shortcomings of the current standard are.

### The 802.16p system shall support a device validity check between the device and the network.

### The 802.16p system shall enable a flexible security suite that can be adjusted per the security requirements of the M2M application.

### The 802.16p system shall support efficient security for small burst transmissions.

### The 802.16p system shall support efficient security for M2M applications with infrequent/low traffic.

### The 802.16p system shall support using the M2M device location information for different applications (e.g. security).

# Performance Requirements

[Editor’s Note: To be added as result of call for contributions]