| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging waiting timer for Voluntarily M2M Access |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Youngsoo Yuk, Jin Lee, Jeongki Kim, Giwon Park, Inuk Jung and Jin Sam Kwak LG Electronics | Email: youngsoo.yuk@lge.com, jin1.lee@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposed a method to reduce the ranging trials from M2M devices. |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging waiting timer for Voluntarily M2M Access

Youngsoo Yuk, Jin Lee, Jeongki Kim, Giwon Park, Inuk Jung and JinSam Kwak

LG Electronics Inc.

# Introduction

Allocating ranging channel is a key issue of M2M access control. In order to control the higher UL access load, Allocating additional ranging opportunities are agreed in the last session. However the size of the ranging channel is very large comparing to other control channels, we should increase the utilization of each ranging opportunity.

In the previous sessions, the ranging optimizations discussed were focused on the special ranging trial cases (e.g. group paging, power outage etc). However, most of the transmission will be done voluntarily and the number of ranging access of MS-initiated should be optimized.

In this contribution, we propose to introduce “ranging waiting parameter” to reduce the number of ranging trials.

# Ranging Waiting Parameter

In order to minimize the ranging contention rate, we proposed a method for reducing number of trials.

One way to reduce the number of ranging access is to aggregate several UL transmissions.

In case of time-tolerable M2M services, in order to suppress the frequent ranging access, we can introduce one QoS parameter, “ranging waiting timer”. This parameter is determined by considering latency margin of the service.

If a service has long latency margin, after data being received from higher layer, MS shall wait perform ranging until ranging waiting timer is expired.

Figure 1 shows the concept of the proposed method. The multiple UL access within single Ranging waiting timer shall be transmitted with one ranging access, the total number of UL ranging from single device can be reduced.

However ranging waiting timer is not expired but the L2 buffer (ARQ or Non ARQ buffer) is possible to be overflow or any urgent traffic is generated, ranging may be allowed.

Figure 1. Concept of ranging waiting timer

# Proposed Text Change

Adopt the following Remedies in IEEE 802.16p-10/0018r2.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1 : In line 23 of page 21 in C802.16p-10/0018r2, insert the description as following]

16.2.12.12 Service Flow/Convergence Sublayer Parameters

Table 782 defines the parameters associated with UL/DL scheduling for a service flow.

Table 782——Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| Ranging Waiting Timer | 24 | frames, AMS shall wait performing ranging access to transmit the data of this flow until this timer is expired. |

[Remedy 2: : In line 23 of page 22 in C802.16p-10/0018r2, insert the description as following]

16.2.18.7 Idle mode for M2M application

16.2.18.7.2 Network re-entry from idle mode for M2M devices

A ranging waiting timer may be assigned for a flow during service negotiation. The ranging waiting timer for the flow shall be started when an MAC SDU of the flow is arrived from the higher layer during idle mode. The M2M device shall wait performing ranging for transmitting the MAC SDU of the flow until the ranging waiting timer is expired. Single ranging waiting timer is defined for a flow, and it shall not be reset even though new MAC SDU of the flow is arrived. It shall be terminated only if any ranging operation is initiated by the expiration of the ranging waiting timer of the other flow or by receiving paging notification from the BS.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------