| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE802.16e Security support for Group Management in M2M environment |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Inuk Jung, Kiseon Ryu and Jin Sam Kwak LG Electronics Hyunjeong Kang, Youngkyo Baek, Rakesh Taori Samsung Electronics | E-mail: inuk.jung@lge.com E-mail: hyunjeong.kang@samsung.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution provides the text for support of security for IEEE802.16e M2M Multi-cast traffic |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE802.16e Security support for Group Management in M2M environment

Inuk Jung, Kiseon Ryu and Jin Sam Kwak

LG Electronics Inc.

Hyunjeong Kang, Youngkyo Baek, Rakesh Taori

Samsung Electronics

# Introduction

In IEEE802.16e, there are several secure ways of providing multi-cast service to a group of devices as the following:

- MTK based multi-cast traffic encryption

- MBRA for MBS service

However, they both are not well applicable in M2M environments since the MTK based multi-cast security keys are provided in unicast manner and the MBRA based MBS security keys incur large overhead due to transmitting the GTEK keys.

Also the MGTEK may be seen as an unnecessary key encryption phased since it is not used for encryption but only used to derive the MTK. Hence, the MGTEK has no use but only serving for a derivation input parameter. Furthermore, such MBS security key update does not consider devices that operate in Idle mode, which implies that such devices need to update their MBS security keys in unicast manner after turning into Active mode.

Most MBS services consider multimedia services, which are less security sensitive compared to M2M environments where security sensitive control messages are transmitted in multi-cast manner, assuming that the M2M devices are managed in groups.

Thus, security suites for such group controlled M2M environment need to be suitably amended. A possible approach is to adopt the security key derivation method from 802.16m, which allows devices to derive TEK’s from a master key with an input of their count values. This approach is 1) overhead efficient, 2) maintains equivalent security level of conventional security methods, 3) low complexity, due to a simpler key hierarchy and 4) has minimal impact on the conventional security suite of 802.16e.

In this contribution, we propose the following sections and related text regarding:

- Key hierarchy for M2M

- Key derivation function of the new M2M Multi-cast GTEK (M2MGTEK)

- Update procedure of the M2MGTEK

  - Including case of subscription cancellation

- Related PKM attributes and parameters

# Proposed Text Change

Adopt the following Remedies in IEEE 802.16p-10/0018r2.

[Remedy 1: Add a new subclause on page 5, line 1 as follows:]

*This subclause is copied from the document P80216-2009, which the modification is based on

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

7.2.2.2.10 Key hierarchy

…

Figure 163 outlines the M2M multi-cast authentication key hierarchy starting from the MAK.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2: Add a new subclause on page 5, line 1 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

7. Security sublayer

…

7.2.2.2.13 M2M Group Traffic Encryption Key (M2MGTEK)

The M2MGTEK is used to encrypt data packets of the multicast service, which is shared among all SSs that belong to the multicast group.

The M2MGTEK is generated based on the MAK, M2M service Group Security Seed (MGSS), MGID and the M2MGTEK_COUNT. The generation and transport of the MAK is outside the scope of the IEEE 802.16 standard. It is provided through means defined at higher layers.

The M2MGTEK is derived as the following:

M2MGTEK <= Dot16KDF(MAK, MGSS | M2MGTEK_COUNT | MGID | “M2MGTEK”, 128)

- Here, the MAK (is generated by network side, which is outside the scope of IEEE802.16 standard

- Here, MGSS is a BS generated random seed value

- Here, the M2MGTEK_COUNT indicates the index of the currently used M2MGTEK, which the devices should apply to derive the M2MGTEK. The update of the M2MGTEK depends on the M2MGTEK_COUNT

- Here, MGID is the M2M Group ID

The M2MGTEK is updated either upon the expiration of PN space or the M2MGTEK lifetime, whichever comes first. In case the PN space expires or the M2MGTEK lifetime, the current M2MGTEK_COUNT is incremented by one by which the devices perform local derivation to derive the new M2MGTEK.

If a device cancels subscription from a group, the BS shall transmit a newly generated MGSS to each device in the group via the unsolicited PKM-RSP message and initialize the M2MGTEK_COUNT. The MGSS value shall be encrypted within the PKM-RSP message. The BS shall exclude the unsubscribed device from such security context update. For devices in connected mode, the BS shall send the new security context via the unsolicited PKM-RSP message. For devices in idle mode, the BS shall page the entire group (i.e., MGID) via the MOB_PAG-ADV message with ‘Action Code’ set to 0b10. For each device that successfully performed network re-entry, the ABS shall send an unsolicited PKM-RSP message including the new MGSS.

If a device is not able to decrypt an encrypted multi-cast data, the device shall initiate a key update request by transmitting the PKM-REQ message to the BS. Here, the MS shall include its MGID. After authenticating the MS, the BS shall respond with the current MGSS and the M2MGTEK_COUNT that are in use via the PKM-RSP message.

Every device that receive a PKM-RSP message with the new MGSS for updating the M2MGTEK shall respond with a PKM-ACK message for successful update acknowledgement.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 3: add subclause 6.3.2.3.9 on page 2, line 27 as follows:]

*This subclause is copied from the document P80216-2009, which the modification is based on

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.2.3 MAC management messages

…

6.3.2.3.9 Privacy key management (PKM) messages (PKM-REQ/PKM-RSP)

PKM employs two MAC message types: PKM-REQ (PKM request) and PKM-RSP (PKM response), as

described in Table 47.

Table 50—PKM message codes

| Code | PKM message type | MAC management message name |
| --- | --- | --- |
| … | … | … |
| 33 | MIH Comeback Response | PKM-RSP |
| 34 | M2M Key Request | PKM-REQ |
| 35 | M2M Key Reply | PKM-RSP |
| 3436-255 | Reserved |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 4: add subclause 6.3.2.3.51 on page 2, line 27 as follows:]

*This subclause is copied from the document P80216-2009, which the modification is based on

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message

The MOB_PAG-ADV message shall be sent on the Broadcast CID or Idle Mode Multicast CID during the BS paging interval. The format of the message is shown in Table 154.

Table 154—MOB_PAG-ADV message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … |  |  |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved Receiving Multi cast traffic |
| If (Action Code == 0b11) { |  |  |
| Multi-cast security key update | 1 | 0: no update on multi-cast security key 1: Multi-cast security key update scheduled |
| Multi-cast Group ID (MGID) | 15 | The multi-cast group ID, which the multi-cast traffic is scheduled for |
| If Multi-cast security key update=1) { |  |  |
| M2MGTEK_COUNT | TBD | The change count of the M2MGTEK, which is to be updated |
| Update_time | TBD | This indicates the time offset of when the updated M2MGTEK will be applied for multi-cast traffic |
| MGSS | TBD | Randomly generated value by BS |
| } |  |  |
| Multicast transmission start time (MTST) | TBD | Least significant TBD bits of the frame number in which the ABS starts sending DL multicast data. Shall be present when the MTST needs to be included in this message. |
| } |  |  |
| … |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 5: add subclause 6.3.2.3.9 on page 7, line 39 as follows:]

*This subclause is copied from the document P80216-2009, which the modification is based on

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

11.9 PKM-REQ/RSP management message encodings

A summary of the TLV encoding format is shown below. The fields are transmitted from left to right.

…

Table 592—PKM attribute types

| Type | PKM attribute |
| --- | --- |
| … | … |
| 47 | GKEK-Parameters |
| 48 | MIH Cycle |
| 49 | MIH Delivery Method and Status Code |
| 50 | M2M Multi-cast SA Descriptor |
| 51 | M2MGTEK -Parameters |
| 5052-255 | Reserved |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------