| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS abnormal or involuntary power down reports in advanced air interface |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Marta Martinez Tarradell, Shantidev Mohanty Intel | E-mail: marta.m.tarradell@intel.com |
| Re: | 802.16p amendment working document |  |
| Abstract | Proposes a method to report abnormal or involuntary power downs by MS to the network. |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS abnormal or involuntary power down reports in advanced air interface

Marta Martinez Tarradell, Shantidev Mohanty

Intel

# Introduction

One of the requirements of IEEE 802.16p is that 802.16p system needs to distinguish between normal and abnormal power down of the devices (see 6.2.8 in [1]).

The report of normal or voluntary power down is already supported in IEEE 802.16 standards ([3] and [4]). During the location update mechanism, MS can send the power down indicator in the RNG-REQ to indicate its normal or voluntary power down.

The abnormal power downs trigger involuntary power outage events that could happen. Current IEEE 802.16 standards ([3] and [4]) do not cover those unpredictable situations. As the M2M Technical Report [2] explains, those abnormal reports would help network operators/service providers to implement efficient outage management methods. This technical document [2] also explains the importance of having low latency mechanism due to the restriction in the backup battery for smart meters.

# Main Idea of this proposal

This contribution proposes that the MS monitors the triggers such as the backup capacitor charge to detect that an abnormal power down has occurred. Then the MS creates a message that includes the abnormal power down report and sends it to the BS to inform it about this event.

# Proposed texts

## Proposed Text #1

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section after line 21 in page 23 of IEEE 802.16p-10/0018r2.

16.2.x Abnormal power down reporting:

When a MS detects an abnormal power down event, it creates a message containing abnormal power down event indication and sends it to the BS to notify this event.

----------------------------------------------  Proposed Text End -----------------------------------------------

# References

- IEEE 80216ppc-10_0004r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)”.

- IEEE 80216p-10_0005, Machine to Machine (M2M) Communications Technical Report

- IEEE P802.16-2009

- P802.16m/D12.