| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Idle mode optimizations for fixed M2M devices |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Honggang Li Rui Huang Joey Chou Shantidev Mohanty Yang-seok Choi Intel Corporation | E-mail: Shantidev.Mohanty @intel.com |
| Re: | RE: Call for contributions for the IEEE 802.16p Amendment working document |  |
| Abstract | Optimizations for fixed M2M devices |  |
| Purpose | For review and adoption into the IEEE 802.16p Amendment working document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Idle mode optimizations for fixed M2M devices

Honggang Li, Rui Huang, Joey Chou, Shantidev Mohanty, Yang-seok Choi

# Background

There are certain applications such as SmartGrid where the M2M devices are always fixed. Thus, for these devices many of the operations during idle mode can be optimized to reduce the signaling overhead, to achieve extended power saving as well as to reduce the network deployment cost.

In this proposal, methods are proposed to optimize the idle mode operation of fixed M2M devices.

# Text Proposal

[Editor’s note: Add the following section after line 48 in page 3 of IEEE 802.16p-10/0018r2.]

---------------------------Begin Text Proposal-------------------------------

6.3.23.10.x Idle mode optimizations for fixed M2M devices

Localized idle mode operation for idle mode M2M devices: A fixed M2M device in idle mode need not perform location update. In addition, for fixed M2M devices that report information periodically do not need to implement paging operation. To eliminate the need for location update and paging, a fixed M2M device may include Localized_Idle_Mode flag set to 1 in the MOB-DREG-REQ message.

When a BS receives a MOB-DREG-REQ with Localized_Idle_Mode flag set to 1, it does not forward the DREG-REQ message to the PC. Also, the BS allocates a much longer resource retain timer for this M2M device so that its context is stored at the BS for faster network re-entry.

Then the BS sends MOB-DREG-RSP with Localized_Idle_Mode_Accepted flag set to 1 or 0. Localized_Idle_Mode_Accepted flag set to 1 indicates that BS accepted M2M device’s request. Then the device transitions to idle mode and does not perform location update or paging operation.

Localized_Idle_Mode_Accepted flag set to 0, the device enters the normal idle mode operation.

-----------------------------------------End Text Proposal-----------------------

# References

- IEEE 80216ppc-10_0004r1, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” November, 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S, Talwar, K. Johnsson, R. Miao, Supporting Power Outage Management for Smart Metering Applications, IEEE C80216p-10_0005, November, 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S, Talwar, K. Johnsson, “Power Outage Alarm for Smart Metering Applications ,” IEEE C802.16ppc-10_0054r1, July 2010.

- NIST priority action plan 2, “Wireless standards for Smart Grid,” sgs-2010-00003r000-NIST_Priority_Action_Plan2_r05.

- IEEE P802.16-2009IEEE

- P802.16m/D7.

- IETF RFC 2866 RADIUS Accounting