| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Idle mode optimizations for fixed M2M devices |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Honggang Li Rui Huang Joey Chou Shantidev Mohanty Yang-seok Choi Intel Corporation Giwon Park LG Electronics Hyunjeong Kang Samsung Electronics | E-mail: Shantidev.Mohanty @intel.com |
| Re: | RE: Call for contributions for the IEEE 802.16p Amendment working document |  |
| Abstract | Optimizations for fixed M2M devices |  |
| Purpose | For review and adoption into the IEEE 802.16p Amendment working document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Idle mode optimizations for fixed M2M devices

Honggang Li, Rui Huang, Joey Chou, Shantidev Mohanty, Yang-seok Choi

Intel Corporation

Giwon Park

LG Electronics

Hyunjeong Kang

Samsung Electronics

# Background

There are certain applications such as SmartGrid where the M2M devices are always fixed. Thus, for these devices many of the operations during idle mode can be optimized to reduce the signaling overhead, to achieve extended power saving as well as to reduce the network deployment cost.

In this proposal, methods are proposed to optimize the idle mode operation of fixed M2M devices.

# Text Proposal

[Editor’s note: Add the following section after line 48 in page 3 of IEEE 802.16p-10/0018r2.]

---------------------------Begin Text Proposal-------------------------------

[Remedy 1. Add the new subsection 6.3.23.10.x after subsection 6.3.23.10 (page 3, line 48) as follows:]

6.3.23.10.x Idle mode optimizations for fixed M2M devices

Localized idle mode operation for idle mode M2M devices: A fixed M2M device in idle mode need not perform the paging group based update. To eliminate the need for paging group based update and allocating the unnecessary paging information (i.e., Paging Group ID, Paging Controller ID), a fixed M2M device may include Localized_Idle_Mode Accepted flag set to 1 in the DREG-REQ message.

When a BS receives a DREG-REQ with Localized_Idle_Mode_Accepted flag set to 1, it does not inform the Paging Controller that the device enters Idle Mode.

Then the BS sends DREG-CMD with Localized_Idle_Mode_Accepted flag set to 1 or 0. Localized_Idle_Mode_Accepted flag set to 1 indicates that BS accepted M2M device’s request. Then the device transitions to idle mode and does not perform paging group based update.

Localized_Idle_Mode_Accepted flag set to 0, the device enters the normal idle mode.

[Remedy 2. Add the table DREG-REQ message (page 2, line 27) as follows:]

6.3.2.3.37 DREG-REQ (SS deregistration request) message

Add the following texts at the end of subclause 6.3.2.3.37

When the DREG-REQ message is sent to a base station, the following TLV may be included:

Localized_Idle_Mode_Accepted flag

Indicator of the Localized Idle Mode for fixed M2M device.

0: The device enters the normal idle mode.

1: The device enters the localized idle mode.

[Remedy 3. Add the text at the end of the description of the DREG-CMD message (page 2, line 39) as follows:]

6.3.2.3.26 DREG-CMD (de/register command) message

Add the following texts at the end of subclause 6.3.2.3.26

When the DREG-CMD message is sent to an M2M device, the following TLVs may be included:

M2M device-specific Idle mode timer

Length of the maximum interval between location update while the M2M device in idle mode

Localized_Idle_Mode_Accepted flag

Indicator of the Localized Idle Mode for fixed M2M device.

0: The device enters the normal idle mode.

1: The device enters the localized idle mode.

[Remedy 4. Add the description of the RNG-REQ message (page 2, line 27) as follows:]

6.3.2.3.5 RNG-REQ (ranging request) message

Add the following texts at the end of subclause 6.3.2.3.5

After the station entered the Localized Idle Mode (i.e., Localized_Idle_Mode_Accepted flag is set to 1 in DREG-REQ/CMD), when the station sends a RNG-REQ message for reentry or location update to a BS, the BS shall not include Paging Controller ID TLV (see 11.1.8.2) in this message

[Remedy 5. Add the description of the RNG-REQ message (page 2, line 27) as follows:]

6.3.2.3.6 RNG-RSP (ranging response) message

Add the following texts at the end of subclause 6.3.2.3.6

After the station entered the Localized Idle Mode (i.e., Localized_Idle_Mode_Accepted flag is set to 1 in DREG-REQ/CMD), when the BS sends a RNG-RSP message for reentry or location update to a station, following Paging Information TLV is included and Paging Controller ID TLV (see 11.1.8.2) is not included in this message.

Paging Information TLV

Bits #0-15: PAGING_CYCLE - cycle in which the paging message is transmitted within the paging group.

Bits #16-31:PAGING_OFFSET - determines the frame within the cycle from which the paging interval starts. Shall be smaller than PAGING_CYCLE value.

Bits #32-39: Paging Interval Length - Max duration in frames of Paging Listening interval. Used in calculation of Paging listening interval; value shall be between 1 and 5 frames (default=2).

[Remedy 6. Add the new subsection 16.2.1.3.2 after subsection 16.2.1.3.1 (page 9, line 40) as follows:]

16.2.1.3.2 Fixed M2M Deregistration ID (FMDID)

A 16-bit value that uniquely identifies a fixed M2M device in the base station that assigns FMDID. An FMDID is assigned to a fixed M2M device by a base station during idle mode entry and released during an explicit network exit (e.g., power down location update) or when the device performs the network reentry.

[Remedy7. Add the new subsection 16.2.18.7.3 after subsection 16.2.18.7.2 (page 22, line 65) as follows:]

16.2.18.7.3 Idle mode optimizations for fixed M2M devices

Localized idle mode operation for idle mode M2M devices: A fixed M2M device in idle mode need not perform the paging group based update. To eliminate the need for paging group based update and allocating the unnecessary paging information (i.e., Paging Group ID, Paging Controller ID), a fixed M2M device may include Localized_Idle_Mode Accepted flag set to 1 in the AAI-DREG-REQ message.

When a BS receives a AAI-DREG-REQ with Localized_Idle_Mode_Accepted flag set to 1, it does not inform the Paging Controller that the device enters Idle Mode.

Then the BS sends AAI-DREG-RSP with Localized_Idle_Mode_Accepted flag set to 1 or 0. Localized_Idle_Mode_Accepted flag set to 1 indicates that BS accepted M2M device’s request. Then the device transitions to idle mode and does not perform paging group based update.

Localized_Idle_Mode_Accepted flag set to 0, the device enters the normal idle mode.

16.2.18.7.3.1 Idle mode operations for fixed M2M devices

When the fixed M2M device enters the idle mode, a Fixed M2M Deregistration ID (FMDID) is assigned to the fixed M2M device and Paging Controller ID, Paging Group ID and Deregistration ID is not required to be assigned to the fixed M2M device.

The ABS can page the fixed M2M devices via group paging or individual paging. When the ABS pages the fixed M2M devices via group paging, it transmits the AAI-PAG-ADV message with MGIDs to the fixed M2M devices. When the ABS individually pages the fixed M2M devices, it transmits the AAI-PAG-ADV message with FMDID to the fixed M2M devices.

PGID information of the PGID-Info message is not applicable to the fixed M2M device because the Paging Group ID is not assigned to the fixed M2M device.

Fixed M2M device does not perform the paging group based update because the Paging Group ID is not assigned to the fixed M2M device. Fixed M2M device performs the timer based update based on the M2M device-specific Idle Mode Timer.

[Remedy 8. Add the table 698. AAI-DREG-REQ message (page 11, line 51) as follows:]

16.2.3.21 AAI-DREG-REQ message

Table 698. AAI-DREG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Deregistration_ Request_Code | 3 | Used to indicate the purpose of this message 0x00: AMS deregistration request from ABS and network 0x01: request for AMS deregistration from S-ABS and initiation of AMS idle mode. 0x02: response for the unsolicited AAIDREG-RSP message with action code0x05 by the ABS. 0x03: reject for the unsolicited AAIDREG-RSP message with action code0x05 by the ABS. This code is applicable only when an AMS has a pending UL data to transmit. 0x04: request for AMS deregistration from S-ABS to enter DCR mode 0x05: response for the unsolicited AAIDREG-RSP message with action code 0x00, 0x01, 0x02 or 0x03 0x06-0x07: reserved |  |
| … | … | … | … |

[Remedy 9. Modify the table 699. AAI-DREG-RSP message (page 12) as follows:]

16.2.3.22 AAI-DREG-RSP message

Table 699. AAI-DREG-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Action Code | 4 | Used to indicate the purpose of this message … 0x10-0x15:reserved |  |
| … | … | … | … |

[Remedy 10. Modify the table 700. AAI-PAG-ADV message (page 17) as follows:]

16.2.3.23 AAI-PAG-ADV

Table 700. AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action Code | 2 | 0b00: Performing network re entry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M is supported |
| if (Action Code == 0b10) { |  |  |  |
| Multicast Transmission Start Time (MTST) | 8 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| For (j=0; j<Num_FMDID; j++) { |  | Num_FMDID indicates the number of FMDIDs included in this paging message [1..32] | Shall be included when the ABS pages the fixed M2M devices. |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistration ID |  |
| Action Code | 1 | 0: Performing network re entry 1: Performing location update |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 1: Send uplink report | Present if M2M is supported |
| … | … | … | … |
| } |  |  |  |

[Remedy 11. Modify the table 678. AAI-RNG-REQ message (page 10) as follows:]

[Modification: DID and Paging Controller ID, Addition: FMDID]

16.2.3.1 AAI-RNG-REQ

Table 678. AAI-RNG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging purpose indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode … 0b1101 = NS/EP call setup 0b1110 – 0b1111 = reserved |  |
| … | … | … | … |
| } else if (Ranging Purpose Indication == 0b0010) { |  |  |  |
| Deregistration Identifier (DID) | 18 |  | If the Localized_Idle_Mode_Accepted flag is set to 1 in AAI-DREG-REQ/RSP message, DID shall not be included in this message. |
| Fixed M2M Deregistration ID (FMDID) | 16 | Used to indicate Fixed M2M Deregistration ID used to identify the fixed M2M device in idle mode 0.. | Only present if the Localized_Idle_Mode_Accepted flag is set to 1 in AAI-DREG-REQ/RSP message. |
| … | … | … | … |
| } |  |  |  |
| Paging Controller ID |  |  | If the Localized_Idle_Mode_Accepted flag is set to 1 in AAI-DREG-REQ/RSP message, Paging Controller ID shall not be included in this message. |
| … | … | … | … |

[Remedy 12. Modify the table 679. AAI-RNG-RSP message (page 11) as follows:]

16.2.3.2 AAI-RNG-RSP

Table 679. AAI-RNG-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Unsolicited bandwidth grant indicator | 1 |  |  |
| … | … | … | … |
| If (Location Update Response == 0x0 ) { |  |  |  |
| … | … | … | … |
| New Fixed M2M Deregistration ID | 16 | New FMDID which the fixed M2M device shall maintain in idle mode. | Only present if the Localized_Idle_Mode_Accepted flag is set to 1 in AAI-DREG-REQ/RSP message. |
| … | … | … | … |
| } |  |  |  |
| … | … | … | … |

-----------------------------------------End Text Proposal-----------------------

# References

- IEEE 80216ppc-10_0004r1, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” November, 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S, Talwar, K. Johnsson, R. Miao, Supporting Power Outage Management for Smart Metering Applications, IEEE C80216p-10_0005, November, 2010.

- R. Golla, W. Bell, M. Lin, N. Himayat, S, Talwar, K. Johnsson, “Power Outage Alarm for Smart Metering Applications ,” IEEE C802.16ppc-10_0054r1, July 2010.

- NIST priority action plan 2, “Wireless standards for Smart Grid,” sgs-2010-00003r000-NIST_Priority_Action_Plan2_r05.

- IEEE P802.16-2009IEEE

- P802.16m/D7.

- IETF RFC 2866 RADIUS Accounting