| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimizing Location update for stationary M2M devices |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Ritesh Kumar Kalle, Debabrata Das International Institute of Information Technology-Bangalore Shantidev Mohanty Intel Corp. | E-mail: riteshkumar.k@iiitb.ac.in Shantidev.mohanty@intel.com |
| Re: | IEEE 802.16p amendment working document |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimizing Location update for stationary M2M devices

Ritesh Kumar Kalle, Debabrata Das

International Institute of Information Technology-Bangalore

Shantidev Mohanty

Intel Corp.

# Background

Currently the RNG-REQ message for 16m devices includes the following mobility information for devices

AMS Mobility Information

0b00 = Slow (0-10km/h)

0b01 = Medium (10-120km/h)

0b10 = Fast (above 120km/h)

0b11 = Reserved

The M2M devices used for smart metering would come under the Slow mobility category above currently.

There are four location update trigger evaluation conditions:

- paging group based update

- timer based update,

- power down update and

- MBS update.

Plus AMS may conduct a Location Update at Will whenever required.

Since the assigned paging group doesn’t change, Trigger Condition 1 is not applicable

There are two possible schemes for Timer based update

Since the device is stationary there is no effect of PG_LU_TIMER on this device for location update as it doesn’t switch the PAGING_GROUP.

The Idle mode timer has a minimum value of 128 sec and a default value of 4096 sec.

The MS has to perform a location update before the expiry of the Idle mode timer and so we can assume the frequency of location updates for stationary M2M devices to be of the order of 60 min.

SMART GRID TRAFFIC MODEL (Example statistics of aggregate Smart Grid traffic IEEE 802.16p-10/0005)

For above traffic patterns in the smart meter example we see that the M2M device is not expected to wakeup from idle states for several 1000s of seconds during which the M2M device is forced to perform a location update before the Idle mode timer expiry as in the current draft.

So we propose the following changes:

Add1 bit of information more to AMS Mobility Information in AAI-RNG-REQ message Field Description

0b00 = Slow (0-10km/h)

0b01 = Medium (10-120km/h)

0b10 = Fast (above 120km/h)

0b11 = Fixed M2M Device

If (AMS Mobility ==0b11)

AMS may disable Location Update trigger based on Idle Mode Timer Expiry

Proposed Text

[Editor’s note: Add the following section after line 48 in page 3 of IEEE 802.16p-10/0018r2.]

---------------------------Begin Text Proposal-------------------------------

6.3.23.10.x Idle mode optimizations for fixed M2M devices

16.2.18.4.3.3 Location update for Fixed M2M device

The Fixed M2M device may disable the Location Update trigger based on Idle Mode Timer Expiry.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------