| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging Channel Design Considerations for M2M |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Shantidev Mohanty Belal Hamzeh Yang-Seok Choi Yi Hsuan Joey Chou Nageen Himayat Kerstin Johnsson Marta Martinez Tarradell Intel corporation | E-mail: { shantidev.mohanty}@intel.com |
| Re: | IEEE 802.16p-10/0018r2 |  |
| Abstract | This contribution proposes methods required to address the large number of ranging contentions arising from M2M devices |  |
| Purpose | To be discussed and adopted into 16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dedicated Ranging Channel Design Considerations for M2M in 16e

Shantidev Mohanty, Belal Hamzeh, Yang-Seok Choi,

Yi Hsuan, Joey Chou, Nageen Himayat, Kerstin Johnsson, Marta Martinez Tarradell

Intel Corporation

# Introduction

Due to large number of M2M devices, the load on ranging channel during network re-entry could be very high. To eliminate ranging channel congestion, this contribution proposes to allocate ranging resources that is proportional to the ranging load.

The proposed methods proposed are evaluated using the simulation methodology proposed by the network re-entry adhoc group (http://dot16.org/ul//upload/m2m_db/C80216p-11_0126.doc)

- Segmented ranging channel for M2M group

  - For the group_i, the ranging resource available is mod(groupi, N) , N is the number of ranging channel per configuration duration.

Simulation results are:

| MTC device number and | 5000 | 30000 |
| --- | --- | --- |
| Distribution | Uniform in 60s |  |
| Collision pro w/o segmented | 0.33% | 2.06% |
| Collision pro with segmented(N = 2) | 0.33% | 2.058% |

# Proposed Text

==================Start of proposed text=====================================

[Editor’s note] Add the following section after line 60 in page 3 of IEEE 802.16p-10/0018r2.

The BS allocates ranging resources proportional to the M2M ranging load. When dedicated ranging resources are used for M2M devices, the size of the dedicated ranging resource as well as the frequency of allocation depends on the ranging channel load.

==================End of proposed text====================================