| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | STID Validity Period and Offset |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Rakesh Taori Samsung Electronics | E-mail: anilag@samsung.com, hyunjeong.kang@samsung.com |
| Re: | Call for Comments on the 802.16p Amendment Working Document , IEEE 802.16p-11/0022 |  |
| Abstract | This contribution proposes modification to multicast operation in idle mode |  |
| Purpose | To be discussed and adopted in IEEE 802.16p-11/0022r1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

STID Validity Period and Offset

Anil Agiwal, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

The section 6.2.1.2.1 in AWD [1] defines STID sharing mechanism. The section mentions that, for the M2M devices sharing the same STID, their STID_Valid_Periodicity values shall be identical and their STID_Valid_Offset values shall be unique. This is not correct.

For sharing the STID, the essential requirement is that the frame in which STID is valid needs to be distinct amongst the M2M devices sharing the same STID. The two M2M devices sharing the same STID and having different STID valid frames may have different STID validity periods.

For example, consider the following, wherein M2M device 1 to M2M device 6 is assigned the same STID but their periods are not identical.

M2M device 1: STID_Valid_Periodicity = 8, STID_Valid_Offset = 1

M2M device 2: STID_Valid_Periodicity = 4, STID_Valid_Offset = 2

M2M device 3: STID_Valid_Periodicity = 4, STID_Valid_Offset = 3

M2M device 4: STID_Valid_Periodicity = 8, STID_Valid_Offset = 4

M2M device 5: STID_Valid_Periodicity = 8, STID_Valid_Offset = 5

M2M device 6: STID_Valid_Periodicity = 8, STID_Valid_Offset = 8

# Proposed text with change marks

6.2.1.2.1 Station identifier (STID)

The STID is also used to identify the M2M devices in the domain of the ABS. The ABS may assign the same STID to multiple M2M devices.

If the assigned STID to an M2M device is shared with other M2M device(s), the ABS shall assign the frame (s) in which the STID is valid for an M2M device. The assigned STID to an M2M device is valid only in the frame (i.e. Framenum) that satisfies the following condition.

Framenum mod STID_Valid_Periodicity = STID_Valid_Offset,

where Framenum denotes the frame sequence number. The parameters STID_Valid_Periodicity and STID_Valid_Offset are transmitted by ABS in AAI-REG-RSP message. For the M2M devices sharing the same STID, the ABS shall assign the STID_Valid_Periodicity and STID_Valid_Offset to each M2M device such that the Framenum for each M2M device is different. their STID_Valid_Periodicity values shall be identical, and their STID_Valid_Offset values shall be unique.

# References

[1] 80216p-11_0022: Enhancements to Support Machine-to-Machine Applications