| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Selection of Network Reentry Type for 16REV3 |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | Call for Comment on IEEE 802.16p 16REV3 AWD (802.16p-11/0021) |  |
| Abstract | This contribution clarifies how an M2M device selects network reentry type. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Selection of Network Reentry Type for 16REV3

Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In the last meeting, we agreed that BS can select a network reentry scheme for M2M device and indicate the selected network reentry scheme in MOB-PAG-ADV message. But, the relevant texts need to be clarified further.

The current AWD says that BS shall indicate a network reentry scheme for M2M device in MOB_PAG-ADV message based on mobility and traffic characteristics of the M2M device. The mobility and traffic characteristics of the M2M device may be one of criteria for selection of network reentry scheme but we can’t mandate the BS to use them for selection of network reentry scheme without clear conditions. In addition, it should be an implementation issue when and how the BS selects a network reentry scheme.

Another problem is that description on certain M2M network re-entry type is ambiguous. The current AWD says that the M2M device shall use a normal ranging channel if the network re-entry type included in MOB_PAG-ADV is set to 0b011. What does it mean by ‘a normal ranging channel’? Is it a ranging channel for legacy mobile stations? Or, is it a ranging channel dedicated for all M2M devices? It should be clarified.

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify the texts on page 10, line 42 as follows;]

Based on the mobility and traffic characteristics of the M2M device, the BS shall indicate the M2M device the network re-entry scheme in MOB_PAG-ADV message.The BS shall indicate a network re-entry scheme for M2M device in MOB_PAG-ADV message. In this case, mobility and traffic characteristics of the M2M device may be used as one of criteria for selection of network reentry scheme.

If the network re-entry type is set to ‘0b000, the M2M device doesn't need to send CDMA code for ranging but decodes UL-MAP IE directly for slot allocation for RNG-REQ message. If the M2M deviceSS receives an UL-MAP containing a Fast Ranging IE at the UL-MAP IE offset that indicated in MOB_PAG-ADV, it shall proceed to send a unicast RNG-REQ on the allocated bandwidth. Timer T55 starts in the frame where the M2M device expects to receive the Fast Ranging IE. Upon expiration of this timer, the M2M device shall not expect the BS to grant an UL allocation via the Fast Ranging IE and shall perform a normal ranging process.

If the network re-entry type is set to ‘0b001’, the BS shall allocate the dedicated ranging channel for M2M device in MOB_PAG-ADV message, the M2M device can finds the dedicated CDMA code assignment and transmission opportunity offset in ‘CDMA code and transmission opportunity assignment’ of MOB_PAG-ADV message for ranging and network re-entry.

If the network re-entry type is set to ‘0b010’, the BS shall allocate the dedicated ranging channel for M2M device in UL-MAP Extended IE message.

If the network re-entry type is set to ‘0b011’, the BS doesn’t allocate a dedicated ranging channel for M2M device and M2M device shall perform a contention-based ranging process on ranging channels allocated for all M2M devices.M2M device can only use the normal ranging channel.

[Remedy 2: Modify Table 172 on page 7 as follows;]

Table 172 – MOB_PAG-ADV message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| … |  |  |
| For (j=0; j<Num_MACs; j++) { | - | - |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Receiving multicast traffic |
| M2M network re-entry type | 3 | Indicate the network re-entry type for M2M device; 0b000: dedicated channel allocation for RNG-REQ 0b001: dedicated ranging channel allocation in MOB_PAG-ADV 0b010: dedicated ranging channel allocation in UL-MAP Extended IE 0b011: no dedicated ranging channel(i.e., contention-based ranging) normal ranging channel 0b100~0b111: reserved |
| If (Action Code == 0b11) { |  |  |
| Multicast Group ID (MGID) | 15 | The multicast group ID, which the multicast traffic is scheduled for |
| Multicast transmission start time (MTST) | TBD | Least significant TBD bits of the frame number in which the ABS starts sending DL multicast data. Shall be present when the MTST needs to be included in this message. |
| } |  |  |
| M2M report code | 1 | Action instruction to M2M device 1: Indication for the M2M device to send the uplink report |
| Reserved | 62 | - |
| } |  |  |
| … |  |  |

----------------- End of the text proposal ---------------------------------------------------------------------------------------