| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distribution of ranging access from M2M devices during network reentry in 16REV3 |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729, jscha}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p 16REV3 AWD (802.16p-11/0021) |  |
| Abstract | This contribution proposes texts for distribution of ranging access from M2M devices. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distribution of ranging access from M2M devices during network reentry in 16REV3

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In the last meeting, it is agreed that a new initial ranging backoff window may be assigned by a MOB_PAG-ADV message but relevant parameter is not defined yet. In this contribution, we propose to add a new ranging backoff start into a MOB_PAG-ADV message. Simulation results for the proposed ranging method can be found in contribution C802.16p-11/0142.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Add the following new paragraph on page 10, line 36]

During paging, a BS may assign a new ranging backoff start to M2M devices by a MOB_PAG-ADV message. If M2M devices receives MOB_PAG-ADV message which includes ranging backoff start TLV, they shall use the ranging backoff start TLV included in the MOB_PAG-ADV message to determine initial backoff window for initial ranging during network reentry or location update. This ranging backoff start shall be only applied to the ranging process that is in response to the MOB_PAG-ADV message.

[Remedy 2: Add a new subcluase on page 24, line 38 as follows]

Insert new subclause 11.17.6

11.16.6 M2M Ranging parameter

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Ranging backoff start | 157 | 1 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range: 0~15 | MOB_PAG-ADV |

----------------- End of the text proposal ----------------------------------------------------------------------------------------