| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distribution of ranging access from M2M devices during network reentry in 16REV3 |  |
| Date Submitted | 2011-09-21 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI Jinsoo Choi LG Electronics Honggang Li Intel Corporation Lei Zhou China Samsung Telecom R&D Center Erik Colban Huawei Technologies | Voice: +82-42-860-0626 E-mail: {kimsk0729, jscha}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p 16REV3 AWD (802.16p-11/0021) |  |
| Abstract | This contribution proposes texts for distribution of ranging access from M2M devices. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distribution of ranging access from M2M devices during network reentry in 16REV3

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

Jinsoo Choi

LG Electronics

Honggang Li

Intel Corporation

Lei Zhou

China Samsung Telecom R&D Center

Erik Colban

Huawei Technologies

# Introduction

In the last meeting, it is agreed that a new initial ranging backoff window may be assigned by a MOB_PAG-ADV message but relevant parameter is not defined yet. In this contribution, we propose to add a new ranging backoff start into a MOB_PAG-ADV message. Simulation results for the proposed ranging method can be found in contribution C802.16p-11/0142.

# Proposed Text

----------------- Start of the text proposal --------------------------------------------------------------------------------------

[Remedy 1: Modify the texts on page 9, line 35 as follows]

For SC and OFDM PHY, the SS shall put together a RNG-REQ message to be sent in an initial ranging interval. The duration of the burst carrying the RNG-REQ message shall be as specified in the Ranging Request Burst Size TLV (see 11.3.1). The CID field shall be set to the noninitialized SS value (zero). For the OFDM PHY, the initial ranging process may include a subchannelized mechanism specified in 8.3.7.2. For the OFDMA PHY, the initial ranging process shall begin by sending initial ranging CDMA codes on the UL allocation dedicated for that purpose (for more details see 6.3.10.3), instead of RNG-REQ messages sent on contention slots. An M2M device may perform ranging process using the initial ranging backoff window start assigned by a MOB_PAG-ADV. The assigned duration of this initial ranging backoff start window may be different from one assigned by the UCD message. This initial ranging backoff start window shall only be only applied to the ranging process that is in response to the MOB_PAG-ADV message.

[Remedy 2: Add the following new paragraph on page 10, line 36]

During paging, a BS may assign a new ranging backoff start to M2M devices by a MOB_PAG-ADV message. If M2M devices receives MOB_PAG-ADV message which includes ranging backoff start TLV, they shall use the ranging backoff start TLV included in the MOB_PAG-ADV message to determine initial backoff window for initial ranging during network reentry or location update. The assigned ranging backoff start may be different from one assigned by the UCD message and shall only be applied to the ranging process that is in response to the MOB_PAG-ADV message.

[Remedy 3: Add the following texts on page 24, line 38as follows]

The following TLV element may appear in each M2M group parameter TLV.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Ranging backoff start | 153.4 | 1 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range: 0~15 |

[Remedy 4: Add a new subcluase on page 24, line 38 as follows]

Insert new subclause 11.17.6

11.17.6 M2M paging parameter

Following M2M paging parameter TLV may be included in MOB_PAG-ADV message.

| Name | Type | Length | value | Scope |
| --- | --- | --- | --- | --- |
| M2M paging parameter | 157 | Variable | Compound TLV to be used in M2M individual paging operation | MOB_PAG-ADV |

Following TLV element may be included in MOB_PAG-ADV message.

| Name | Type | Length | value |
| --- | --- | --- | --- |
| Ranging backoff start | 157.1 | 1 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range: 0~15 |

----------------- End of the text proposal ----------------------------------------------------------------------------------------