| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distribution of ranging access from M2M devices during network reentry in 16.1 |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729, jscha}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p 16.1 AWD (802.16p-11/0022) |  |
| Abstract | This contribution proposes texts for distribution of ranging access from M2M devices. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distribution of ranging access from M2M devices during network reentry in 16.1

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In the last meeting, it is agreed that a new initial ranging backoff window may be assigned for network reentry from idle mode but detailed procedure is not defined yet. This contribution proposes to assign a new ranging backoff start by using AAI-PAG-ADV message. Simulation results for the proposed ranging method can be found in contribution C802.16p-11/0142.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify texts on page 29, line 53 as follows]

An M2M device may perform ranging process using the initial ranging backoff window assigned by TBD message. The duration of this initial backoff window may be different from one assigned by SFH SP3.

During paging, a BS may assign a ranging backoff start to M2M devices by AAI-PAG-ADV message. The ranging start included in the AAI-PAG-ADV message may be different from one assigned by SFH SP3. If M2M devices receives AAI-PAG-ADV message which includes ranging backoff start, they shall use the ranging backoff start included in the AAI-PAG-ADV message to determine initial backoff window for initial ranging during network reentry or location update. This ranging backoff start shall be only applied to the ranging process that is in response to the MOB_PAG-ADV message.

[Remedy 2: Modify Table 706 as follows]

Table 706—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For (i=0; i<M; i++) { |  |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| For (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for AMS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..218 - 1 | Present if the S-SFH Network Configuration bit == 0b0 |
| MAC Address Hash | 24 | Used to identify the AMS to be paged | Present if the S-SFH Network Configuration bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08: 32768 superframes 0x09: 262144 superframes 0x10: 4194304 superframes 0x1108-0x15: Reserved | Present if the S-SFH Network Configuration bit == 0b0 Values 0x08-0x10 shall be applied to M2M devices only. |
| Action Code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| M2M Report code | 1 | Indication for the M2M device to send the uplink report 0b0: reserved 0b1: Send uplink report | Present if M2M is supported |
| } |  |  |  |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action Code | 2 | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| If (Action Code == 0b00) { |  |  |  |
| M2M network re-entry type | 3 | Indicate the network re-entry scheme for M2M device; 0b000: dedicated channel allocation for AAI-RNG-REQ 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated ranging channel allocation, NS-SCH 0b011~0b111: reserved |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type = 0, to indicate Resource index for AAI-RNG-REQ; If M2M network re-entry type = 1, 2 to indicate dedicated ranging channel allocation; |  |
| Dedicated channel allocation timer | TBD |  |  |
| } |  |  |  |
| If (Action Code == 0b00 or 0b01) { |  |  |  |
| Ranging backoff start | 4 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range 0~15 | Present if needed |
| } |  |  |  |
| If (Action Code == 0b10) { |  |  |  |
| Multicast transmission start time (MTST) | 8 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| } |  |  |  |
| For (j=0; j<Num_FMDID; j++) { |  | Num_FMDID indicates the number of FMDIDs included in this paging message [1..32] | Shall be included when the ABS pages the fixed M2M devices. |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistration ID |  |
| Action Code | 1 | 0: Performing network reentry 1: Performing location update |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send uplink report 1: Send uplink report | Present if M2M is supported |
| } |  |  |  |
| } |  |  |  |
| Ranging backoff start | 4 | Initial backoff window size for ranging contention during network reentry or location update, expressed as a power of 2. Range 0~15 | Present if needed |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: This is the last fragment of the AAI-PAG-ADV message 0b1: This is not the last fragment of the AAI-PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: Reserved 0b1: There is emergency information | Optional Present if there is emergency information |

----------------- End of the text proposal ----------------------------------------------------------------------------------------