| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamic Allocation of Ranging Channel for M2M Devices |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729, jscha}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p 16.1 AWD (802.16p-11/0022) |  |
| Abstract | This contribution proposes texts for dynamic allocation of ranging channel for M2M devices. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamic Allocation of Ranging Channel for M2M Devices

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

According to the current AWD, ABS may assign M2M specific ranging resources dedicated for M2M devices. In general, there are two kinds of allocation methods. One is a static allocation and the other is a dynamic allocation.

In 16e system, static allocation of ranging channels dedicated for M2M devices is announced through UCD message and BS may allocate additional ranging channels for M2M devices dynamically by using M2M Ranging Allocation UL-MAP Extended IE.

In 16m system, static allocation of ranging channels dedicated for M2M devices is announced through SCD message but there is no method for dynamic allocation of ranging channels for M2M devices.

In this contribution, we propose to use Broadcast assignment A-MAP for dynamic allocation of ranging channels for M2M devices, which is very similar to the method used in 16e system.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Modify Table 866 on page 35, line 28 as follows:]

Table 866—Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reservedThis IE carries multicast assignment information for M2M application or ranging channel allocation information for M2M devices |
| … | … | … |
| Else {//Function Index == 0b11 |  |  |
| SubFunction Index | 1 | 0b0: Multicast assignment information for M2M application 0b1: Ranging channel allocation information for M2M devices |
| If (SubFunction Index == 0b0) { |  |  |
| MGID | 15 |  |
| Burst Size | 6 |  |
| Resource Index | 11 |  |
| Long_TTI_Indicator | 1 |  |
| Reserved | 1 |  |
| } |  |  |
| else {//SubFunction Index==0b1 |  |  |
| Number of Ranging Opportunities (N) |  | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| Dedicated ranging indicator | 1 | 0: this ranging channel is used for purpose of normal ranging 1: this ranging channel is used for the purpose of dedicated ranging indicated in in the AAI-PAG-ADV message |
| } |  |  |
| Reserved | 27/22 |  |
| } |  |  |
| } |  |  |
| } |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------