| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Small Burst Transmission |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5587 E-mail: jscha@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p 16REV3 AWD (802.16p-11/0021) |  |
| Abstract | This contribution proposes texts for small burst transmission. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Small Burst Transmission

Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

Both IEEE 802.16p [1] and 3GPP [2] define small burst transmission as one of main requirements for M2M communications. Main goal of small burst transmission is to allow M2M device to transmit small amount of data efficiently regardless of its state. In order to meet this requirement, M2M device shall be able to transmit/receive small burst with small overhead in connected state. In addition, M2M device in idle state shall be able to transmit/receive small burst without requiring full connection establishment. However, we spent much time to provide an efficient method only for SMS transmission. SMS does not mean a general small burst transmission and it’s just one of services which require small burst transmission capability. Therefore, we need to define more general procedure to support small burst transmission.

In this contribution, we propose to generalize the current SMS procedure for small burst transmission. The generalization of the current procedure means that M2M devices can use the same procedure for transmission of small amount of data regardless of contents of the small burst. So, there is no change in transmission procedure itself. Only the change is that M2M SBT type TLV is used to indicate the type of small burst (i.e., SMS or others).

# References

- IEEE 802.16p-10/0042r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)”

- 3GPP TS 22.368 v11.2.0, “Service Requirements for Machine-Type Communications (MTC) Stage 1 (Release 11)”

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify texts on page 12, line 8 as follows]

6.3.36 M2M small burst transmission (SBT)

M2M device in idle mode may transmit small burst without performing network reentry to request full connection establishment. The exact amount that is considered to be small burst is configurable by a network operator per M2M device but the definition of configuration procedure is out of scope of this standard.

To support UL SMSSBT from an M2M device in idle mode, two-round RNG-REQ/RSP are proposed as follows:

If the M2M device identifies the bandwidth allocation is enough for piggybacking SMS contentssmall amount of data in RNG-REQ from the UL-MAP, it will omit the 1st round of RNG-REQ/RSP and send the RNG-REQ piggybacking SMSsmall amount of data directly in second round of RNG-REQ.

The M2M device sends the RNG-REQ with M2M SMSSBT Request TLV to indicate it has a SMSsmall amount of data to send. If the BS receives the RNG-REQ with M2M SMSSBT Request TLV successfully, it may accept or reject the request. In this case the BS shall transmit a RNG-RSP with SMSSBT Response TLV with an action code instructing the M2M device how to proceed. If the BS accepts the SMSSBT Request, the BS shall transmit a RNG-RSP with SMSSBT Response TLV, with a Basic CID and a Temp CID Timer to be used for resource allocation for SMSSBT transmission. For fixed M2M device, the M2M device can send RNG-REQ for SMSSBT with the purpose indication for location update, but the paging related parameters can be removed from the RNG-REQ message to reduce the overhead because the BS is aware of its mobility information. This concludes the first round of RNG-REQ/RSP.

If the M2M device receives a RNG-RSP rejecting its SMSSBT Request, it shall proceed according to the action code. If the M2M device receives a RNG-RSP accepting its SMSSBT Request, it shall wait for bandwidth allocation for RNG-REQ with M2M SMS on its Basic CID and send a RNG-REQ with an M2M SBT Type TLV and an M2M SMSSBT payload TLV. If SMS packet the RNG-REQ with an M2M SBT Type TLV and an M2M SMSSBT payload TLV is received successfully, the BS sends RNG-RSP with SMSSBT confirmation to indicate the successful reception of the RNG-REQ message. This concludes the second round of RNG-REQ/RSP.

The Basic CID is released once the M2M device receives the SMSSBT Confirmation, or when the Temp CID Timer expires.

DL SMSM2M SBT type TLV and M2M SBT payload TLV may be included in RNG-RSP message when the action code of MOB_PAG-ADV indicates location update.

For DL SBTSMS transmission, the BS should send a Basic CID and a Temp CID Timer. When the M2M device receives the RNG-RSP with the DL SMSM2M SBT type TLV, M2M SBT payload TLV, a Basic CID and the Temp CID Timer, it may wait for bandwidth allocation for the RNG-REQ on the Basic CID. When SMS packetthe RNG-RSP with small amount of data is received successfully, a RNG-REQ message is sent as a confirmation of the DL M2M SMSSBT from the M2M device.

[Remedy 2: Modify Table 685 on page 20 as follows]

Table 685- RNG-REQ message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| Ranging Purpose Indication | 6 | 1 | Bit 0: HO indication (when this bit is set to 1 in combination with other included information elements indicates the MS is currently attempting to HO or network reentry from idle mode to the BS) Bit 1: Location update request (when this bit is set to 1, it indicates the MS action of idle mode location update process) Bit 2: Seamless HO indication (when this bit is set to 1 in combination with other information elements indicates the MS is currently initiating ranging as part of the seamless HO procedure) Bit 3: Ranging Request for Emergency Call Setup (when this bits is set to 1, it indicates MS action of Emergency Call Process) Bit 4: MBS update. When this bit is set to 1, the MS is currently attempting to perform location update due to a need to update service flow management encodings for MBS flows. Bit 5: Abnormal Power Down Indication. When this bit is set to 1, MS indicates that an abnormal or involuntary power down occurs. Bit56-7: Reserved | - |
| M2M SMSSBT Request | 25 | 1 | Bits 0-7: No. of bytes of SMSSBT payload | OFDMA |
| M2M SBT Type | xx | 1 | 0: SMS 1: normal | OFDMA |
| M2M SMSSBT payload | 26 | Variable | SBT payload If M2M SBT type=0, maximum length of M2M SBT payload (i.e., SMS message) is 140 bytes M2M SMS message content up to 140 bytes Padding bits to align boundary of byte | OFDMA |
| M2M SMSSBT Confirmation | 27 | 1 | Bit 0: SMS confirmation 0 – NACK 1 – ACK Bits 1-7: Reserved | OFDMA |
| Ranging Retries | 28 | 1 | The number of ranging retries in this ranging process Bits 0-1: Indicates the number of retrials in the channel ranging access as follows; 00 – Success in the first attempt 01 – Success in the second attempt 10 – Success in the third attempt 11 – Success in the 4th or later attempt | OFDMA |
| MIMO feedback information | 41 | 1 | Bit 0: Matrix indicator. This field suggests the preferred STC/MIMO matrix for the MS: 0b0: Matrix A 0b1: Matrix B Bit 1-4: DL effective CINR as defined in Table 520 Bits 5-7: Reserved | All |

[Remedy 3: Modify Table 688 on page 21 as follows]

Table 688- RNG-RSP message encodings

| NAME | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M2M SMSSBT Response | 45 | 1 | Bits 0-1: accept or reject SMS request 0b0: reject 0b1: accept If (reject) { Bits 2-3: action code 0b00: network re-entry 0b01-0b11: reserved } else { Reserved } Bits 4-7: Reserved | OFDMA |
| Temp CID Timer | 46 | 1 | Life time duration for the Basic CID assigned by BS |  |
| M2M SMSSBT Confirmation | 47 | 1 | Bit 0: SMS confirmation 0b0 – NACK 0b1 – ACK Bits 1-7: Reserved |  |
| M2M SBT Type | xx | 1 | 0: SMS 1: normal | OFDMA |
| M2M SMSSBT payload | 48 | Variable | SBT payload If M2M SBT type=0, maximum length of M2M SBT payload (i.e., SMS message) is 140 bytes M2M SMS message content up to 140 bytes Padding bits to align boundary of byte | OFDMA |

----------------- End of the text proposal ----------------------------------------------------------------------------------------