| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of multicast operation of M2M devices in idle mode in 802.16e |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes the method of indicating the end of M2M multicast data transmission for lower power consumption of M2M devices in idle mode |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of multicast operation for M2M devices of idle mode in 802.16e

Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

According to the IEEE P802.16p REV3 AWD [1], when a BS finishes sending the multicast data, the BS sends the MOB_MTE-IND message for lower power consumption of M2M devices in idle mode. In this case, if the M2M device does not receive this message, the device will wake up until next paging listening interval. This will increase the unnecessary power consumption of M2M devices in idle mode, especially in case of long paging cycle.

# Proposal

For power saving of M2M device we propose that M2M devices should enter the paging unavailable interval when the device does not receive the multicast data or the multicast end indication (i.e., MOB_MTE-IND) within specific time.

That is, M2M device starts a specific timer (i.e., M2M Multicast Traffic Reception timer) either at the MTST or when the device receives the paging message and this timer is reset whenever the multicast data is received and stops when the multicast end indication is received. If the device does not receive the multicast data or end indication until this timer expires, the device enters the paging unavailable interval for lower power consumption of M2M device.

# Reference

[1] IEEE 802.16p-10_0021, IEEE P802.16p REV3 AWD - Enhancements to Support Machine-to-Machine Applications

# Text proposal

- Added texts: blue underlined texts

- Deleted texts: red strikeout texts

---- Start of proposed text ----

[Remedy #1: Add the following texts at the end of subclause 6.3.2.3.51 in IEEE P802.16p REV3 AWD:]

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message

Add the following texts at the end of subclause 6.3.2.3.51

When the MOB-PAG-ADV message is sent to an M2M device, the following TLV may be included:

M2M Multicast Traffic Reception timer

This timer is required for M2M devices in idle mode to receive the M2M multicast data, in unit of frames (0~255). An M2M device receiving the MOB_PAG-ADV indicating the multicast traffic reception starts this timer at the frame indicated by the Multicast transmission start time TLV if the Multicast transmission start time TLV is included in the MOB_PAG-ADV. Otherwise, the device shall start this timer when the device receives the MOB_PAG-ADV. This timer shall be reset whenever the multicast data are received and stop when the MOB_MTE-IND message is received. If this timer expires, the device shall enter the paging unavailable interval as specified in 6.3.22.4.

[Remedy #2: Modify the subclause 11.17.5 in IEEE P802.16p REV3 AWD as following:]

11.17.5 M2M group paging parameter

…

The following TLV element may appear in each M2M_Group_Paging_Parameters TLV.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Multicast transmission start time (MTST) | 153.3 | 1 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data |
| M2M Multicast Traffic Reception timer | 153.4 | 1 | This timer is required for M2M devices in idle mode to receive the M2M multicast data, in unit of frames (0~255). |

[Remedy #3: Modify the subclause 6.3.28 in IEEE P802.16p REV3 AWD as follows:]

6.3.34 Support of multicast operation for machine to machine application

Add new subclause 6.3.34.1

6.3.34.1 M2M multicast operation in idle mode

A BS may provide a multicast service for M2M devices in idle mode with or without requiring network reentry of the M2M devices. Before a BS sends DL multicast data, the BS may transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The Multicast transmission start time TLV may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of Multicast transmission start time TLV shall be less than the start time of the next paging listening interval of the devices receiving the MOB_PAG-ADV message. The M2M device may power down until the frame indicated by the Multicast transmission start time TLV in the MOB_PAG-ADV message.

When the multicast data transmission ends, the BS shall notify the end of multicast data transmission to M2M devices by sending the AAIMOB_-MTE-IND message. Upon receiving the AAIMOB_-MTE-IND message, the device may enter the paging unavailable interval as specified in 16.2.18.2. 6.3.22.4

If Multicast transmission start time TLV is included in the MOB_PAG-ADV message, M2M devices receiving the MOB_PAG-ADV message shall start the Multicast Traffic Reception timer at the frame indicated by the Multicast transmission start time TLV. Otherwise, the device shall start the Multicast Traffic Reception timer when the device receives the MOB_PAG-ADV. The M2M Multicast Traffic Reception timer shall be reset whenever the multicast data are received. The device shall stop the M2M Multicast Traffic Reception timer when the MOB_MTE-IND message is received. If the M2M Multicast Traffic Reception timer expires, the device shall enter the paging unavailable interval as specified in 6.3.22.4.

---- End of proposed text ----