| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Multicast Assignment in IEEE P802.16 Rev3 |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes the text for resource allocation of multicast data transmission in IEEE P802.16 REV3 |  |
| Purpose | For discussion in 802.16p TG and adoption in to the IEEE P802.16p REV3 AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M multicast assignment in IEEE P802.16 REV 3

Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

In IEEE 802.16 REV3 a CID (e.g., basic CID, transport CID, multicast CID, etc.) is used in DL/UL MAP IEs to assign the DL/UL allocation to MSs. However, in M2M multicast service the CID cannot be reused to allocate the M2M multicast burst because of the insufficiency of CID space and in M2M multicast service an MGID instead of CID generally indicates a M2M multicast service flow.

So, we propose the new DL extended IE (i.e., M2M Multicast Assignment IE) which is used to allocate the resource of M2M multicast burst based on the MGID and an MGID related to the multicast burst is included in the new extended IE. The CID associated with the MGID will be included in the MAC header of multicast burst.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Add the following subclause 8.4.5.3.2 into the IEEE P802.16p-10/0021:]

8.4.5.3.2 DL-MAP Extended IE format

8.4.5.3.2.3 DL-MAP Extended-3 IE encoding format

Modify Table 409 as Indicated

Table 409—Extended-3 DIUC code assignment for Extended-2 DIUC = 15

| Extended-3 DIUC (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Power Boosting IE |
| 0x1 | MR DL-MAP Monitor IE |
| 0x2 | DL_Burst_Transmit_IE |
| 0x3 | OFDMA Periodic_Channel_Measurement_IE |
| 0x4 | M2M multicast assignment IE |
| 0x45-0xF | Reserved |

[Remedy #2: Add the subclause 8.4.5.3.xx into the IEEE P802.16p REV3 AWD:]

Add new subclause 8.4.5.3.xx as indicated

8.4.5.3.xx M2M Multicast Assignment IE format

In the DL-MAP, a BS shall transmit the M2M Multicast Assignment MAP IE to indicate the DL allocation in which M2M Multicast data is sent.

Table xxx—M2M Multicast Assignment IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| M2M Multicast Assignment IE(){ | - | - |
| Extended-2 DIUC | 4 | M2M Multicast Assignment IE () = 0xF (Extended-3 DIUC) |
| Length | 8 | Length in bytes |
| Extended-3 DIUC | 4 | 0x04 |
| MGID | 15 |  |
| DIUC | 4 |  |
| OFDMA Symbol Offset | 8 | The offset of the OFDMA symbol measured in OFDMA symbols from beginning of the DL frame in which the DL-MAP is transmitted. Counting from the frame preamble and starting from 0 |
| Subchannel offset | 7 | The offset of the first OFDMA symbol of the MBS region measured in OFDMA symbols from beginning of this DL frame |
| No. Subchannels | 7 |  |
| NO. OFDMA symbols | 7 |  |
| Boosting | 3 | Refer to Table 321 |
| Repetition Coding Indication | 2 | 0b00—No repetition coding 0b01—Repetition coding of 2 used 0b10—Repetition coding of 4 used 0b11—Repetition coding of 6 used |
| if !(byte boundary) { |  |  |
| Padding Nibble | variable | Padding to reach byte boundary |
| } |  |  |
| } | - | - |

[Remedy #3: Add the following texts at the end of the 6.3.34 as following:]

6.3.34 Support of multicast operation for machine to machine application

To access the multicast service, the M2M device receiving the M2M Multicast Assignment IE compares the MGID included in the M2M multicast Assignment IE with the MGIDs assigned during DSA procedure. If there is same MGID in its MGID list, the M2M device compares the CID associated with the MGID with CID in multicast MPDU.

6.3.34.1 M2M multicast operation in idle mode

…

---- End of proposed text ----