| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of M2M Multicast service in IEEE P802.16 Rev3 |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes the text to clarify the operation of M2M multicast service in IEEE P802.16 Rev3 |  |
| Purpose | For discussion in 802.16p TG and adoption in to the IEEE 802.16p Rev3 AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up of M2M multicast service in IEEE P802.16 Rev3

Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

This contribution proposes the text related to multicast connection establishment procedure and multicast operation of M2M device in connected mode in 802.16e because of no details for them.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the subclause 6.3.34 in the IEEE P802.16p REV3 AWD as follows:]

6.3.34 Support of multicast operation for machine to machine application

Multicast Service for M2M applications provides an efficient method for concurrent transport of DL data common to M2M devices belonging to an M2M group using an MGID in a BS. Multicast service is associated with a BS and is offered in the downlink only. Each multicast connection is associated with a service flow provisioned with the QoS and traffic parameters for that service flow. Service flows to carry multicast data are instantiated on individual M2M devices participating in the service while in Normal Mode. During such instantiation, the M2M device learns the parameters that identify the service and associated service flows.

6.3.34.1 Multicast operation

A BS may establish a DL multicast service by creating a multicast connection with each M2M device to be associated with the service. Since a multicast connection is associated with a service flow, it is associated with the QoS and traffic parameters of that service flow. For multicast connections, ARQ is not applicable, but a common security key is used to provide encryption and integrity protection for multicast traffic as described in 7.2.2.2.10 and 7.2.2.2.14.

6.3.34.2 Multicast connection establishment

When an M2M device registers to receive multicast services, the S-BS or the M2M device may initiate the DSA procedure for multicast connections. The M2M device's discovery and registration of multicast services with the BS through upper layer signaling are outside the scope of this standard.

The DSC messages are used to change multicast service flows, but the multicast service flows are not deleted unless the M2M device exits from a network. The M2M device shall retain service flow information associated multicast service during idle mode. The BS shall send the DSA-REQ/RSP to the M2M device with the relevant multicast parameters including MGID.

6.3.34.13 M2M multicast operation in idle mode

…

---- End of proposed text ----