| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of MOB_PAG-ADV message |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes the text for cleaning up the MOB_PAG-ADV message |  |
| Purpose | For discussion in 802.16p TG and adoption in to the IEEE P802.16p REV3 AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of MOB_PAG-ADV message

Jeongki Kim, Youngsoo Yuk, Giwon Park, Inuk Jung, Jinsam Kwak

LG Electronics

# Introduction

MGID is used not for individual paging but for group paging and MGID has already been defined as TLV format (11.17.5). Therefore, the related fields (i.e., MGID, MTST) should be deleted in MOB_PAG-ADV message.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the table 701 in the IEEE P802.16p-10/0021:]

Table 172—MOB_PAG-ADV message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_PAG-ADV_Message_format() { | — | — |
| … | … | … |
| For (j = 0; j < Num_MACs; j++) { | — | — |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Receiving multicast traffic reserved |
| M2M network re-entry type | 3 | Indicate the network re-entry type for M2M device: 0b000: dedicated channel allocation for RNG-REQ 0b001: dedicated ranging channel allocation in MOB_PAG-ADV 0b010: dedicated ranging channel allocation in UL-MAP Extended IE 0b011: normal ranging channel 0b100-0b111: reserved |
| If (Action Code == 0b11) { |  |  |
| Multicast Group ID (MGID) | 15 | The multicast group ID, which the multicast traffic is scheduled for |
| Multicast transmission start time (MTST) | 4 | Least significant TBD bits of the frame number in which the ABS starts sending DL multicast data. Shall be present when the MTST needs to be included in this message. |
| } | — | — |
| M2M report code | 1 | Action instruction to M2M device 1: Indication for the M2M device to send the uplink report |
| Reserved | 62 | — |
| } |  |  |
| … |  |  |

[Remedy #2: Add the following texts at the end of subclause 6.3.2.3.51 in the IEEE P802.16p-10/0021:]

Add the following texts at the end of subclause 6.3.2.3.51

When MOB_PAG-ADV message is sent to M2M devices, the following TLVs may be included:

M2M group paging parameters TLV (see 11.7.5)

M2M group paging parameters is a compound TLV that provides a group paging method of paging all M2M devices belonging to an M2M group. Each M2M group paging parameters TLV contains M2M group ID TLV indentifying an M2M group paged and Action code TLV which indentifies the operation of M2M devices after receiving the MOB_PAG-ADV. According to the value of Action code TLV, other TLVs (e.g., Multicast transmission start time TLV in Action code 0b10) may be included in this M2M group paging parameters TLV.

---- End of proposed text ----