| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of AAI-DREG-REQ/RSP message |  |
| Date Submitted | Sep 09, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p AAI AWD texts |  |
| Abstract | This is to clarify the AAI-DREG-REQ/RSP message |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of AAI-DREG-REQ/RSP message

Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes that an FMDID is assigned to a fixed M2M device by a base station during the idle mode entry. However, there is no FMDID in idle mode initiation message.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Add the table 704. AAI-DREG-REQ message (page 10, line 06) as follows:]

6.2.3.21 AAI-DREG-REQ message

Table 704. AAI-DREG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Deregistration_ Request_Code | 3 |  |  |
| If (Deregistration_Request_Cod e == 0x01) { |  |  |  |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| … | … | … | … |

[Remedy 2. Modify the table 705. AAI-DREG-RSP message (page 10, line 47) as follows:]

6.2.3.22 AAI-DREG-RSP message

Table 705. AAI-DREG-RSP message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Action Code | 4 | Used to indicate the purpose of this message … 0x10-0x15:reserved |  |
| if (Action Code == 0x05) { |  |  |  |
| … | … | … | … |
| M2M paging offset |  |  |  |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Fixed M2M Device Identifier (FMDID) | 16 | Used to indicate Fixed M2M Deregistration ID used to identify the fixed M2M device in idle mode. | Only present if the Localized_Idle_Mode_Accepted flag is set to 1. |
| Paging controller ID | 48 |  | This parameter shall not be presented when the Localized_Idle_Mode_Accepted flag is set to 1. |
| Paging group ID | 16 |  | This parameter shall not be presented when the Localized_Idle_Mode_Accepted flag is set to 1. |
| Deregistration ID | 18 |  | Present when the S-SFH Network Configuration bit == 0b0 If the Localized_Idle_Mode_Accepted flag is set to 1, this is not included in this message. Instead, FMDID is included in this message. |
| … |  |  |  |
| if (Action Code == 0x07) { |  |  |  |
| … | … | … | … |
| M2M paging offset |  |  |  |
| Localized_Idle_Mode_Accepted flag | 1 | 0: The device enters the normal idle mode. 1: The device enters the localized idle mode. | This parameter shall be presented when the fixed M2M device enters the idle mode. |
| Fixed M2M Device Identifier (FMDID) | 16 | Used to indicate Fixed M2M Deregistration ID used to identify the fixed M2M device in idle mode. | Only present if the Localized_Idle_Mode_Accepted flag is set to 1. |
| Paging controller ID | 48 |  | This parameter shall not be presented when the Localized_Idle_Mode_Accepted flag is set to 1. |
| Paging group ID | 16 |  | This parameter shall not be presented when the Localized_Idle_Mode_Accepted flag is set to 1. |
| Deregistration ID | 18 |  | Present when the S-SFH Network Configuration bit == 0b0 If the Localized_Idle_Mode_Accepted flag is set to 1, this is not included in this message. Instead, FMDID is included in this message. |
| … |  |  |  |

# References

[1] IEEE P802.16p AAI AWD