| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of the M2M polling procedure in WirelessMAN OFDMA system |  |
| Date Submitted | Sep 09, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p REV3 AWD text |  |
| Abstract | This is to clarify the M2M polling procedure in WirelessMAN OFDMA system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of the M2M polling procedure in WirelessMAN OFDMA system

Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

The text related to M2M polling procedure of current draft [1] is an ambiguous and is not understandable. This contribution proposes a clarification of the M2M polling procedure.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Modify the text in 6.3.22.6 (page 9, line 56) as follows:]

6.3.22.6 BS Broadcast Paging message

MOB_PAG-ADV with M2M report code may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed M2M devices. When a M2M device receives the DREG-RSP message with the Transmission Type set to 1 and Max number of paging cycle TLV, the M2M device may shall wait for the MOB_PAG-ADV with M2M report code as long as Max number of paging cycle * paging cycle before sending uplink data.

When the M2M device receives the MOB_PAG-ADV with M2M report code within Max number of paging cycle * paging cycle, it may send the data to the BS. If the M2M device does not receive at least one MOB_PAG-ADV with M2M report code within Max number of paging cycle * paging cycle, it should not send the uplink it may voluntarily send the data to the BS.

# References

[1] IEEE P802.16p REV3 AWD