| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging resource allocation for idle mode fixed M2M device in WirelessMAN OFDMA system |  |
| Date Submitted | Sep 20, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p REV3 AWD texts |  |
| Abstract | This is to propose the ranging resource allocation for idle mode fixed M2M device in WirelessMAN OFDMA system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging resource allocation for idle mode fixed M2M device in WirelessMAN OFDMA system

Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes that if the M2M network re-entry type is set to 0b000 (i.e., dedicated channel allocation of RNG-REQ), fixed M2M device doesn't need to send CDMA code for ranging but decodes UL-MAP IE directly for slot allocation for RNG-REQ message. However, there is no UL-MAP IE offset parameter in MOB_PAG-ADV message. Thus, we clarify it. Moreover, network reentry procedure for fixed M2M device described in draft [1] is incomplete. We also clarify the network reentry procedure of fixed M2M device using network re-entry type indicator.

# Modification motive

- M2M network re-entry type

In addition to network reentry, this parameter can be used for other purpose (e.g., BS triggered location update).

Thus, M2M network access type is more appropriate than M2M network re-entry type.

- M2M network re-entry type (0b000 dedicated channel allocation for RNG-REQ)

When the M2M network re-entry type is set to 0b000, resource information (i.e., UL-MAP offset) for RNG-REQ message is included in MOB_PAG-ADV message. ‘Resource’ is appropriate rather than ‘Channel’. Thus, we modified ‘dedicated channel allocation for RNG-REQ’ to ‘Resource allocation for RNG-REQ’.

Network re-entry type can be included in group paging message (including MGID) or individual paging message (including MAC address hash). Thus, we clarified it.

- In case M2M device receive the MOB_PAG-ADV message with MGID and M2M network re-entry type is set to 0b000, the UL-MAP IE monitoring information (e.g., M2M dedicated resource start offset, Dedicated resource timer) shall be defined to help UL-MAP IE reception of M2M device. That is, M2M device start to monitor the UL-MAP IE for RNG-REQ at M2M dedicated resource start offset during the Dedicated resource timer. We clarified the network reentry procedure using UL-MAP IE monitoring information.

- In case M2M device receive the MOB_PAG-ADV message with individual identifier (i.e., MAC address hash) and M2M network re-entry type is set to 0b000, the M2M device decodes the UL-MAP IE including Fast Ranging IE at the UL-MAP IE offset indicated in MOB_PAG-ADV message and sends a unicast RNG-REQ on the allocated bandwidth.

- Clarification of MOB_PAG-ADV message

Network re-entry type and M2M report code is only useful to M2M device. Thus, TLV format is appropriate rather than mandatory parameter.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 6.3.22.11.1 (page 10, line 46) as follows:]

6.3.22.11.1 Network reentry from idle mode for M2M devices

If the M2M network re-entry access type is set to ‘0b000, the M2M device doesn't need to send CDMA code for ranging but decodes UL-MAP IE directly for slot allocation for RNG-REQ message. If the SS receives an UL-MAP containing a Fast Ranging IE at the UL-MAP IE offset that indicated in MOB_PAG-ADV, it shall proceed to send a unicast RNG-REQ on the allocated bandwidth. When the M2M device receives group paging message (i.e., the MOB_PAG-ADV with MGID) and M2M network access type (i.e., 0: Resource allocation for RNG-REQ), it starts to monitor the UL-MAP IE containing a Fast Ranging IE to obtain the resource of RNG-REQ message at ‘UL MAP start offset for RNG-REQ’ during the ‘Resource monitor timer’. ‘Resource monitor timer’ starts in the frame where the M2M device expects to receive the Fast Ranging IE. If the M2M device does not decode the Fast Ranging IE until the expiration of the ‘Resource monitor timer’, it performs contention-based ranging.

When the M2M device receives individual paging message (i.e., the MOB_PAG-ADV with MAC address hash) and M2M network access type (i.e., 0b00: Resource allocation for RNG-REQ), it decodes the UL-MAP IE containing a Fast Ranging IE to obtain the resource of RNG-REQ message at ‘UL-MAP offset for RNG-REQ’ and shall proceed to send a unicast RNG-REQ on the allocated bandwidth. Timer T55 starts in the frame where the M2M device expects to receive the Fast Ranging IE. Upon expiration of this timer, the M2M device shall not expect the BS to grant an UL allocation via the Fast Ranging IE and shall perform contention-based ranging.

If the M2M device receives individual paging message (i.e., the MOB_PAG-ADV with MAC address hash) and the M2M network re-entry access type is set to ‘0b001’, the BS shall allocate the dedicated ranging channel for M2M device in MOB_PAG-ADV message, the M2M device can find the dedicated CDMA code assignment and transmission opportunity offset in ‘CDMA code and transmission opportunity assignment’ of MOB_PAG-ADV message for ranging and network re-entry.

If the M2M device receives individual paging message (i.e., the MOB_PAG-ADV with MAC address hash) and the M2M network re-entry access type is set to ‘0b010’, the BS shall allocate the dedicated ranging channel for M2M device in UL-MAP Extended IE message.

If the M2M device receives individual paging message (i.e., the MOB_PAG-ADV with MAC address hash) and the M2M network re-entry access type is set to ‘0b011’, M2M device can only use the normal ranging channel.

[Remedy 2. Modify the table 172 (page 7, line 60) as follows:]

Table 172 – MOB_PAG-ADV message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| for (j = 0; j < Num_MACs; j++) { | - | - |
| MS MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. Thee polynomial for the calculation is 0x1864CFB |
| Action Code | 2 | Paging action instruction to MS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Receiving multicast traffic |
| M2M network re-entry type | 3 | Indicate the network re-entry type for M2M device: 0b000: dedicated channel allocation for RNG-REQ 0b001: dedicated ranging channel allocation in MOB_PAG-ADV 0b010: dedicated ranging channel allocation in UL-MAP Extended IE 0b011: normal ranging channel 0b100-0b111: reserved |
| if (Action Code == 0b11) { |  |  |
| Multicast Group ID (MGID) | 15 | The multicast group ID, which the multicast traffic is scheduled for |
| Multicast transmission start time (MTST) | TBD | Least significant TBD bits of the frame number in which the ABS starts sending DL multicast data. Shall be present when the MTST needs to be included in this message. |
| } |  |  |
| M2M report code | 1 | Action instruction to M2M device 1: Indication of the M2M device to send the uplink report |
| Reserved | 26 |  |
| } |  |  |
| … |  |  |

[Remedy 3. Add the new TLV in page 24, line 39 as follows:]

11.17.5 M2M group paging parameter

…

Following TLV element may be included in MOB_PAG-ADV message.

| Name | Type | Length | value |
| --- | --- | --- | --- |
| M2M network access type | 156.4 | 1 | Indicate the network access scheme for M2M device; 0b0: Resource allocation (i.e., UL MAP IE offset) for RNG-REQ 0b1: Contention-based ranging |

Following TLV element may be included in MOB_PAG-ADV message when the M2M network access type is set to 0b000.

| Name | Type | Length | value |
| --- | --- | --- | --- |
| UL MAP start offset for RNG-REQ | 156.5 | 1 | This parameter indicates the offset that M2M device start to monitor the resource (i.e., UL MAP IE offset) for the RNG-REQ message. |
| Resource monitor timer | 156.6 | 1 | Time duration that M2M device monitors the resource for RNG-REQ message. |

11.17.6 M2M individual paging parameter

Following M2M individual paging parameter TLV may be included in MOB_PAG-ADV message.

| Name | Type | Length | value | Scope |
| --- | --- | --- | --- | --- |
| M2M individual paging parameter | 157 | Variable | Compound TLV to be used in M2M individual paging operation | MOB_PAG-ADV |

Following TLV element may be included in MOB_PAG-ADV message when the Action code is set to 0b01 (i.e., Perform ranging to establish location and acknowledge message) or 0b10 (i.e., Enter network)..

| Name | Type | Length | value |
| --- | --- | --- | --- |
| M2M network access type | 157.1 | Variable | Indicate the network re-entry type for M2M device: 0b00: Resource allocation (i.e., UL MAP IE offset) for RNG-REQ 0b01: Additional dedicated ranging channel allocation in MOB_PAG-ADV 0b10: Additional dedicated ranging channel allocation in UL-MAP Extended IE 0b11: No additional dedicated ranging channel (i.e., Contention-based ranging) Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices For (i=0; i<Num_MAC_Address_Hashs; i++){ 3bit M2M network access type } |

Following TLV element may be included in MOB_PAG-ADV message when the M2M network access type is set to 0b000.

| Name | Type | Length | value |
| --- | --- | --- | --- |
| UL-MAP offset for RNG-REQ | 157.2 | Variable | Indicate offset that UL-MAP IE containing Fast Ranging IE for RNG-REQ message is transmitted. Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices For (i=0; i<Num_MAC_Address_Hashs; i++){ RNG-REQ offset } |

Following TLV element may be included in MOB_PAG-ADV message.

| Name | Type | Length | value |
| --- | --- | --- | --- |
| M2M report code | 157.3 | Variable | Bits 0-7: Num_MAC_Address_Hash, Number of M2M devices For (i=0; i<Num_MAC_Address_Hashs; i++){ 1bit M2M Report code } |

# References

[1] IEEE P802.16p REV3 AWD