| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system |  |
| Date Submitted | Sep 09, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Heejeong Cho, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p AAI AWD texts |  |
| Abstract | This is to propose the ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system

Giwon Park, Youngsoo Yuk, Heejeong Cho, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes network reentry procedure based on the M2M network re-entry type but reentry procedure or parameters indicated in AAI-PAG-ADV message were incompletely defined.

Thus, we clarify the network reentry procedure using the M2M network re-entry type indicated in AAI-PAG-ADV.

# Modification motive

- M2M network re-entry type

In addition to network reentry, this parameter can be used for other purpose (e.g., BS triggered location update).

Thus, M2M network access type is more appropriate.

- M2M network re-entry type (0b000 dedicated channel allocation for RNG-REQ)

When the M2M network re-entry type is set to 0b000, resource information (i.e., Assignment A-MAP offset) for AAI-RNG-REQ message is included in AAI-PAG-ADV message. ‘Resource’ is appropriate rather than the ‘Channel’. Thus, we modified ‘dedicated channel allocation for AAI-RNG-REQ’ to ‘Resource allocation for AAI-RNG-REQ’.

Network re-entry type can be included in group paging message (including MGID) or individual paging message (including DID or FMDID). Thus, we clarified it as follow;

- ‘Dedicated Channel Allocation’ in group paging  M2M dedicated resource start offset

- ‘Dedicated Channel Allocation Timer’ in group paging Dedicated resource timer

- ‘Dedicated Channel Allocation’ in individual paging  AAI-RNG-REQ offset

- ‘Dedicated Channel Allocation Timer’ in individual paging  not useful

In case M2M device receive the AAI-PAG-ADV message with MGID and M2M network re-entry type is set to 0b000, the Assignment A-MAP IE monitoring information (e.g., M2M dedicated resource start offset, Dedicated resource timer) shall be defined to help Assignment A-MAP IE reception of M2M device. That is, M2M device start to monitor the Assignment A-MAP IE for AAI-RNG-REQ at M2M dedicated resource start offset during the Dedicated resource timer. We clarified the network reentry procedure using Assignment A-MAP IE monitoring information.

In case M2M device receive the AAI-PAG-ADV message with individual identifier (i.e., DID or FMDID) and M2M network re-entry type is set to 0b000, the M2M device decodes the Assignment A-MAP IE at the Assignment A-MAP IE offset indicated in AAI-PAG-ADV message and sends a unicast AAI-RNG-REQ on the allocated bandwidth.

- M2M network re-entry type (0b001 Dedicated ranging channel allocation S-RCH, 0b010 Dedicated ranging channel allocation NS-RCH)

Current Draft [1] does not specifically define the ‘Dedicated Channel Allocation’ and ‘Dedicated Channel Allocation Timer’ in M2M network re-entry procedure. Thus we clarified it.

- ‘Dedicated Channel Allocation / Dedicated Channel Allocation Timer’ of group paging are significantly different from ‘Dedicated Channel Allocation / Dedicated Channel Allocation Timer’ of individual paging.     We clarified the Dedicated Channel Allocation and Dedicated Channel Allocation Timer in group paging or individual paging as follow:

- ‘Dedicated Channel Allocation’ in group paging  M2M ranging opportunity subframe index

- ‘Dedicated Channel Allocation Timer’ in group paging Periodicity of the M2M ranging

- ‘Dedicated Channel Allocation’ in individual paging  Ranging Start Frame Number, Ranging Opportunity Index, Ranging Opportunity Subframe Index

- ‘Dedicated Channel Allocation Timer’ in individual paging  not useful

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 6.2.18.7.2 (page 29, line 40) as follows:]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

If the M2M network re-entry access type is set to ‘0’, the M2M device doesn't need to send CDMA code for ranging but sends AAI-RNG-REQ message with using the channel allocation information indicated in ‘Dedicated channel allocation’ in AAI-PAG-ADV message.

When the M2M device receives the AAI-PAG-ADV with MGID and M2M network access type (i.e., 0: Resource allocation for AAI-RNG-REQ), it starts to monitor the Assignment A-MAP IE to obtain the resource of AAI-RNG-REQ message at M2M dedicated resource start offset during the Dedicated resource timer. If the M2M device does not decode the Assignment A-MAP IE until the expiration of the Dedicated resource timer, it performs normal ranging using the ranging resources defined in Table 839 in 6.3.5.5.1.2.

When the M2M device receives the AAI-PAG-ADV with FMDID and M2M network access type (i.e., 0: Resource allocation for AAI-RNG-REQ), it decodes the Assignment A-MAP IE to obtain the resource of AAI-RNG-REQ message at AAI-RNG-REQ offset.

If the M2M network re-entry access type is set to ‘1’, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the dedicated S-RCH allocation is used for ranging.

If the M2M network re-entry access type is set to ‘2’, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the dedicated NS-RCH allocation is used for ranging.

When the M2M device receives the AAI-PAG-ADV with DID or FMDID and M2M network access type (i.e., ‘1: dedicated ranging channel allocation, S-RCH’ or ‘2: dedicated ranging channel allocation, NS-RCH’), it performs the ranging for network re-entry using the dedicated ranging channel indicated in AAI-PAG-ADV message.

[Remedy 2. Modify the table 706 in page 17, line 07 as follows:]

Table 706 – AAI-PAG-ADV message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| for (i=0; i<M; i++) { |  |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| for (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the AMS or mobile M2M device to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) | Present if the S-SFH Network Configuration bit == 0b0 |
| … |  |  |  |
| Acton Code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| M2M network access type | 3 | Indicate the network access scheme for M2M device 0b000: Resource allocation for AAI-RNG-REQ 0b001: Dedicated ranging channel allocation, S-RCH 0b010: Dedicated ranging channel allocation, NS-RCH 0b011: Normal ranging as defined in Table 839 in 6.3.5.5.1.2 0b100-0b111:reserved |  |
| if (M2M network access type == 0b000) { |  |  |  |
| AAI-RNG-REQ offset |  | Indicate offset that Assignment A-MAP IE for AAI-RNG-REQ message is transmitted. |  |
| } |  |  |  |
| if (M2M network access type == 0b001 ¦ 0b010) { |  |  |  |
| Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. |  |
| Ranging Start Frame Number | 8 | The 8 least significant bits of the absolute frame number at the ABS where the AMS starts to perform ranging process. This is calculated based on paging message reception time. |  |
| Ranging Opportunity Index | 1 | Indicates the index of the allocated ranging opportunity. |  |
| Ranging Opportunity Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. 0b0: 0b01 0b1: 0b10 |  |
| } |  |  |  |
| M2M Report code | 1 |  |  |
| } // End of for (j=0; j<Num_AMSs; j++) |  |  |  |
| } // End of for (i=0; i<M; i++) { |  |  |  |
| for (j=0; j<Num_MGID; j++ { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| if(Action code ==0b00 ¦ 0b01) { |  |  |  |
| M2M network re-entry access type | 3 | Indicate the network re-entry access scheme for M2M device 0b000: dedicated channel allocation Resource allocation for AAI-RNG-REQ, 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated ranging channel allocation, NS-RCH 0b011: Normal ranging as defined in Table 839 in 6.3.5.5.1.2 0b0110b100-0b111:reserved |  |
| if (M2M network access type == 0b001 ¦ 0b010) { |  |  |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type = 0, to indicate Resource Index for AAI-RNG-REQ; If M2M network re-entry type = 1, 2 to indicate dedicated ranging channel allocation; |  |
| Dedicated channel allocation timer | TBD |  |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. |  |
| Periodicity of the M2M ranging | 3 | Indicates the periodicy of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: Reserved] |  |
| } |  |  |  |
| if (M2M network access type == 0b000) { |  |  |  |
| M2M dedicated resource start offset |  | This parameter indicates the offset that M2M device start to monitor the resource (i.e., A-MAP IE) for the AAI-RNG-REQ message. |  |
| Dedicated resource timer |  | Time duration that M2M device monitors the resource for AAI-RNG-REQ message. |  |
| } |  |  |  |
| } End of if (Action code ==0b00 ¦ 0b01) { |  |  |  |
| if(Action Code == 0b10) { |  |  |  |
| Multicast Transmission Start Time (MTST) | 8 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| } |  |  |  |
| for (j=0; j<Num_FMDID; j++) { |  | Num_FMDID indicates the number of FMDIDs included in this paging message [1..32] | Shall be included when the ABS pages the fixed M2M devices. |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistration ID |  |
| Action Code | 1 | 0: Performing network re entry 1: Performing location update |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 1: Send uplink report | Present if M2M is supported |
| M2M network access type | 3 | Indicate the network access scheme for M2M device 0b000: Resource allocation for AAI-RNG-REQ 0b001: Dedicated ranging channel allocation, S-RCH 0b010: Dedicated ranging channel allocation, NS-RCH 0b011: Normal ranging as defined in Table 839 in 6.3.5.5.1.2 0b100-0b111:reserved |  |
| if (M2M network access type == 0b000) { |  |  |  |
| AAI-RNG-REQ offset |  | Indicate offset that A-MAP IE for AAI-RNG-REQ message is transmitted. |  |
| } |  |  |  |
| if (M2M network access type == 0b001 ¦ 0b010) { |  |  |  |
| Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. |  |
| Ranging Start Frame Number | 8 | The 8 least significant bits of the absolute frame number at the ABS where the AMS starts to perform ranging process. This is calculated based on paging message reception time. |  |
| Ranging Opportunity Index | 1 | Indicates the index of the allocated ranging opportunity. 0b0: 0b01 0b1: 0b10 |  |
| Ranging Opportunity Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| } |  |  |  |
| } // End of for (k=0; k<Num_FMDID; k++) { |  |  |  |
| … |  |  |  |

# References

[1] IEEE P802.16p AAI AWD