| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system |  |
| Date Submitted | Sep 21, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Heejeong Cho, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p AAI AWD texts |  |
| Abstract | This is to propose the ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging resource allocation for idle mode fixed M2M device in WirelessMAN IMT-Advanced system

Giwon Park, Youngsoo Yuk, Heejeong Cho, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes network reentry procedure based on the M2M network re-entry type but reentry procedure or parameters indicated in AAI-PAG-ADV message were incompletely defined.

Thus, we clarify the network reentry procedure using the M2M network re-entry type indicated in AAI-PAG-ADV.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 6.2.18.7.2 (page 29, line 40) as follows:]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

If the M2M network re-entry access type is set to ‘0b00’, the M2M device doesn't need to send CDMA code for ranging but sends AAI-RNG-REQ message with using the channel resource allocation information indicated in ‘Dedicated channel allocation’ in AAI-PAG-ADV message.

When the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and M2M network access type is set to 0b00 (i.e., Resource allocation for AAI-RNG-REQ), it starts to monitor the Fixed M2M Ranging Assignment A-MAP IE to obtain the resource for AAI-RNG-REQ message at ‘Assignment A-MAP start offset for AAI-RNG-REQ’ during the ‘Resource monitor timer’. If the M2M device does not decode the Fixed M2M Ranging Assignment A-MAP IE until the expiration of the ‘Resource monitor timer’, it performs contention-based ranging.

If the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and the M2M network re-entry access type is set to ‘0b01’, the ABS shall additionally allocate the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) for M2M devices in AAI-PAG-ADV message, the dedicated S-RCH allocation is used for ranging.

If the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and the M2M network re-entry access type is set to ‘20b10’, the ABS shall additionally allocate the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) for M2M devices in AAI-PAG-ADV message, the dedicated NS-RCH allocation is used for ranging.

If the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and the M2M network re-entry access type is set to 0b11, M2M device performs the normal ranging using the ranging resources defined in Table 839 in 6.3.5.5.1.2.

When the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with FMDID) and M2M network access type is 0b00 (i.e., Resource allocation for AAI-RNG-REQ), it decodes the Fixed M2M Ranging Assignment A-MAP IE to obtain the resource of AAI-RNG-REQ message at ‘Assignment A-MAP offset for AAI-RNG-REQ’.

When the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with DID or FMDID) and M2M network access type is set to 0b01 (i.e., dedicated ranging channel allocation in AAI-PAG-ADV), the ABS shall allocate the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) for M2M devices in AAI-PAG-ADV message and information of the dedicated ranging channel is included in the AAI-PAG-ADV message.

When the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with DID or FMDID) and M2M network access type is set to 0b10 (i.e., dedicated ranging channel allocation in broadcast assignment A-MAP IE), the ABS shall allocate the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) for M2M devices in AAI-PAG-ADV message and information of the dedicated ranging channel is included in the Broadcast Assignment A-MAP IE.

If the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with DID or FMDID) and the M2M network re-entry access type is set to 0b11, M2M device performs the normal ranging using the ranging resources defined in Table 839 in 6.3.5.5.1.2.

[Remedy 2. Modify the table 706 in page 17, line 07 as follows:]

Table 706 – AAI-PAG-ADV message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| for (i=0; i<M; i++) { |  |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| for (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the AMS or M2M device to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) | Present if the S-SFH Network Configuration bit == 0b0 |
| … |  |  |  |
| Acton Code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| M2M network access type | 2 | Indicate the network re-entry type for M2M device; 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b11: No dedicated ranging channel |  |
| if (M2M network access type == 0b00) { |  |  |  |
| Assignment A-MAP offset for AAI-RNG-REQ |  | Indicate offset that Assignment A-MAP IE for AAI-RNG-REQ message is transmitted. |  |
| } |  |  |  |
| M2M Report code | 1 |  |  |
| } // End of for (j=0; j<Num_AMSs; j++) |  |  |  |
| } // End of for (i=0; i<M; i++) { |  |  |  |
| for (j=0; j<Num_MGID; j++ { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| if(Action code ==0b00 ¦ 0b01) { |  |  |  |
| M2M network re-entry access type | 32 | Indicate the network re-entry access scheme for M2M device 0b000: dedicated channel allocation Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ, This type is only applicable to fixed M2M device (i.e., Localized_Idle_Mode_Accept flag was set to 1 at the idle mode initiation.). Except fixed M2M device, Mobile M2M device shall perform the contention-based ranging. 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated ranging channel allocation, NS-RCH 0b11: No dedicated ranging channel 0b011-0b111:reserved |  |
| if (M2M network access type == 0b01 ¦ 0b10) { |  |  |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type = 0, to indicate Resource Index for AAI-RNG-REQ; If M2M network re-entry type = 1, 2 to indicate dedicated ranging channel allocation; |  |
| Dedicated channel allocation timer | TBD |  |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. |  |
| Periodicity of the M2M ranging | 3 | Indicates the periodicity of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: Reserved] |  |
| } |  |  |  |
| if (M2M network access type == 0b00) { |  |  |  |
| Assignment A-MAP start offset for AAI-RNG-REQ |  | This parameter indicates the offset that M2M device start to monitor the resource (i.e., Assignment A-MAP IE) for the AAI-RNG-REQ message. |  |
| Resource monitor timer |  | Time duration that M2M device monitors the resource (i.e., Assignment A-MAP IE) for AAI-RNG-REQ message. |  |
| } |  |  |  |
| } End of if (Action code ==0b00 ¦ 0b01) { |  |  |  |
| if(Action Code == 0b10) { |  |  |  |
| Multicast Transmission Start Time (MTST) | 8 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| } |  |  |  |
| for (j=0; j<Num_FMDID; j++) { |  | Num_FMDID indicates the number of FMDIDs included in this paging message [1..32] | Shall be included when the ABS pages the fixed M2M devices. |
| Fixed M2M Deregistration ID (FMDID) | 16 | Fixed M2M Deregistration ID |  |
| Action Code | 1 | 0: Performing network re entry 1: Performing location update |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 1: Send uplink report | Present if M2M is supported |
| M2M network access type | 2 | Indicate the network re-entry type for M2M device; 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b11: No dedicated ranging channel |  |
| if (M2M network access type == 0b00) { |  |  |  |
| Assignment A-MAP offset for AAI-RNG-REQ |  | Indicate offset that A-MAP IE for AAI-RNG-REQ message is transmitted. |  |
| } |  |  |  |
| } // End of for (k=0; k<Num_FMDID; k++) { |  |  |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. | Optional, This parameter shall be present if the M2M network access type of individually paged M2M devices is set to 0b01. |
| Periodicity of the M2M ranging | 3 | Indicates the periodicity of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: Reserved] | Optional, This parameter shall be present if the M2M network access type of individually paged M2M devices is set to 0b01. |
| } |  |  |  |
| … |  |  |  |

# References

[1] IEEE P802.16p AAI AWD