| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | New M2M specific Assignment A-MAP IE for network reentry of fixed M2M device in WirelessMAN IMT-Advanced system |  |
| Date Submitted | Sep 09, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p AAI AWD |  |
| Abstract | This is to propose the new M2M specific A-MAP IE for network reentry of fixed M2M device in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

New M2M specific A-MAP IE for network reentry of fixed M2M device in WirelessMAN IMT-Advanced system

Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes that if the M2M network re-entry type is set to 0b000 (i.e., dedicated channel allocation of AAI-RNG-REQ), fixed M2M device performs the ranging procedure using the dedicated channel allocation for AAI-RNG-REQ, A-MAP IE offset for AAI-RNG-REQ is indicated in AAI-PAG-ADV. However, there is no description of Assignment A-MAP IE for transmission of AAI-RNG-REQ message.

Thus, we define new M2M specific A-MAP IE and propose the scheme of ranging resource allocation for network reentry of fixed M2M device.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 6.2.18.7.2 (page 29, line 40) as follows:]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

If the M2M network re-entry type is set to ‘0’, the M2M device doesn't need to send CDMA code for ranging but sends AAI-RNG-REQ message with using the channel allocation information indicated in ‘Dedicated channel allocation’ in AAI-PAG-ADV message.

[Remedy 2. Modify the table 795b in page 29, line 24 as follows:]

Table 795b – Scheme selection of network re-entry for M2M

| M2M Network re-entry type | Network re-entry scheme | Note |
| --- | --- | --- |
| 0 | Dedicated channel allocation for AAI-RNG-REQ, Fixed M2M Ranging Assignment A-MAP IE offset for AAI-RNG-REQ is indicated in AAI-PAG-ADV | Fixed M2M, known traffic pattern, UL synchronization not required |

[Remedy 3. Add the table 849 in page 33, line34 and add the new type indicator in this table as follows:]

Table 849. Description of CRC mask

| Masking Prefix (1bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking code |
|  | 0b000 | 12 bit STID or TSTID |
|  | 0b001 | Refer to Table 850 |
|  | 0b010 | Refer to Table 851 |
|  | 0b011 | LSB 12 bits of FMDID |

[Remedy 4. Add the new subclause 6.3.5.5.2.4.14.1(page 36, 10) as follows:]

6.3.5.5.2.4.14 Extended Assignment A-MAP IE

6.3.5.5.2.4.14.1 Fixed M2M Ranging Assignment A-MAP IE

The Fixed M2M Ranging Assignment A-MAP IE is used to allocate ranging channel of idle mode fixed M2M device.

The M2M device that is assigned an FMDID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b011, and masking code = LSB 12 bits of FMDID to decode the assignment A-MAP IE. The MSB 4 bits of FMDID is included in Fixed M2M Ranging Assignment A-MAP IE.

Table 867a – Fixed M2M Ranging Assignment A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M Fixed Ranging Assignment A-MAP IE() { |  |  |
| A-MAP IE Type | 4 |  |
| Extended Assignment A-MAP IE type | 4 | Fixed M2M Ranging Assignment A-MAP IE |
| MSB 4bits of Fixed M2M device Identifier (FMDID) | 4 | Remaining bit of FMDID except masking code (i.e., LSB 12 bits of FMDID) |
| Uplink/Downlink Indicator | 1 | Indicates whether the following fields are for resource assignment in the uplink or in the downlink. 0b0: Uplink 0b1: Downlink |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Burst Size | 5 |  |
| Reserved | 11 |  |
| } |  |  |

# References

[1] IEEE P802.16p AAI AWD