| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | New M2M specific Assignment A-MAP IE for network reentry of fixed M2M device in WirelessMAN IMT-Advanced system |  |
| Date Submitted | Sep 21, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p AAI AWD |  |
| Abstract | This is to propose the new M2M specific A-MAP IE for network reentry of fixed M2M device in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

New M2M specific A-MAP IE for network reentry of fixed M2M device in WirelessMAN IMT-Advanced system

Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes that if the M2M network re-entry type is set to 0b000 (i.e., dedicated channel allocation of AAI-RNG-REQ), fixed M2M device performs the ranging procedure using the dedicated channel allocation for AAI-RNG-REQ, A-MAP IE offset for AAI-RNG-REQ is indicated in AAI-PAG-ADV. However, there is no description of Assignment A-MAP IE for transmission of AAI-RNG-REQ message.

Thus, we define new M2M specific A-MAP IE and propose the scheme of ranging resource allocation for network reentry of fixed M2M device.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Add the table 849 in page 33, line34 and add the new type indicator in this table as follows:]

Table 849. Description of CRC mask

| Masking Prefix (1bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking code |
|  | 0b000 | 12 bit STID or TSTID |
|  | 0b001 | Refer to Table 850 |
|  | 0b010 | Refer to Table 851 |
|  | 0b011 | LSB 12 bits of FMDID or LSB 12 bits of DID |

[Remedy 2. Add the new subclause 6.3.5.5.2.4.14.1(page 36, 10) as follows:]

6.3.5.5.2.4.14 Extended Assignment A-MAP IE

6.3.5.5.2.4.14.1 Fixed M2M Ranging Assignment A-MAP IE

The Fixed M2M Ranging Assignment A-MAP IE is used to allocate ranging channel of idle mode fixed M2M device.

The fixed M2M device with FMDID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b011, and masking code = LSB 12 bits of FMDID to decode the assignment A-MAP IE. The MSB 4 bits of FMDID is included in Fixed M2M Ranging Assignment A-MAP IE.

The fixed M2M device with DID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b011, and masking code = LSB 12 bits of DID to decode the assignment A-MAP IE. The MSB 6 bits of DID and paging cycle is included in Fixed M2M Ranging Assignment A-MAP IE.

Table 867a – Fixed M2M Ranging Assignment A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M Fixed Ranging Assignment A-MAP IE() { |  |  |
| A-MAP IE Type | 4 |  |
| Extended Assignment A-MAP IE type | 4 | Fixed M2M Ranging Assignment A-MAP IE |
| M2M Device Identifier type | 1 | 0: DID 1: FMDID |
| If (M2M Device Identifier type == 0) { |  |  |
| MSB 6bits of Deregistration Identifier (DID) | 6 | MSB 6bits of Deregistration Identifier (DID) |
| Paging Cycle | 4 | Paging cycle |
| } |  |  |
| If (M2M Device Identifier Type == 1) { |  |  |
| MSB 4bits of Fixed M2M device Identifier (FMDID) | 4 | MSB 4bits of Fixed M2M device Identifier (FMDID) |
| } |  |  |
| Uplink/Downlink Indicator | 1 | Indicates whether the following fields are for resource assignment in the uplink or in the downlink. 0b0: Uplink 0b1: Downlink |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Burst Size | 5 |  |
| Reserved | 11 |  |
| } |  |  |

# References

[1] IEEE P802.16p AAI AWD