| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of the network re-entry type indicator in WirelessMAN IMT-Advanced system |  |
| Date Submitted | Sep 09, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE P802.16p AAI AWD texts |  |
| Abstract | This is to clarify the network re-entry type indicator in WirelessMAN IMT-Advanced system. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of the network re-entry type indicator in WirelessMAN IMT-Advanced system

Giwon Park, Youngsoo Yuk, Jeongki Kim, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

Current draft [1] describes that if the M2M network re-entry type is set to 0b000 (i.e., dedicated channel allocation of AAI-RNG-REQ), fixed M2M device may not perform the code ranging when it performs the idle mode network reentry. Similarly, when the fixed M2M device performs the location update, it may not perform the code ranging. That is, if the ABS trigger location update of fixed M2M device via AAI-PAG-ADV message with network re-entry type (i.e., dedicated channel allocation of AAI-RNG-REQ), fixed M2M device may not perform the code ranging when it performs the location update.

Thus, M2M network access type is appropriate rather than M2M network re-entry type.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 6.2.18.7.2 (page 29, line 20) as follows:]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

Based on the mobility and traffic characteristics of the M2M device, the ABS can select the proper M2M network re-entry access type for M2M device based on the Table 795b, and the ABS shall inform the M2M device the M2M network re-entry access type in AAI-PAG-ADV message.

[Remedy 2. Modify the text in 6.2.18.7.2 (page 29, line 40) as follows:]

If the M2M network re-entry access type is set to ‘0’, the M2M device doesn't need to send CDMA code for ranging but sends AAI-RNG-REQ message with the channel allocation in ‘Dedicated channel allocation’ in AAI-PAG-ADV message.

If the M2M network re-entry access type is set to ‘1’, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the dedicated S-RCH allocation is used for ranging.

If the M2M network re-entry access type is set to ‘2’, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the dedicated NS-RCH allocation is used for ranging.

[Remedy 3. Modify the table 795b in page 29, line 24 as follows:]

Table 795b – Scheme selection of network re-entry access for M2M

| M2M Network re-entry access type | M2M Network re-entry access scheme | Note |
| --- | --- | --- |
| 0 | Dedicated channel allocation for AAI-RNG-REQ, A-MAP IE offset for AAI-RNG-REQ is indicated in AAI-PAG-ADV | Fixed M2M, known traffic pattern, UL synchronization not required |
| 1 | Dedicated ranging channel allocation for M2M group, S-RCH used for ranging | Fixed M2M, UL Synchronization required |
| 2 | Dedicated ranging channel allocation for M2M group, NS-RCH used for ranging | Mobile M2M, known traffic pattern |

[Remedy 4. Modify the table 706 in page 17, line 22 as follows:]

Table 706 – AAI-PAG-ADV message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Action Code | 2 | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| If (Action Code == 0b00 ¦ 0b01) { |  |  |  |
| M2M network re-entry access type | 3 | Indicate the network re-entry access scheme for M2M device 0b000: dedicated channel allocation for AAI-RNG-REQ 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated ranging channel allocation, NS-RCH 0b011-0b111: reserved | Present if M2M is supported |
| Dedicated channel allocation | TBD | If M2M network re-entry access type = 0, to indicate Resource Index for AAI-RNG-REQ; If M2M network re-entry access type = 1, 2 to indicate dedicated ranging channel allocation; |  |
| … | … | … | … |

# References

[1] IEEE P802.16p AAI AWD