| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MGID Assignment |  |
| Date Submitted | 2011-09-16 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | E-mail: mhtao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Comments 80216p-11_0022 |  |
| Abstract | This contribution proposes that a FID can be associated with multiple MGIDs. |  |
| Purpose | To be discussed and agreed in IEEE 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MGID Assignment

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

# Introduction

If we can allow a FID to be associated with multiple MGIDs, the following scenario can be realized.

1. MS1 and MS2 subscribe the same multicast service (e.g., streaming video).

2. MS1 requires high quality for the service while MS2 requires only low quality (e.g., MS1 requires 1024x768 streaming video, MS2 requires 640x480 streaming video).

3. MS1's FID is associated with MGID X and MGID Y, and MS2's FID is associated with MGID X only.

4. MS1 will obtain more data due to the additionally assigned MGID Y, and hence has higher service quality.

Figure 1. Service differentiation realized by different MGID assignment.

# Text Proposal

Adopt the following text modifications to IEEE 802.16p-11/0022:

--------------------------  Begin Text Proposal   -------------------------------

[Remedy #1: Insert the following blue text to page 4, line 55]

6.2.1.3.1 M2M Group Identifier (MGID)

An MGID is assigned to a service flow of an M2M device by a network entity after initial network entry through DSA procedure and released during an explicit network exit (e.g., power down location update) or when the M2M device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network or the network explicitly deletes the service flow associated with the MGID. The MGID can be re-assigned during connected state and idle state. During connected state, the MGID may be changed, and deleted by DSC, and DSD procedure respectively. <INS> A service flow is allowed to be associated with multiple MGIDs. <\INS>

[Remedy #2: Insert the following blue text and delete the following red text from page 8, line 54]

6.2.3.2 AAI-RNG-RSP

Table 685—AAI-RNG-RSP message field description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  | … | … |
| For(i=0;i<Num_MGFID;i++){ | 1 | Number of MGID and FID (Num_MGFID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if it needs to be updated |
| Current MGID | 15 |  |  |
| Current FID | 4 |  |  |
| For(j=0;j<Num_MGID;j++){ |  | Number of MGIDs to be associated with the FID. |  |
| New MGID | 15 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

[Remedy #3: Insert the following blue text from page 22, line 54]

6.2.3.47.1 AAI-DSA-REQ

Table 740—AAI-DSA-REQ message field description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  | … | … |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| MGID | 15 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an 802.16p BS initiates AAI-DSA-REQ. |
| } |  |  |  |
| MGSS | 64 | MGSS (M2M service Group Security Seed) for an M2M group | May be present when an ABS initiates AAI-DSA-REQ for this service flow that is related with M2M multicast service |

[Remedy #4: Insert the following blue text from page 23, line 24]

6.2.3.47.2 AAI-DSA-RSP

Table 741—AAI-DSA-RSP message field description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| { |  |  |  |
| FID | 4 | An identifier of a service flow |  |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| MGID | 15 | MGID to be added | Shall be present if this service flow is related with M2M multicast service and when an M2M device initiates AAI-DSA-REQ. |
| } |  |  |  |
| } |  |  |  |

[Remedy #5: Insert the following blue text from page 23, line 52]

6.2.3.47.4 AAI-DSC-REQ

Table 743—AAI-DSC-REQ message field description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| For(i=0;i<Num_MGID;i++){ |  |  |  |
| MGID | 15 | MGID to be changed to | Shall be present if MGID needs to be changed |
| } |  |  |  |

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r2: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE 802.16p-10/0022: WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems– Enhancements to Support Machine-to-Machine Applications.

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.

[4] IEEE P802.16.1™/D1, WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems.