| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MGID Assignment |  |
| Date Submitted | 2011-09-12 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | E-mail: mhtao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Comments 80216p-11_0021 |  |
| Abstract | This contribution proposes that a service flow can be associated with multiple MGIDs. |  |
| Purpose | To be discussed and agreed in IEEE 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MGID Assignment

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

# Introduction

If we can allow a service flow to be associated with multiple MGIDs, the following scenario can be realized.

1. MS1 and MS2 subscribe the same multicast service (e.g., streaming video).

2. MS1 requires high quality for the service while MS2 requires only low quality (e.g., MS1 requires 1024x768 streaming video, MS2 requires 640x480 streaming video).

3. MS1's service flow is associated with MGID X and MGID Y, and MS2's service flow is associated with MGID X only.

4. MS1 will obtain more data due to the additionally assigned MGID Y, and hence has higher service quality.

# Text Proposal

Adopt the following text modifications to IEEE 802.16p-11/0021:

--------------------------  Begin Text Proposal   -------------------------------

[Remedy #1: Insert the following blue text to page 4, line 21]

6.3.1 Addressing and connections

An MGID is assigned to a service flow of an M2M device by a network entity after initial network entry through DSA procedure and released during an explicit network exit (e.g., power down location update). The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned during normal mode and idle mode. During normal mode, the MGID may be changed by DSC procedure. Reassignment procedure during idle mode is TBD. <INS> A service flow is allowed to be associated with multiple MGIDs. <\INS>

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r2: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE 802.16p-10/0021: Part 16: Air Interface for Broadband Wireless Access Systems—Enhancements to Support Machine-to-Machine Applications.

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.

[4] IEEE P802.16Rev3/D1, IEEE Draft Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems.