| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Using Predefined BR Index to Identify M2M Devices Sharing STID |  |
| Date Submitted | 2011-9-9 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee, Tsung-Yu Tsai Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Call for Comments on 802.16p (M2M) 16.1AWD |  |
| Abstract | This contribution proposes to identify M2M devices with the same STID using different Predefined BR index. |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Using Predefined BR Index to Identify M2M Devices Sharing STID

Yi-Ting Lin, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee, Tsung-Yu Tsai

Institute for Information Industry (III)

# Introduction

During the San Francisco July meeting, STID-sharing scheme is proposed to increase ID capacity for large number of M2M devices. Based on the assigned STID_Valid_Periodicity and STID_Valid_Offset, an M2M device is allowed to transmit and receive data at specific frames. However, the STID-sharing scheme does not prohibit the M2M from performing bandwidth request at any frame. Therefore, different Predefined BR indexes carried in the quick access message are used for identifying the M2M devices sharing STID. In this contribution, we use the reserved bit of BR action 0b11 to accomplish the design.

# Reference

[1] IEEE 802.16m-2011, “Part 16: Air Interface for Broadband Wireless Access Systems: Amendment 3: Advanced Air Interface,” May 2011.

[2] IEEE 802.16p-11/0022, “P802.16p AAI AWD,” Aug. 2011.

- Text proposal for inclusion in the 802.16p AWD

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

6.2.3.47.1 AAI-DSA-REQ

Table 740 – AAI-DSA-REQ Message Field description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| For(i = 1; i≤ N-Predefined-BRindices; i++) { |  | The mapping of predefined BR index used in quick access message to BR size and BR actions N-Predefined-BR-indices is the number of predefined BR indices [1..15] |  |
| Predefined BR index | 4 | Predefined BR index | Present if N-Predefined-BR-indices is not zero |
| BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: ReservedM2M Extensions | Present if N-Predefined-BR-indices is not zero |
| If (BR action == 0b11) { |  |  |  |
| Purpose Indication | 2 | 0b01: Used for identifying M2M device sharing the same STID. 0b10-0b11: reserved |  |
| If (Purpose Indication == 0b01) { |  |  |  |
| STID_Valid_Offset | 3 | STID_Valid_Offset of the M2M device which requests UL bandwidth |  |
| BR size | 11 | Number of bytes requested by the Predefined BR index |  |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------