| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the polling procedure for fixed M2M devices |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD(based on 802.16Rev3) |  |
| Abstract | This contribution proposes texts for polling procedure |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the polling procedure for fixed M2M device

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

This contribution proposes to clarify the polling operation using MOB_PAG-ADV message.

At first, polling procedure for fixed M2M device is optional. Therefore parameter M2M report code may be included as TLV such as transmission type and Max Number of paging cycle.

Next, the unit of length field is byte. Therefore the value in the length field has to be changed to apply the similar mechanism to that adopted for 16m system.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

6.3.2.3.51 MOB_PAG-ADV (BS broadcast paging) message

Change the contents of Table 172 as follows

Table 172- MOB_PAG-ADV message format

| syntax | Size(bit) | Notes |
| --- | --- | --- |
| … |  |  |
| M2M report code | 1 | Action instruction to M2M device 1: Indication for the M2M device to send the uplink report |
| Reserved | 623 |  |
| } |  |  |
| ……. |  |  |

Add the following texts at the end of sub-clause 6.3.2.3.51

When the MOB_PAG-ADV message is used to poll fixed M2M devices for periodic uplink non-real time data transmission, the following TLV may be included:

M2M report code( see 11.17.6)

Action instruction to M2M device:

One bit in TLV corresponds to one MS paged. Each bit set to 1 indicates for the M2M device to send the uplink report. The order of the assignments is the same as the order of appearance of MS MAC address in this message.

Insert new subclause 11.17.6

11.17.6 M2M report code

The following TLV may be used by an BS to indicate action instruction to fixed M2M device.

| Name | Type | Length(bits) | Value | Scope |
| --- | --- | --- | --- | --- |
| M2M report code | 157 | Variable: (8+Num_MACs) | Bit 0-7 : Num_MACs Subsequent Num_MACs bits: For(i=0;i<Num_MACs;i++){ 1bit M2M Report code } | MOB_PAG-ADV |

11.14. DREG-CMD/REQ message encodings

Modify texts as follows

Add new subclause 11.14.1

11.4.1 M2M device specific idle mode timer

| Name | Type | Length | value | scope |
| --- | --- | --- | --- | --- |
| M2M device specific Idle mode timer | 53 | 3 | Length of the maximum interval between two consecutive location updates while the M2M device is in idle mode | DREG-CMD |
| Transmission Type | 54 | 1 | Bit 0: allowed to send data only after receiving a paging message with M2M report code Bit 1-7: reserved | DREG-CMD |
| Max number of paging cycle | 55 | 82 | This is max number of paging cycle for M2M device to wait for MOB_PAG-ADV with M2M report code. The unit is the duration of the paging cycle | DREG-CMD |

----------------- End of the text proposal ----------------------------------------------------------------------------------------