| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M ranging indicator in the AAI-SCD message |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD(based on 802.16.1/D0) |  |
| Abstract | This contribution proposes texts for M2M ranging indicator in the AAI-SCD message |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M ranging indicator in the AAI-SCD message

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

This contribution proposes to clarify the M2M ranging indicator in the AAI-SCD message.

According to the section 6.2.18.7.2, if an ABS assigns ranging resources dedicated for M2M devices, M2M devices shall perform ranging for network (re-)entry using dedicated ranging resources. And the configuration of ranging assignment for M2M devices is indicated through M2M ranging indicator in the AAI-SCD message.

However, if the dedicated ranging resources are included in the AAI-SCD message, M2M devices can ascertain that ABS assigns ranging resources dedicated for M2M devices without M2M ranging indicator.  Therefore, it is unnecessary to indicate the configuration of ranging assignment for M2M device through M2M ranging indicator in the AAI-SCD message.

In addition, the ranging configuration with M2M ranging indicator set to 0b10 is contradictory to the group’s agreement that M2M devices shall perform ranging using dedicated resources in case that BS assigns ranging resources dedicated for M2M2 devices.

In case of the ranging configuration with M2M ranging indicator set to 0b11, access class restriction in the AAI-SCD message may provide the same functionality.

Therefore we propose to delete M2M ranging indicator from the AAI-SCD message.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Modify the contents of Table 714 as follows

Table 714 –AAI-SCD message format

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| …… |  |  |  |
| M2M ranging indicator | 2 | Indicator the ranging configuration for M2M devices. 0b00: normal ranging as defined in Table 833 in 16.3.5.5.1.2 0b01: dedicated ranging for M2M devices 0b10: allow both normal and dedicated ranging 0b11: no ranging(not allow network re-/entry |  |
| If(M2M ranging indicator == 0b01) or (M2M ranging indicator== 0b01){ |  |  |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices | Present if an ABS assigns ranging resources dedicated for M2M devices |
| Periodicity of the M2M ranging | [3] | Indicated the periodicity of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001:transmission in the first frame in every superframe 0b010:transmission in the first frame in every even numbered superframe. i.e., mod(superframe number,2)==0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4)==0 [0b100 ~ 0b111:reserved] | Present if an ABS assigns ranging resources dedicated for M2M devices |
| } |  |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------