| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the M2M configuration change count |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD(based on 802.16.1/ D0) |  |
| Abstract | This contribution proposes texts for AAI-SCD message |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the M2M Configuration Change Count

Soojung Jung, Jaesun Cha, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

This contribution proposes to clarify the M2M configuration change count in the AAI-SCD message.

To avoid the unnecessary decoding/update the dedicated ranging information for M2M devices, additionally ABS can transmit M2M configuration change count in the AAI-SCD message and M2M SCD count in the S-SFH SP3 IE.

However, for this purpose, an exception should be added to the description of Configuration Change Count in the AAI-SCD message.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Modify the contents of Table 714 as follows

Table 714-AAI-SCD message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Configuration Change Count | 4 | The value is increased whenever the contents of this message except the dedicated ranging information for M2M devices are changed. The value rolls over from 0 to 15 |  |
| …… |  |  |  |
| M2M Configuration Change Count | 4 | The value is increased whenever the contents of the dedicated ranging information for M2M devices are changed. The value rolls over from 0 to 15. The operation of this field is same with Configuration Change Count as defined in 6.2.3.31 |  |
| ….. |  |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------