| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | S-SFH update for M2M device in idle mode |  |
| Date Submitted | 2011-09-09 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak LG Electronics | E-mail: heejeong.cho@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution provides the text for S-SFH update for M2M device in idle mode. |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

S-SFH update for M2M device in idle mode

Heejeong Cho, Youngsoo Yuk, Inuk Jung and Jin Sam Kwak

LG Electronics Inc.

# Introduction

This contribution includes the proposed text for the S-SFH update procedure for M2M device in idle mode. In WirelessMAN-Advanced Air Interface Systems [1], an ABS transmits P-SFH IE containing the S-SFH scheduling information, S-SFH change count, S-SFH SP change bitmap, and S-SFH application hold indicator. Whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) in any of the S-SFH IEs change, the S-SFH change count shall be incremented by 1 modulo 16 only in the specific superframe number (SFN) that satisfies SFN modulo S-SFH change cycle = 0, where the S-SFH change cycle is indicated in S-SFH SP3 IE, as shown in Figure 1.

Figure 1 –S-SFH change cycle in S-SFH SP3 IE of WirelessMAN-Advanced Air Interface Systems

The modified S-SFH change count shall be maintained until the next superframe (satisfying SFN modulo S-SFH change cycle = 0) in which the S-SFH change count is incremented.

An AMS shall compare the value of S-SFH change count in the current P-SFH IE with that stored last, whenever the validity check of the last stored S-SFH SP IEs is required.

— If there is no difference between two S-SFH change counts, the AMS may not decode S-SFH IE during a certain period where S-SFH change count remains unchanged.

— Else if the difference between two S-SFH change counts is one, the AMS shall update the S-SFH SP IE(s) whose bit in the S-SFH SP change bitmap is set to 1.

— Else if the difference between two S-SFH change counts is greater than one, the AMS shall update all S-SFH SP IEs.

The AMS in sleep mode can perform the validity check of the last stored S-SFH SP IEs before its sleep window termination, as defined in the following:

The AMS in sleep mode shall ensure that it has up-to-date system information for proper operation. To ensure that the AMS has up-to-date system information, the following scenarios may occur during a sleep window of the sleep mode AMS.

— Case 1: If there is no possible change on any S-SFH SP IE (i.e., S-SFH SP1 IE, SP2 IE and SP3 IE) during AMS’s sleep window (i.e., the sleep window terminates before the SFH where the S-SFH SP IE may change), then the AMS determines it does not have to decode this S-SFH SP IE during its sleep window.

— Case 2: If there is only one S-SFH SP IE change possibility during the sleep window, then the AMS performs operations defined in 6.2.24 to receive this S-SFH SP IE during the sleep window at SFH just before the sleep window termination where this SP IE is transmitted.

— Case 3: If there are multiple S-SFH SP IEs change possibilities during the sleep window, then the AMS performs operations defined in 6.2.24 for each of these S-SFH SP IEs during the sleep

However, the AMS in idle mode performs the validity check of the last stored S-SFH SP IEs at the beginning of the paging listening interval. If the P-SFH indicates a change in SFH, then the AMS shall acquire the latest S-SFH information when the system information is broadcast by the ABS. The AMS may not perform any operations until it gets the S-SFH information from the ABS. The following three problems are able to be encountered. These problems can be a critical issue to an M2M device because long paging cycle (e.g., 32768, 262144 and 4194304 superframes) may be applied to the M2M device.

- Problem 1 <Long delays in paging response>

  - In order to receive a paging message, the M2M device should wait until next paging cycle or second paging offset.

- Problem 2 <Endless loop of SFH information update without receiving the paging message>

  - S-SFH information can be changed again before the listening interval of the next paging cycle.

- Problem 3 <Invalid S-SFH change count>

  - If the M2M device’s paging cycle is greater than or equal to S-SFH change cycle * 2 bit size of S-SFH change count, S-SFH information can be changed more than 16 times during one paging cycle.

Therefore, we propose the S-SFH update procedure for M2M device in idle mode to solve these problems. The proposed S-SFH update procedure is similar to procedure of the AMS in sleep mode. Difference between these procedures is that the M2M device must perform the validity check of the S-SFH change count.

# References

[1] IEEE P802.16.1™/D1, “WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems”

[2] IEEE 80216p-11_0022, “Enhancements to Support Machine-to-Machine Applications”

# Proposed Text Change

Adopt the following Remedies in IEEE 80216p-11_0022.

[Remedy 1: Add a new subclause on page 30, line 55 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.18 Idle mode

6.2.18.7 Idle mode for M2M application

6.2.18.7.4 S-SFH update in idle mode

The M2M device in idle mode shall ensure that it has up-to-date system information for proper operation. To ensure that the M2M device has up-to-date system information, the following scenarios may occur during its paging unavailable interval.

— Case 1: If there is no possible change on any S-SFH SP IEs (i.e., S-SFH SP1 IE, SP2 IE and SP3 IE) during M2M device’s paging unavailable interval (i.e., the paging unavailable interval terminates before the SFN in which the S-SFH SP IEs may change), then the M2M device determines it does not have to decode any S-SFH SP IEs during its paging unavailable interval.

— Case 2: If there is change possibility of the S-SFH SP IE(s) during the paging unavailable interval, then the M2M device performs the S-SFH update procedure in the S-SFH change cycle duration(s) just before the paging unavailable interval termination. If SFH change count is valid, the M2M device acquires the S-SFH SP IE(s), which is decided according to value of the S-SFH change count as defined in 6.2.24. Otherwise, the M2M device acquires all S-SFH SP IEs regardless of value of the S-SFH change count. “SFH change count is valid” means that the M2M device’s paging unavailable interval is less than m in Equation (x), or the current SFN is less than SFNm in Equation (y).

m = S-SFH change cycle  2 s                                                   (x)

where

m               is minimum effective time of the S-SFH change count.

S-SFH change cycle is the minimum duration over which the contents of the S-SFH remain the same.

s                is bit size of the S-SFH change count.

SFNm = SFNv + m                                                             (y)

where

SFNm is superframe number to which the S-SFH change count is valid from SFNv.

SFNv is superframe number in which the validity check of the last stored S-SFH has been performed

lastly.

m   is minimum effective time of the S-SFH change count.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------